[image: image216.jpg]alhua



                        Dahua Network Camera Web 3.0 Operation Manual                                                    

Dahua Network Camera Web3.0 Operation Manual
     Version 1.0.1
Dahua Vision Technology CO., LTD
Table of Contents
11
Network Config


11.1
Network Connection


11.2
Log in


62
Live


62.1
Encode Setup


72.2
System Menu


72.3
Video Window Function Option


82.4
Video Window Setup


82.4.1
Image Adjustment


92.4.2
Original Size


92.4.3
Full Screen


102.4.4
Width and Height Ratio


102.4.5
Fluency Adjustment


102.4.6
Rules Info


102.4.7
Zoom and Focus


102.4.8
Fisheye/Trigger Track


153
PTZ Control


163.1
Scan


173.2
Preset


173.3
Tour


183.4
Pattern


193.5
Assistant


193.6
Light Wiper


204
Playback


204.1
Playback


214.1.1
Function of Play


224.1.2
Playback File


244.1.3
Playback Cut


244.1.4
Record Type


244.1.5
Progress Bar


254.1.6
Assistant Function


254.2
Picture Playback


264.2.1
Play


264.2.2
Playback File


284.2.3
Snapshot Type


305
Setup


305.1
Camera


305.1.1
Conditions


425.1.2
Video


545.1.3
Audio


565.2
Network


565.2.1
TCP/IP


595.2.2
Connection


615.2.3
PPPoE


625.2.4
DDNS


645.2.5
IP filter


645.2.6
SMTP （e-mail)


665.2.7
UPnP


675.2.8
SNMP


695.2.9
Bonjour


705.2.10
Multicast


705.2.11
4G


725.2.12
WIFI


755.2.13
802.1x


765.2.14
QoS


775.2.15
HTTPs


865.3
Event


865.3.1
Video detection


935.3.2
Audio Detection


955.3.3
Smart Plan


955.3.4
Intelligence Behavior Analytics


1035.3.5
Face Detection


1055.3.6
People Counting


1075.3.7
Heat Map


1105.3.8
Alarm


1145.3.9
Abnormity


1175.4
Storage Management


1175.4.1
Schedule


1215.4.2
Destination


1245.4.3
Record control


1245.5
System


1245.5.1
General


1275.5.2
Account


1305.5.3
PTZ


1315.5.4
Default


1325.5.5
Import/Export


1325.5.6 Remote control


1335.5.6
Auto Maintenance


1345.5.7
Upgrade


1345.6
Information


1345.6.1
Version


1355.6.2
Log


1355.6.3
Online User


1376
Alarm


1397
Log out




Important
The following functions are for reference only. Some series products may not support all the functions listed below. 

1 Network Config
1.1 Network Connection

Network camera and PC connection mainly has two ways, see Figure 1- 1 and Figure 1-2.
[image: image1.png]cable

PC




Figure 1- 1
[image: image2.png]cable — cable
switch or

router





Figure 1-2
Before you access network camera via the Internet, you need to have its IP address. User can use Config Tool to search IP of the network camera. Please refer to Config Tool manual. 
1.2 Log in 

It needs to install WEB plug-in when you use WEB client for the first time, the exact operation steps are shown as follows: 

Open IE and input network camera address in the address bar. (The factory default IP address is 192.168.1.108).

After successful connection, the login interface is shown as in Figure 1-3; input your user name and password. Default factory username is admin and password is admin.
[image: image3.jpg]Usemanme: | admin

Password:





Figure 1-3
The system will display “Update Password” prompt box for your first login, users need to modify the password and save it properly. 
[image: image4.png]First login, suggest updating password.

New Password

Confirm Password

Don't show me again

ok

Cancel





Figure 1-4 
After successful login, you may see the interface shown as in Figure 1-5. 

[image: image5.jpg]E)/IE]

TEcHNOLOGY

wansusan | susveam1 | susveamz fRtlTce -]

here to download and install the plug-in

R R P E e





Figure 1-5 

Click on “Please click here to download and install the plug-in”. The system pops up warning information to ask you whether run or save this plug-in.  See Figure 1-6. 

[image: image6.png]Do you want to run or save this file?
Name: webplugin.exe
Type: Applcation, 1.08MB
From: 10.10.4.198

Fun

potentialy haim your computer If you do ot st the soure, do not

' ‘Whil fls fiom the Iteret can be useful this fle type can
o orsave i softuare.





Figure 1-6 

You must either run or save the file to local and install it. Follow the following steps. Click on run, you will see Figure 1-7and Figure 1-8.

[image: image7.png]= 'Setlip - webplugin.exe

Ready to Install
Setupis now ready to begin instaling webplugin.exe on your computer.

Click Install ta continue with the nstalation.





Figure 1-7
[image: image8.png]EL

Instaliing
Please wait wihile Setup installs webplugin.exe on your computer,

Registering fles.





Figure 1-8
When plug-in installation completes, the installation page closes automatically. The web-end will refresh automatically, and then you can view video captured by the camera.

[image: image9.jpg]@Jhua

% R R e e





Figure 1-9 
2 Live
After you logged in, you can see the live monitor window. See Figure 2-1.
[image: image10.jpg]@Jhua

% R R e e





Figure 2-1 
There are four sections:

· Section 1: Encode setup bar
· Section 2: System menu 

· Section 3: Window function option bar
· Section 4: Window adjust bar 
2.1 Encode Setup 
Note: Some series don’t support sub stream 2.

The encode setup interface is shown as in Figure 2-2.

[image: image11.png]protocol IS





Figure 2-2 
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Main stream 
	Click it to enable main stream video monitoring and click again to disable it. Generally for storage and monitor.  

	Sub Stream 1
	Click it to enable Sub Stream1 video monitoring and click again to disable it. When network bandwidth is insufficient, it substitutes main stream for monitoring. 

	Sub Stream 2
	Click it to enable Sub Stream 2 video monitoring and click again to disable it. When network bandwidth is insufficient, it substitutes main stream for monitoring.

	Protocol 
	You can select stream media protocol from the dropdown list. 

There are three options: TCP/UDP/Multicast


2.2 System Menu 

System menu is shown as in Figure 2-3.
Please refer to chapter 2 Live, chapter 3 PTZ, chapter 4 Playback, chapter 5 Setup, chapter 6 Alarm, chapter 7 Log out  for detailed information.  

[image: image12.png]Live PZ Playback Ssetup Alarm Logout




Figure 2-3 

2.3 Video Window Function Option 

The interface is shown as below. See Figure 2-4.


[image: image13.png]o JN=THGE X 1¢)




Figure 2-4 
Please refer to the following sheet for detailed information. 

	SN
	Parameter
	Function 

	1
	Relay-out
	It shows if there is any alarm output, status description is as follows: 
· Red: means there is alarm output. 
· Grey: means alarm is over. 

Click on the button to force alarm to be on or off. 

	2
	Digital Zoom
	· When the video is in the original status, click it you can select any zone to zoom in. In the non-original status, you can drag the zoom-in zone in specified range. Right click mouse to restore previous status. 
· Click it; you can use the middle button of the mouse to zoom in/out the video size. 

	3
	Snapshot
	Click on the button to snapshot, save picture to path in Ch 5.1.2.5.

	4
	Triple Snapshot 
	Click it to take snapshot upon the video at the frequency of one picture per second. All images are saved to path in Ch 5.1.2.5.

	5
	Record
	Click it to record the video. All videos are saved to path in Chapter 5.1.2.5.

	6
	Easy focus 
	Click it, you can see there are two parameters on the preview video：AF Peak and AF Max.

AF Peak: It is to display the video definition during the focus process. 

AF Max: It is the most suitable value for the video definition. 

The close the AF Peak and AF Max is, the better the focus effect is. 

	7
	Audio 
	Turn on or off audio when you are monitoring.

	8
	Talk 
	Click it to start or end bidirectional talk.

	9
	Help 
	Click it to open help file. 


2.4 Video Window Setup  

The interface is shown as in Figure 2-5.
[image: image14.png][ONONONONONGONGNO]




Figure 2-5
2.4.1 Image Adjustment
See Figure 2-6 for image adjustment.
[image: image15.png]Image Adjustment




Figure 2-6
Click this button to display/hide image control interface. Click it to open picture setup interface. This interface is on the top right pane. 

Please refer to the following sheet for detailed information. 

	Parameter
	Function 

	Video setup 
	
[image: image16.png]



	It is to adjust monitor video brightness. 
	Note:

· All the operations here apply to WEB end only.

· Please go to Setup->Camera->Conditions to adjust corresponding items. 

	
	
[image: image17.png]



	It is to adjust monitor video contrastness.
	

	
	
[image: image18.png]



	It is to adjust monitor video hue.
	

	
	
[image: image19.png]



	It is to adjust monitor video saturation.
	

	
	Reset

Re
	Restore brightness, contrastness saturation and hue to system default setup.
	


2.4.2 Original Size
Click this button to go to original size. It is to display the actual size of the video stream. It depends on the resolution of the bit stream.

2.4.3 Full Screen

Click it to go to full-screen mode.  Double click the mouse or click the Esc button to exit the full screen.

2.4.4 Width and Height Ratio

Click it to restore original ratio or suitable window. 

2.4.5 Fluency Adjustment

There are three levels of fluency for you to select (Realtime, Normal, and Fluency). The default is normal. 
2.4.6 Rules Info
Click the button, preview image will display intelligent rules after enabling; it is “enable” by default. 
2.4.7 Zoom and Focus
Click this button and the focus zooming interface appears on the right of preview interface, as shown in

Figure 2-7, click left mouse button to adjust focus zooming configuration. 

Note: 

· The product series which support motorized zoom, synchronous focus and back focus have this button.   
· Auto-focus after zoom and focus adjustment.
2.4.8 Fisheye/Trigger Track 
Click the button, installation mode and display mode interface will show up on the right of the preview interface, see Figure 2-8 and Figure 2-9, single click to switch different installation modes and display modes for fisheye, or switch different display modes for trigger track, it is enabled by default.
Note: 
It is only supported by some models. 
[image: image20.jpg]Zoom and Focus

z00m  specs EOED

Tl
Focus spees EOIED

Far

53 Auto Focus

Restore All

= P
16




                                                                   Figure 2-7
	Parameter
	Function

	Zoom
	Adjust the focal length of the lens by clicking or long pressing “+”“-”buttons.
The speed is used to adjust the length of one step during single click.

	Focus
	Adjust the sharpness of the lens by clicking or long pressing“+”、“-” buttons.
Step length is used to adjust the length of one step with one click.

	Auto-focus
	Click to adjust the image definition automatically.

Note:

Other lens operations are not allowed during the process of auto-focus.

	Reset All
	Reset the lens to zero position to eliminate the accumulative error of lens.

Note:
Please reset when the image adjustment is not clear or operating zoom focus many times.

	Refresh
	Synchronize the location of drag slider of lens and zoom focus after hardware zoom focusing.

	Regional Focus 
	Click it and use the mouse to select a zone, then the device can auto focus within the specific region. 


                         In-ceiling Mount                       Wall Mount                       Ground Mount 
[image: image21.png]Installation Mode




 [image: image22.png]Installation Mode

¥ da




 [image: image23.png]



Figure 2-8
	parameter
	Note 

	Installation Mode 
	Three modes which are ceiling mount, wall mount and ground mount.

	Display Mode 
	It represents the display mode of the current image (default supports original image mode), the display modes may be different according to different installation modes. It is shown as follows: 
· Ceiling: 1P+1、2P、1+2、1+3、1+4、1P+6、1+8。

· Wall: 1P、1P+3、1P+4、1P+8。

· Ground: 1P+1、2P、1+3、1+4、1P+6、1+8。

Note: 
The default displays original image mode when switching installation mode. 

	In-ceiling/Wall/Ground
	[image: image24.png]


Original image
	It menas the original image without de-warpping

	In-ceiling/Ground
	[image: image25.png]


1P+1
	360°expanded rectangular panorama + independent sub image, the sub image and the subbox in the expanded rectangular panorama support zoom and movement, for the expanded rectangular panorama also supports left and right starting point movement.  

	
	[image: image26.png]il




2P
	Two related 180° expanded rectangular pictures, two subwindows form 360° panorama anytime, which is also called “dual panorama”. Two expanded rectangular pictures both support left and right movement starting point, which are also linked by eachother.  

	
	[image: image27.png]QLT




1+2
	Original image + 2 independent sub images, both the sub image and the subbox in the original image support zoom and movement. The original image also supports changing starting point by rotation (no such display mode for ground installation). 

	
	[image: image28.png]


1+3
	Original image + 2 independent sub images, both the sub image and the subbox in the original image support zoom and movement. The original image also supports changing starting point by rotation

	
	[image: image29.png]IQH




1+4
	Original image + 4 independent sub images, both the sub image and the subbox in the original image support zoom and movement. The original image also supports changing starting point by rotation

	
	[image: image30.png]


1P+6
	360°expanded rectangular panorama +6 independent sub image, both the sub image and the subbox in the expanded rectangular panorama support zoom and movement, for the expanded rectangular panorama also supports left and right starting point movement.

	
	[image: image31.png]


1+8
	Original image + 8 independent sub images, both the sub image and the subbox in the original image support zoom and movement. The original image also supports changing starting point by rotation

	Wall 
	[image: image32.png]


1P
	From left to right 180° expanded rectangular panorama, which supports up and down movement and changes vertical angle of view. 

	
	[image: image33.png]


1P+3
	180° expanded rectangular panorama+3 independent sub images, both the sub images and the sub box in the expanded rectangular panorama support zoom and movement, expanded rectangular panorama supports up and down movement and changes vertical angle of view. 

	
	[image: image34.png]


1P+4
	180° expanded rectangular panorama+4 independent sub images, both the sub images and the sub box in the expanded rectangular panorama support zoom and movement, expanded rectangular panorama supports up and down movement and changes vertical angle of view.

	
	[image: image35.png]


1P+8
	180° expanded rectangular panorama+8 independent sub images, both the sub images and the sub box in the expanded rectangular panorama support zoom and movement, expanded rectangular panorama supports up and down movement and changes vertical angle of view.


[image: image36.jpg]Display Mode





Figure 2-9

Enable the trigger track on the interface of tripwire or intrusion, and draw the rule of tripwire or intrusion, the scene of trigger track window will change according to the moving object when it triggers rule alarm until the moving object disappears from the view range of the camera. Please refer to “5.3.4 IVS” for more details about the rules drawing and parameter config of tripwire and intrusion. 
Trigger track includes three modes which are 1P, 1P+3 and 1P+5. 
· [image: image37.png]


 1P: Original picture

· [image: image38.png]


 1P+3: Original picture and three trigger track windows, it can adjust the location and size of three trigger track windows on the original picture. 
· [image: image39.png]


 1P+5: Original picture and five trigger track windows, it can adjust the location and size of five trigger track windows on the original picture. 

3 PTZ Control 

Here you can view direction keys, speed, zoom, focus, iris, preset, tour, scan, pattern, aux on, off and PTZ setup button. See Figure 3-1.
Note: 

Before PTZ operation, please make sure you have properly set PTZ protocol. (Please refer to Ch 5.5.3).

Currently only IPC-HFXXXX series and –PT series products can support PTZ function. 
	Parameter
	Note

	PTZ direction
	· PTZ supports eight directions: left/right/up/down/upper left/upper right/bottom left/bottom right.

	Speed
	It controls rotation speed. The longer the step length, the higher the speed. Step length control PTZ, zoom, focus and iris. 

	[image: image40.png]


Quick Position
	Use mouse to draw a box in monitoring video, PTZ will rotate and focus to quickly positioning. 


[image: image41.jpg]


 Figure 3-1
[image: image42.png]000
0G0
000
speeat-2): EEY
oo
Ry
e~ 0

PTZ Settings





Figure 3-2 
PTZ setting interface is shown as in Figure 3-3.

Here you can set scan, preset, tour, pattern, assistant function and light and wiper plus view coordinate.
[image: image43.png]PTZ Setlings.





Figure 3-3 

3.1 Scan

Scan interface is shown in Figure 3-4.

[image: image44.png]PTZ Settings





Figure 3-4
Steps to scan are:

Step 1. Click on Set button, display [image: image45.png]


 icon. 

Step 2. Move via direction key to select left, click on Set Left to set left border of camera 

Step 3. Move via direction key to select right, click on Set Right to set right border of camera. 

Step 4. Complete scan path setup. 

3.2 Preset

Preset interface is shown in Figure 3-5.
[image: image46.png]PTZ Seting

Preset

Q Goto

© SetPreset

+Add.





Figure 3-5
Steps to preset are:

Step 1. In preset box, input preset value. 

Step 2. Click on Go to, camera rotates to preset position. 

Step 3. Use direction key to rotate camera, and in preset box input preset value. 

Step 4. Click on Add to add a preset. Range of preset relates to PTZ protocol. 

3.3 Tour

Tour interface is shown in Figure 3-6.

[image: image47.png]PTZ Settings





Figure 3-6
Steps to tour are:

Step 1. In tour box, input tour path value. 

Step 2. Click on Add. Range of tour relates to PTZ protocol. 

Step 3. In preset box, input preset value. 
Step 4. Click on Add as to add a preset in this tour. If click on Del, it deletes this preset in tour. 

Note:

You can add more than one preset here, or delete more than one preset. 

3.4 Pattern

Pattern interface is shown in Figure 3-7.

[image: image48.png]1~
Stop

EXEEEN





Figure 3-7
Steps to set pattern are shown as follows:

Step 1 

Input pattern serial number value in the box, click “Add” and it will display “Start Rec” and “Stop Rec”. 

Step 2 

Click “Start Rec” to implement a series of operations such as zoom, focus, iris, direction and so on. 

Step 3 

Click “Stop Rec” to complete the setting of a pattern path.

Step 4 

Click “Start” and it will start pattern according to the pattern path which has been set; click “Stop” and the pattern ends. 
3.5 Assistant
Assistant interface is shown in Figure 3-8.

[image: image49.png]PTZ Settings





Figure 3-8
Steps to assistant are:
Step 1. In assistant box input assistant value. 

Step 2. Click on Aux On to turn on aux function. 

 
Click on Aux off to turn off aux function. 

3.6 Light/Wiper

Light wiper interface is shown in Figure 3-9.

[image: image50.png]PTZ Setlings.





Figure 3-9
Steps to light wiper are:

· Click on Enable to enable light wiper function. 

· Click on Disable to disable light wiper function. 

4 Playback

Web client playback supports video playback and picture playback. 
Note:

Before playback, user shall set storage management as in Ch. 5.4. 

4.1 Playback 
The playback interface is shown as in Figure 4-1.

[image: image51.png]



Figure 4-1
There are four sections:

· Section 1: Function of play

· Section 2: Playback file
· Section 3: Play time cut
· Section 4: Record type

· Section 5: Progress bar

· Section 6: Assistant function

4.1.1 Function of Play
The function of play is shown as in Figure 4-2 and Figure 4-3.
[image: image52.png]00000 O =0

O e 6 ©® @ 1




Figure 4-2
[image: image53.png]00000 O =0

O® e 6 ® O &




Figure 4-3
	Parameter
	Function

	1 Play
	When you see this button, it means pause or not played record. Click on this button, switch to normal play status. 

	2 Stop
	Click this button to stop playing. 

	3 Play by frame
	Click on this button to go to next frame. 

Note: 

You shall pause record when you use this function. 

	4 Slow
	Click on this button to play slowly. 

	5 Quick
	Click on this button to play quickly. 

	6 Silent
	When this button displays, it means audio is silent. Click on this button to switch back to normal. 

	7 Volume
	Click on left mouse to adjust volume. 

	8 -1 Fisheye 
	Click this button and fisheye device can adjust display mode according to different installation mode during the process of playback.

	8 -2 Rule Info
	Click the button and it will playback and display intelligent rules and object detection box if the video is equipped with intelligent rule info after the function is enabled, it is off by default. 


4.1.2 Playback File
In calendar, blue date represents data currently has video record or snapshot. See Figure 4-4.
[image: image54.png]e e
ouscv: RG]
< |

‘Sun Mon Tue Wed Thu Fri Sat

1|2

8 9

3.4 5 6 7

10 11 12 13 14




Figure 4-4
	Parameter
	Function

	File Type
	· Select “dav”, as video playback. 

· Select “jpg” as picture playback. 

	Data Source
	Default is SD card. 


Step 1. Click on data in blue, time axis displays record file progress bar in color. While, green represents normal record, yellow represents motion detect record, red represents alarm record, and blue represents manual record. 
Step 2. Click on certain time on progress bar, playback starts from this time.  See Figure 4-5.

[image: image55.png]



Figure 4-5
Step 3. Click on file list[image: image56.png]


, select date file will be displayed in list. 
Step 4. Double click on file in list, playback this file and display file size, start time and end time. 
See Figure 4-6..
[image: image57.jpg]00 : 00 : 00 - 23 : 59 : 59 |{e
General, Event, Alam, Manual |

Download Format @ dav © mp4

Start Time  File Type.

©
©
©
©
©
©
©
©
©

<t D

Begin Time:
End Time:

File Size:





Figure 4-6
	Parameter
	Function

	[image: image58.png]


Search
	It means records within searched start time and end time on the date.

	Download Format
	There are two formats:  dav, mp4.

	[image: image59.png]


Download
	· click the download button and download file to path in Ch. 5.1.2.5.
· System does not support download and playback at the same time.   

	[image: image60.png]


Back
	Click on back button to go to calendar interface. 


4.1.3 Playback Cut

Note:

Playback cut function will automatically pause playing record as playback cut and playback cannot be at the same time. 

Step 1. Click on start time to cut on time axis. This time must be within progress bar range. 

Step 2. Move mouse to cut icon[image: image61.png]


. You will be ask to select start time. Click on cur icon [image: image62.png]


 as finish cutting. 
Step 3. Click on playback cut end time on time axis. This time must be within progress bar range. 
Step 4. Move mouse to cut icon[image: image63.png]


 you will be asked to select end time. Click on cut icon [image: image64.png]


 as finish cutting.
Step 5. Click on Save button to save file cut to path in Ch 5.1.2.5. See Figure 4-7.
[image: image65.png]



Figure 4-7
4.1.4 Record Type
After checking record file type, only selected file will be displayed in progress bar and file list. Users can also select the record type to be displayed via the dropdown box which is above the file list. See Figure 4-8.
[image: image66.png]RecordType @Al [General ® [@Motion @ [MAlarm  [dManual B




Figure 4-8
4.1.5 Progress Bar
[image: image67.png]



Figure 4-9
	Parameter
	Function

	[image: image68.png]


24 hours
	Click on it, means video in past 24 hours.

	[image: image69.png]


2 hours
	Click on it, means video in past 2 hours.

	[image: image70.png]


1 hour
	Click on it, means video in past 1 hour.

	[image: image71.png]30min




30 min
	Click on it, means video in past 30 min.


4.1.6 Assistant Function

Video playback assistant function is shown in Figure 4-10.

[image: image72.png]



Figure 4-10
	Parameter
	Function

	[image: image73.png]


 Digital Zoom
	· Click on it, video in playback status if is in original size, user can zoom in any area, If it is not in its original size, Right mouse click to restore its original size. 
· Click on this button, you can scroll to zoom in. 

	[image: image74.png]


 Snapshot
	Click on this button, you can snapshot video under playback status. Snapshot will be saved to path in Ch. 5.1.2.5.


4.2 Picture Playback

Web client picture playback interface has the following three functions:
	Parameter 
	Function

	1
	Play function bar

	2
	Playback file bar

	3
	Snapshot type bar


See Figure 4-11.
[image: image75.jpg]alhua T (T T A p—
[

FieTye 2 ]

<

Sun Mon Tue Wed Thu Fri Sat

(>)

Snapshot Type Bl Al El General m [ Event m H Alarm





Figure 4-11
4.2.1 Play 

[image: image76.png]



Figure 4-12
Default icon is [image: image77.jpg]


  and it means pause or not played picture. Click on play button to switch to normal play status. Icon become [image: image78.jpg]



Click on it to pause. 
4.2.2 Playback File

[image: image79.png]rie e [N
ouscv: TG
< |

‘Sun Mon Tue Wed Thu Fri Sat

1|2]a]s]s]e]7

8 9 10 11 12 13 14




Figure 4-13
Step 1. Click on file list[image: image80.png]


, select snapshot file of the date. 
Step 2. Double click on file in list, to play this snapshot. 
	Parameter
	Function

	[image: image81.png]


 Search
	It means all snapshot files within the start time and end time of selected date.

	[image: image82.png]


Download
	Click the download button to open snapshot file or directly download to local according to the browser types.

	[image: image83.png]


 Back
	Click the back button to return to calendar interface and re-select time. 


[image: image84.png]al, Event, Alarm

StartTime ~ File Type
132316 W
132455
13:25:09
132519

132540
132550
132609
132619

10 132741

11 132750

12 132801

1
2
3
4
5 132520
5
7
s
9

EEEEEEE _ ®m
. Q00000000000

DRRVEN





Figure 4-14
4.2.3 Snapshot Type

After checking snapshot file type, in file list only display file of selected type. Users can also select the snapshot type to be displayed via the dropdown box above the file list. See Figure 4-15.

[image: image85.png]SnapshotType El Al [ General m [ Event @ [ Alarm





Figure 4-15
5 Setup
Web client setup support camera, network, time, storage, system and system info view. 

5.1 Camera
The camera setting includes conditions, profile management, zoom and focus. 
5.1.1 Conditions
Note:

The camera parameter may be different according to different models, please refer to the actual product for more details.

5.1.1.1 Picture 
Note: 

The device which supports true WDR fails to support long exposure when true WDR is enabled. 

Step 1 

Select “Setup > Camera > Conditions > Picture” and the system will display the “Picture” image which is shown in Figure 5-1. 
[image: image86.jpg]Conditions

Profile| Day

> Video

> o b it Standar

Network = Y Expostre Brightness (]

Event » Backight ot o]

storage »wB Saturation (=]

system .
Y E Dare Nt Sharpness (=]

OO OO0

Information
4 » IR Light e

Mirtor ) ON @ OFF

Fiip 0° [-]

Default Refresh save





Figure 5-1
Step 2 

Set picture parameters; please refer to the following sheet for more details about parameter setting. 

	Parameter 
	Note

	Style 
	It is to set the picture style, which includes standard, soft and vivid. 

	Brightness
	It is to adjust the image overall brightness via linear adjustment mode. The larger the number is, the brighter the picture is, and on the contrary it is opposite. The picture get blurry easily when the value is set too big. 

	Contrast
	It is to adjust the picture contrast. The bigger the value is, the bigger the bright contrast becomes, and on the contrary it is smaller. The dark area becomes darker and the bright area becomes overexposed easily when the value is set too big. The picture becomes blurry when the value is set too small. 

	Saturation
	It is to adjust the color darkness and lightness. The color becomes darker when the value is bigger; on the contrary it becomes lighter. The value causes no influence to the overall brightness of the image. 

	Sharpness
	It is to adjust the sharpness level of the picture edge. The bigger the sharpness value is, the more obvious the image edge becomes, the image is likely to generate noise more easily when the value is set too big. 

	Gamma
	It is to change image brightness and improve the dynamic display range of the image via nonlinear adjustment mode. The bigger the value is, the brighter the picture becomes, and on the contrary it is opposite. 

	Mirror
	After mirror is enabled, the monitoring image will be displayed invertedly. 

	Flip 
	It is to change the display direction of the monitoring image. 

It includes following options: 

· Normal: The monitoring picture is normally displayed. 
· Flip mode 1: The monitoring picture is displayed with clockwise rotation 90°
· Flip mode 2: The monitoring picture is displayed with anticlockwise rotation 90°
· 180°: The monitoring picture is displayed upside down. 

Note: 

Please set the video resolution as 1080P or lower when applying flip mode for some devices. 

	EIS
	It can realize electronic anti-flicker function via comparison algorithm of image difference, which can effectively solve the problem of image jittering during application and it can make the image clearer. 


Step 3 
Click “Save” and complete the image parameter config of the camera. 

5.1.1.2  Exposure 
Step 1 
Select “Setup > Camera > Conditions > Exposure”. The system will display the “Exposure” interface which is shown in Figure 5-2. 

[image: image87.jpg])/ IE]

TEcknower

Camera

> Video
> Audio
Network
Event
storage
system
Information

Conditions

[Profile Management] Zoom and Focus

Default

Refresh

save

» IR Light

Profile| Normal

Antificker| Outdoor
Mode | Auto

Autois @ ON O OFF

IDNR@ ON O OFF

Grade (5] 0





Figure 5-2

Step 2 

It is to set the exposure parameter, please refer to the following sheet for more details. 

	Parameter
	Note

	Anti-flicker 
	· You can switch to exposure mode when it is in outdoor mode, it can realize the result in the corresponding exposure mode. 

· 50Hz: When the current is 50Hz, system can auto adjust the exposure according to the environment brightness in case there is any stripe. 

· 60Hz: When the current is 60Hz, system can auto adjust the exposure according to the environment brightness in case there is any stripe.

	Mode 
	It is the camera exposure mode. 
Note: 
· When “Anti-flicker” is “Outdor”, the “exposure mode” can be set as “gain priority” or “shutter priority” mode. 

· Different devices have different exposure modes; please refer to the actual interface. 
It includes the following options: 

· Auto: It can auto adjust the image brightness according to the environment. 

· Gain priority: The device can auto adjust according to the gain range which is set by priority during normal exposure range according to the different scene brightness. The device will auto adjust shutter value if the image brightness fails to reach the effect and the gain value has reached to upper limit or lower limit, which is to make the image reach the best brightness. 
· Shutter priority: The device can auto adjust according to the shutter range which is set by priority during normal exposure range according to the different scene brightness. The device will auto adjust shutter value if the image brightness fails to reach the effect and the shutter value has reached to upper limit or lower limit, which is to make the image reach the best brightness. 

· Iris priority: Iris value is fixed, the device can auto adjust the shutter value if the image brightness fails to reach effect and the shutter value has reached the upper limit or lower limit, the device can auto adjust the gain value to make the image reach the best brightness. 
· Manual: It is to manually set gain value and shutter value, adjust the the displayed brightness of the image. 

	Auto Iris
	It can set the parameter when the camera installs auto iris.

· The lens iris can auto adjust the size according to the environment after auto iris is enabled, then the image brightness will change accordingly.

· The iris value reaches the max when disabling auto iris, the lens iris will not change according to the environment brightness. 

	3D NR 
	It is to process the image with multiframe (at least two frames), it can realize noise reduction of the image by using the interframe information between the previous and latter frame. 

	Grade 
	It can set the parameter when “3D NR” is enabled. 

The bigger the grade is, the better NR effect it can realize. 


Step 3 
Click “Save” to complete the parameter config of camera exposure. 

5.1.1.3 Backlight 

Backlight mode can be divided into BLC, WDR and HLS. 

BLC: it can avoid cucoloris phenomenon of the darker area in the backlight environment.

WDR: It can suppress the overbright area and compensate darker area by enabling WDR, which can make the overall image clear. 

HLC: It is to weaken the high light, which can be applied in the areas such as toll gate, entrance and exit of the parking lot and etc. As for extreme light, it can snapshot the human face in the dark environment and it can realize better effect for the details of the plate number. 
Step 1 

Select “Setup > Camera > Condition > Backlight”, the system will display the interface of “Backlight” which is shown in Figure 5-3. 

[image: image88.jpg]E)/IE]

Tecinower

Camera Conditions  |Profile Management| Zoom and Focus

> Video

> Audio § » Picture

Mode |OFF
Network 5 T~ » Exposure

Event
storage

System » Day &Night
Information » IRLight

Default Refresh





Figure 5-3

Step 2 

Set the backlight parameter. 

· When the “Mode” is set as “Scene Self-adaptation”, the system will auto adjust the image brightness according to the environmental brightness, which is to make the object display clearly in the scene. 
· When the “Mode” is set as “BLC”, it can select default mode or customized mode. 

· When selecting “Default” mode, the system can realize auto exposure according to the environment, which is to make the image in the darkest area clear to be seen. 

· When selecting “Customized” mode, the system can realize exposure upon the selected area after it set customized area, which is to make the image of the selected area reach appropriate brightness. 

·  When the “Mode” is set as “WDR”, it will lower the brightness of the area with high brightness and enhance the brightness of the area with low brightness, which is to make the objects in both high brightness and low brightness area display clearly. 
Note: 

There may be video loss of a few seconds when the camera is switched from non WDR mode to WDR mode. 

· When the “Mode” is set as “HLC”, the system will constrain the brightness of the area with high brightness and decrease the size of the halo area, which is to lower the brightness of the whole image. 
Step 3 

Click “Save” to complete the config of the backlight mode. 

5.1.1.4 WB 

WB is used to restore the white object in the scene by the camera, after WB mode is set, it can make the white object look white in different environments. 
Step 1 

Select “Setup > Camera > Conditions > WB”, the system will display the interface of “WB”, which is shown in Figure 5-4. 

[image: image89.jpg]EV/IE]

TecHNoLGY

Camera Conditions  |Profile Management| Zoom and Focus

> Audio o Mode | Auto [+]

Network 5 o » Exposure
Event » Backight

System » Day &Night
Information » IR Light

[psan__[_Retesn_J[__soe ]





Figure 5-4

Step 2 

Set WB mode. 

· When the “Mode” is set as “Auto”, the system can auto compensate white balance upon different color temperatures, which is to make the image color normal. 

· When the “Mode” is set as “Natural”, the system can auto compensate white balance to the scene without artificial light, which is to make the image color normal. 

· When the “Mode” is set as “Street Lamp”, the system can auto compensate white balance to the outdoor scene at night, which is to make the image color normal. 
· When the “Mode” is set as “Outdoor”, the system can auto compensate white balance to the most outdoor scenes with natural light and artificial light, which is to make the image color normal. 
· When the “Mode” is set as “Manual”, it can manually set the value of red gain and blue gain; the system can compensate the different color temperatures in the environment according to the settings. 

· When the “Mode” is set as “Regional Custom”, it is to set customized area, the system can compensate white balance to different color temperature of the images in the area, which is to make the image color normal. 

Step 3 
Click “Save” to complete the config of WB mode. 

5.1.1.5 Day & Night 
It is to set the switch between color mode and black & white mode. 
Step 1 

Select “Setup > Camera > Conditions > Day & Night” and the system will display the interface of “Day & Night”, which is shown in Figure 5-5. 

[image: image90.jpg]> Video
> Audio
Network
Event
storage
system
Information

Conditions

[Profile Management] Zoom and Focus

Default

Refresh

save

» Picture
» Exposure
» Backlight
»we

» IR Light

Profile| Normal

Hode | Auto
Sensitivity Middle
Delay 65

FEE =





Figure 5-5

Step 2 

Set day & night parameter; please refer to the following sheet for more details. 

	Parameter 
	Note 

	Mode
	It is to set the camera image displayed as color or black & white mode. 

Note: 

The setting of “Day/Night Mode” is not affected by the setting of “Profile Management”. 

It includes the following options: 

· Color: The camera image is displayed as color image. 

· Auto: The camera can auto select color image or black & white image to be displayed according to the environmental brightness. 

· Black & white: The camera image is displayed as black & white image. 

	Sensitivity 
	The parameter can be set when the “Day/Night Mode” is “Auto”. 

It is to set the sensitivity of the switch between image color display and black & white display. 

	Delay
	The parameter can be set when the “Day/Night Mode” is “Auto”. 

It is to set the switch delay between image color display and white & black display. The smaller the delay is, the faster of the switch becomes between color display and black & white display. 


Step 3 

Click “Save” to complete the config of day/night mode. 

5.1.1.6 Defog 
The image quality will become weak when the device is in the environment with fog or haze, you can enable defog function to adjust the image definition. 
Step 1 

Select “Setup > Camera > Conditions > Defog” and the system will display the interface of “Defog” which is shown in Figure 5-6. 

[image: image91.jpg]aJhua

TECHNOLOGY

Profile Day

» Picture o OFF =
» Exposure

» Backlight
» WB

» Day &Night





Figure 5-6

Step 2 

It is to set defog mode according to the actual scene. 

· When the “Mode” is set as “Manual”, it is to manually set intensity and air light mode, the system will adjust the image definition according to the intensity and air light mode which have been set previously. As for air light mode, you can set manual or auto. 
· When the “Mode” is set as “Auto”, the system will auto adjust the image definition according to the actual scene. 

· When the “Mode” is set as “Off”, then the defog function is disabled. 

Step 3 
Click “Save” to complete the config of defog mode. 

5.1.1.7 IR Light 

You can directly set the mode of IR light if the device is equipped with IR light. 
Step 1 

Select “Setup > Camera > Conditions > IR Light” and the system will display the interface of “IR Light” which is shown in Figure 5-7. 
[image: image92.jpg])V IE]

TEcHNOLDGY

Camera

> Video
> Audio
Network
Event
storage
system
Information

Conditions

[Profile Management] Zoom and Focus

Default

Refresh

save

» Picture
» Exposure
» Backlght
»wB

» Day &Night

Profile| Normal

Mode |SmartlR [=]





Figure 5-7

Step 2 

It is to set IR light mode according to the actual scene. 

· When the “Mode” is set as “Manual”, it can manually set the brightness of IR light; the system will realize light compensation to the image according to the IR light intensity. 

· When the “Mode” is set as “Smart IR”, the system can adjust the light brightness according to the actual scene. 

· When the “Mode” is set as “Zoomprio”, the system can auto adjust the IR light according to the brightness change of the actual scene. 
· The system will enable near light by priority when the actual scene becomes dark, the system will enable the far light when the near light fails to meet the requirement of scene brightness even if it is adjusted to the brightest. 

· The system will adjust far light brightness by priority to off and then adjust the brightness of near light when the actual scene becomes bright. The system will always disable far light when the focal length of the lens is adjusted to a certain wide angle, which is to avoid near overexposure. Meanwhile, it can manually set light compensation to slightly adjust the brightness of IR light. 

· When the “mode” is set as “Off”, it will not enable the IR light. 

Step 3 

Click “Save” and complete the config of IR light. 

5.1.1.8 Profile Management 
Step 1 

Select “Setup > Camera > Conditions > Profile Management” and the system will display the interface of “Profile Management”. 

Step 2 

Set profile management. 

· When the “Profile Management” is set as “Normal”, the system will monitor according to the normal config. 
[image: image93.png]> Video

> Audio
Network
Event
Storage
System
Information





Figure 5-8
· When the “Profile Management” is set as “Full Time”, you can select “Always Enable” in “Day” or “Night’, the system will monitor according to the config of “always enable”.
[image: image94.png](ELlLis AT T TR AT AT T

Camera
>

> Video

> Audio
Network
Event

Storage
System
Information




Figure 5-9

· When the “Profile Management” is set as “Schedule”, you can set some period as day and another period as night, for example, if it sets 0:00~12:00 as day, 12:00~24:00 as night, then the system will monitor by adopting corresponding config in different periods. 

[image: image95.png]@hua

Camera
o

> video

> Audio

Network

Event

storage

system

Information

Conditions

Profile Management

Period sefiing

Profile Management||  Zoom and Focus

€ Nomal € FullTime @ Schedule

&

y M Night

0

Default

Refiesh





Figure 5-10 

Step 3 
Click “Save” to complete the settings of profile management. 

Note: 

Click “Default” to restore the device to default config; click “Refresh” to check the latest config file of the device. 

5.1.1.9 Zoom and Focus 

Note: 
Only motorized vari-focal devices support focus and zoom function. 
Step 1 

Select “Setup > Camera > Conditions > Zoom and Focus” and the system will display the interface of “Zoom and Focus” which is shown in Figure 5-11. 

[image: image96.png]@hua

Camera Conditions

> Video

> Audio
Network
Event
storage
system
Information

oom wioE (=) (F

20 ) Restore Al

Focus NEAR

(=, (&) PR speed [20 =] | ClAutoFocus Refiesh





Figure 5-11

Step 2 

Adjust the focal length of the lens. 

After it is zoomed, set “Speed” and press “+”, “-“or drag the sliding block directly to adjust. 

Step 3 

Adjust the lens definition. 

After it is focused, set “Speed” and press “+”, “-“or drag the sliding block directly to adjust. 

Note: 

· Speed is used to set the length by pressing “+” and “-“.
· After adjusting the focal length of the lens or click “Auto Focus”, the device will auto adjust the image definition, it is not allowed to implement other lens operation during auto focus. 

· After several times of zoom and focus, the image fails to be adjusted clear, click “Restore All’ to reset the lens to zero and remove the accumulative error of the lens. 

· Click “Refresh” and the device will automatically synchronize the hardware to the location of sliding block of lens zoom and focus. 
5.1.2 Video 
5.1.2.1 Video 
Step 1 

Select “Setup > Camera > Video > Video” and the system will display the interface of “Video” which is shown in Figure 5-12 or Figure 5-13. 
[image: image97.png]aJhua

Camera Video Snapshot Overlay Path

> Conditions.
Wain Stream Sub Stream
Network Enable Sub Stream 2 v
Event Encode Mode H264H 2 Encode Mode H26eH v
Storage Smart Codec OFF v @ Resolution D1 (704°480) v
System Resolution 2304x1296 (2304°1296 ¥ Video Clip
icnates Frame rate(FPS) | 30 2 Frame rate(FPS) 6 v

BitRate Type v BitRate Type CBR v

Reference Bit Rate Reference BitRate  48-806KDIS

BitRate 2 BitRate 8% 2
IFrame interval |60 @30~150) |Frame Interval [ 12 (6-150)
@] Watermark Setings.

Watermark Character | DigitalCCTV





Figure 5-12 (Non fisheye)

[image: image98.jpg]Network
Event
Storage
system
Information

Video

Installation Mode

Record Mode

Main Stream

Code-Stream Type
Encode Mode
Resolution

Frame rate(FPS)
BitRate Type
Reference Bit Rate
BitRate

IFrame Interval
sve

Watermark Settings

Sub stream
Enavle

General =] Code-Stream Type

H264 =] Encode Mode.

24322432 (2432°2432)[ | Resolution

15 [=] Frame rate(FPS)
CBR BitRate Type.

Reference Bit Rate

30
1(of)

(15~150) IFrame Interval

=]
] BitRate
=]

sve

Watermark Character  DigitalCCTV.

Sub Stream 1
General

H.264

640x640 (640°640)
15

CBR

(15~150)

ElE EEEEEE




Figure 5-13 (Fisheye)
Step 2 

Set video bit stream, please refer to the following sheet for more details about the parameters. 
	Parameter 
	Function 

	Installation Mode
	It will display the parameter when the device is fisheye. 

There are three installation modes for fisheye which are ceiling, wall mount and ground installation, please select installation mode according to the actual installation scene of the fisheye. 
The system will begin to switch after switching installation mode, it will prompt successfully saved after it is switched successfully.

Note: 

The device end will output the dewarped video stream after configuring installation and record mode, when the device is accessed to third-party platform, it will directly display the dewarped image on the third-party platform. 

	Record Mode
	It will display the parameter when the device is fisheye. 

The system will begin to switch after record mode is switched, it will prompt saved successfully after it is switched successfully. 

The record mode will change according to the different installation modes. 

· 1O: the original picture which is not dewarped. 
· 1P: 360°rectangular panorama.

· 2P: The mode can be set when the “Installation Mode” is set as “Ceiling” or “Ground”. It is the 2 related 180°rectangular image, the two subwindows can both form 360°panorama anytime. 
· 1O+3R: original image + 3 independent sub images, both the sub image and sub boxes in the original image can support zoom and movement.
· 1R: Original image + independent sub image, the sub boxes of the original image support zoom and movement. 
· 4R: original image + 4 independent sub images, both the sub image and sub boxes in the original image support zoom and movement. 

· 2R: Original image + 2 independent sub images, the sub boxes of the original image can support zoom and movement, the sub image supports up and down movement, which can change the vertical angle of view. 

Note: 

The device end will output dewarped video stream after configuring installation and record mode, when the device is accessed to the third-party platform, it will directly display the dewarped image on the third-party platform. 



	Sub Stream Enable 
	Select “Enable” to enable sub stream.

The device supports enabling sub stream 1 and sub stream 2 at the same time.

	Smart Codec 
	It can enhance image compression performance and reduce storage space by enabling intelligent encoding.
Note: 

After intelligent encoding is enabled, the device will not support third stream, ROI or intelligent event detection, please refer to the actual interface for more details. 

	Encode mode 
	There are five options: H.264 (main profile standard, H.264H (high profile standard), H.264B (Baseline Profile), H.265 (main profile standard) and MJPEG encode.

· The H.264, H.264H both are H264 bit stream. H.264 is the Main Profile encode and you need to enable the sub stream function in your camera and set the resolution as CIF. Then you can monitor via the Blackberry cell phone. 
· The H.265 is the main profile encode mode. 
· MJPEG: In this encode mode, the video needs to large bit stream to guarantee the video definition. You can use the max bit stream value in the recommend bit to get the better video output effect. 

	Resolution 
	There are multiple resolutions. You can select from the dropdown list. 

For each resolution, the recommended bit stream value is different.

Note: When video is under rotating status, you cannot set resolution higher than 1080P (excluding 1080P).

	Video Clip 
	The function is only supported by sub stream 2, please refer to “Video Clip” for more details below the sheet. 

	Frame Rate (FPS)
	PAL: 1～25f/s，1-50f/s  NTSC: 1～30f/s or 1~60f/s.
The frame rate may vary due to different resolutions. 

	Bit Rate Type 
	There are two options: VBR and CBR. 

Please note, you can set video quality in VBR mode. 

Under MJPEG mode, only CBR is available. 

	Recommended Bit 
	Recommended bit rate value according to the resolution and frame rate you have set. 

	Bit Rate 
	· In CBR, the bit rate here is the max value.  In dynamic video, system needs to low frame rate or video quality to guarantee the value. 

· The value is null in VBR mode. 

· Please refer to recommend bit rate for the detailed information. 

	SVC 
	Frame rate can be encoded by layer. It is a flexible encoding method. By default, it is 1 as 1 layer. You also can set 2/3/4 layers.

	I Frame 
	Here you can set the P frame amount between two I frames. The value ranges from 1 to 150.  Default value is 50.

Recommended value is frame rate *2.

	Watermark Settings
	Select “Watermark Setting” and enable watermark function. After the watermark function is enabled, you can check if the video is tampered via verifying watermark character.

	Watermark Character 
	It is the character of watermark verification, it is DigitalCCTV by default. 


· Video Clip 

1. Select “Sub Stream 2” in the drop-down box, click “Enable”. 

2. Enable “Video Clip”, click[image: image99.jpg]


.
The system will display the interface of “Area”, which is shown in Figure 5-14. 

[image: image100.jpg]Area

Resolution

D1 (704°430)

]

save

Cancel





Figure 5-14

3. Select the needed resolution and clip the needed image on the interface, which is shown in Figure 5-15. 

4. Click “Save”. You can check the clipped video in the preview interface (the sub stream 2 preview interface only displays the clipped area), which is shown in Figure 5-15. 

[image: image101.jpg]TR s e
SR AL

& i




Figure 5-15
Step 3 

Click “Save” to complete video stream setup. 

5.1.2.2 Snapshot 
The snapshot interface is shown as in Figure 5-16.
[image: image102.png]@Jhua

Camera
v
Network
Event

storage
system
Information

Video

Snapshot Type
Image
Qualty

Interval

Snapshot

General

080P (1920°1080)

1S

Default





Figure 5-16
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Snapshot type 
	There are two modes: general (schedule) and Event (activation).

	Image size 
	It is the same as the resolution of main stream. 

	Quality 
	It is to set the image quality. There are six levels. 

	Interval 
	It is to set snapshot frequency. Optional1~7s/picture, customized. 


5.1.2.3 Video Overlay 
The video overlay interface is shown as in Figure 5-17.

[image: image103.jpg]E) UE]

> Conditions

R~ 4 » Privacy Masking SGN=1C"

» Channel Title
> Audo

» Time Title
Network

Text Overl:

Event ’ S
Storage » Picture Overlay
System » Counting

Information

e -

[ Defur [ Refesh ][ save





Figure 5-17
[image: image104.jpg]E)/E]

> Conditions

» Privacy Masking Enable O Disable

it P » Channel Title Input channel tite
> Audio e

(s » Time Title [1rc ]
Event i ' » TextOverlay

S » Picture Overlay

System > » Counting

Information





Figure 5-18
[image: image105.jpg]@hua

> Condiion:

© Enable () Disable

» Privacy Masking
»

» ChannelTie (] WeekDisplay

> Audio

Network bl T

Event. . > TextOveray
Storage » Picture Overlay.

System » Counting
Information

(oo [ RetenJ[see ]





Figure 5-19
[image: image106.jpg]aJhua

> Conditions

» Privacy Masking  ©) Enable

>
» Channel Title Input Text:
> Audio put

— e
Network /7

Event : o Em
» PewreOveray ||

» Counting 1

Storage
System

Information





Figure 5-20
[image: image107.jpg]@hua

> Conditions

> Audio
Network
Event
Storage
System

Information

» Privacy Masking
» Channel Titie
» Time Title

» TextOverlay

» Counting

Upload Picture

Requirement for picture upload
1. Max size is 16k

2. Max resolution is 128x128 pixels
3. 256 colors, bmp format.





Figure 5-21
[image: image108.jpg]E)/E]

> Conditions
> Audio
Network
Event
Storage
System

Information

Privacy Masking
» Channel Title Statistics Type
» Time Title Enter Number

Leave Number

» Text Overlay
» Picture Overlay =~ Text Align





Figure 5-22
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Privacy Masking
	· Here you can privacy mask the specified video in the monitor video. 

· System max supports 4 privacy mask zones.

	Time Title
	· You can enable this function so that system overlays time information in video window. 

· You can use the mouse to drag the time tile position. 

	Channel Title 
	· You can enable this function so that system overlays channel information in video window.

· You can use the mouse to drag the channel tile position.

	Text Overlay
	· You can enable this function to overlay text in video window. 
·  Private protocol is connected to mobile NVR, support GPS information overlay.

· You can use the mouse to drag location box to adjust its position. Alignment include align left and align right.

	Picture Overlay
	· You can enable this function to display overlay picture. Click on disable to turn it off. 

· Click on Upload Picture to overlay local picture into monitoring window. You can drag the yellow box to move it. 

Note:

You cannot enable text and picture overlay at the same time.  

	Counting 
	· Check “Enable” to display the counting statistics data in the video monitoring window; check “Disable” not to display.
· There are enter number and leave number for statistics type and left align and right align for OSD info.

	Refresh
	· Set privacy mask, channel title, time title, location, overlay and save the change. You can click on Refresh to see effect. 

	Default 
	· Click it to restore default config. 

	Save
	· Click it to complete video settings. 


5.1.2.4 ROI 
Note: Some series don’t support ROI setup function.  

[image: image109.jpg]@hua

Paih

> Condiions
>

Enable (@) Disable

Image Qual v
e image Qualiy | 6(best)

Network
Event
Storage
System
Information

[ | —





Figure 5-23
[image: image110.jpg]Snapshot Overtay Path

Enable (@) Disable
o Image Quslity | 6{best) v
> hudio ge Quality | 6{best)

Network
Event
Storage
System
Information

Remoie || Deiee__] OriRoncict)

[ | —





Figure 5-24 

	Parameters
	Note 

	Enable 
	Check “Enable”, then it will display the ROI in the video monitoring window;

Check “Disable”, then it won’t display.                          

	Image Quality 
	· Set the image quality of ROI, ranging from 1～6, default is 6. 
Note: 

For fisheye device, it ranges from 1~6 (best), default is 6 (best)

· Able to set area block, max 4 areas. 


5.1.2.5 Path 

The storage path interface is shown as in Figure 5-25.

Here you can set snap image saved path and the record storage path.
· The default monitor image path is C:\Users\admin\WebDownload\LiveSnapshot. 
· The default monitor record path is C:\Users\admin\WebDownload\LiveRecord.
· The default playback snapshot path is C:\Users\admin\WebDownload\PlaybackSnapshot.

The default playback download path is C:\Users\admin\WebDownload\PlaybackRecord. 

The default playback cut path is: C:\Users\admin\WebDownload\VideoClips.

Note:

Admin is locally logged in PC account. 

Please click the Save button to save current setup. 
[image: image111.png]&)/ IE]

Camera
> Condtions
[ veeo |
> Audio
Network
Event
Storage
system
Information

Video

Live Snapshot
Live Record

Playback Snapshot
Playback Download

Video Clips

Snapshot

C:\Users\admin\WebDownload\LiveSnapshot
C:\Users\adminWebDownload\LiveRecord
C:\Users\admin\WebDownload\PlaybackSnapshot

C:\Users\admin\WebDownload\PlaybackRecord

C:\Users\admin\WebDownload\VideoClips

Browse.
Browse.
Browse.
Browse.

Browse.





Figure 5-25
5.1.3 Audio 

Please note some series devices do not support audio function.
5.1.3.1  Audio
The audio interface is shown as below. See Figure 5-26.

[image: image112.jpg]Audio

Encode
Main Stream
Network [v] Enable
Event Encode Mode GT1A [=]
storage ‘Sampling Frequency | 16k [=]
system
Sub Stream
Information
Enable Sub Stream 1
Encode Mode GT11A

‘Sampling Frequency | 16k

[=]
[=]
[=]

Atribute
Audioln Type Lineln
Noise Fiter Enable
Microphone Volume (=] 0
Speakervolume (=) 0

Defaut Refresh save





Figure 5-26
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Enable 
	You can enable audio only when video is enabled. 

After selecting the “Enable” of main stream or sub stream, the network transmission stream is the audio/video composite stream, otherwise it only includes video image. 

	Encode mode
	The encode mode of the main stream and extra stream include G.711A, G.711Mu, G.726 and ACC. The default mode is G.711A. 
The setup here is for audio encode mode and the bidirectional talk encode both.

	Sampling Frequency
	The sampling frequency of audio. It includes the following options: 
· 8K 

· 16K 

· 32K 

· 48K

· 64K 

	AudioIn Type 
	Two modes to select: LineIn, Mic. 
Device needs to connect external audio input source under LineIn mode, and it doesn’t need to connect external audio input source under Mic mode. 

	Noise Filter
	Enable the function and it can filter relevant noise.

	Microphone Volume 
	Adjust microphone volume from 0～100.
Note:
Supported by some devices.

	Speaker Volume
	Adjust speaker volume from 0～100.
Note:
Supported by some devices.


5.1.3.2 Alarm audio
[image: image113.jpg]@lhua

> Conditions
> Video

>

Network
Event

Storage

system
Information

Alarm Audio

alam1.pem

alarm2.pem

Add Audio File





                                                       Figure 5-27
[image: image114.png]Add Audio File

@ Record € Upload

Audio File pem

Record





                                                          Figure 5-28
Audio management currently supports PCM format to record and PCM, wav two forms to upload，and can select an audio file for the audio alarm linkage.
The way to download the web alarm playback audio file into local is as follows: 
Step 1  Use the left mouse button to click the hollow circle [image: image115.png]


 in the “choice” column on the left, and shows[image: image116.png]


，which means effective choice of alarm audio. 
Step 2  Use the right mouse button to click [image: image117.png]


，select “save target as” to download.
5.2 Network 

5.2.1 TCP/IP

The TCP/IP interface is shown as in Figure 5-29. It supports IPv4 and IPv6. IPv4 supports static IP and DHCP. IPv6 supports static IP only. When users manually modify IP address, WEB will automatically jump to the new IP address.
[image: image118.jpg]@hua

Camera

> Connection
> PPROE

> DDNS

> IP Fiter

> SUTP(Emai)
> UpnP

> SNMP

> Bonjour

> Multcast

> Wil

> 802.1x

> Qos

Event
Storage
System
Information

TCPIIP

HostName

Ethemet Card
Mode

WAC Address

1P Version

1P Address

Subnet mask
Default Gateway
Preferred DNS Server|

Alternate DNS Server

IPC

Wire(DEFAULT)
& static € DHCP

02 . a9

8
8 .8 . 4

[ Enable ARP/Ping to set IP address senice

Setas Default

Default

Reffesh





Figure 5-29
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Host Name 
	It is to set current host device name. It max supports 15 characters.  

	Ethernet Card
	Please select the Ethernet port. Default is wired. 
Please note you can modify the default Ethernet card if there is more than one card.

	Mode
	There are two modes: static mode and the DHCP mode. Select DHCP mode, it auto searches IP, and you cannot set IP/subnet mask/gateway. Select static mode, you must manually set IP/subnet mask/gateway. 

	Mac Address 
	It is to display hose Mac address. 

	IP Version 
	It is to select IP version. IPV4 or IPV6.
You can access the IP address of these two versions.

	IP Address 
	Please use the keyboard to input the corresponding number to modify the IP address and then set the corresponding subnet mask and the default gateway. 

	Preferred DNS
	DNS IP address.

	Alternate DNS 
	Alternate DNS IP address. 

	Enable ARP/Ping set device IP address service. 
	You can use ARP/Ping command to modify or set the device IP address if you know the device MAC address. 
Before the operation, please make sure the network camera and the PC in the same LAN. This function is on by default. 

You can refer to the steps listed below. 
Step 1: Get an IP address. Set the network camera and the PC in the same LAN.

Step 2: Get the physical address from the label of the network camera.

Step 3: Go to the Run interface and then input the following commands. 

arp  –s  <IP Address>  <MAC>  

ping  –l  480  –t  <IP Address>

Such as：arp  -s  192.168.0.125  11-40-8c-18-10-11

ping  -l  480  -t  192.168.0.125

Step 4: Reboot the device. 

Step 5: You can see the setup is OK if you can see there are output information such as “Reply from 192.168.0.125 …” from the command output lines.  Now you can close the command line. 

Step 6: Open the browse and then input http://<IP address>. Click the Enter button, you can access now. 


5.2.1.1 Easy4ip

Click “Enable” to connect the device to network, and then you can connect to the device via mobile client of Easy4ip after “Online” is displayed on the interface (connection mode: add device serial number or scan QR code). See Figure 5-30. 

[image: image119.jpg]@lhua [ e ronsoc | oo | mem | ioson ]

Camera Easy4ip
Network

Connection

Status

PPPOE B
DDNS

1P Filter
‘SMTP(EMail)
UPnP

SNMP
Bonjour
Multicast

8021x

aos

HTTPS
Event
Storage
System
Information




Figure 5-30

5.2.2 Connection 
5.2.2.1 Connection
The connection interface is shown as in Figure 5-31.

[image: image120.jpg]@lhua T ST TN TN

Camera Connection ONVIF
Network
> TCPIP
%

PPPOE UDP Port
DDNS HTTP Port
1P Fiter RTSP Port
SMTP(EMai) HTTPS Port
UPnP
SNMP

Bonjour

Multicast
802.1x
Qos

HTTPS
Event
Storage
System
Information




Figure 5-31
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Max connection 
	It is the max Web connection for the same device. The value ranges from 1 to 20. Default connection amount is 10.

	TCP port 
	Port range is 1025~65534. The default value is 37777. You can input the actual port number if necessary. 

	UDP port 
	Port range is 1025~65534. The default value is 37778. You can input the actual port number if necessary.

	HTTP port 
	Port range is 1025~65524. The default value is 80. You can input the actual port number if necessary.

	RTSP port
	· The default value is 554. Please leave blank if use default. User uses QuickTime or VLC can play the following formats. BlackBerry can play too. 

· Real-time monitoring URL format, please require real-time RTSP media server, require channel no., bit stream type in URL. You may need username and password. 

· User uses BlackBerry need to set encode mode to H.264B, resolution to CIF and turn off audio. 

URL format is:
rtsp://username:password@ip:port/cam/realmonitor?channel=1&subtype=0

Username/password/IP and port. 

The IP is device IP and the port default value is 554. You can leave it in blank if it is the default value. 

Follow standard RTP protocol and when encode mode is MJPEG, the max resolution only supports 2040*2040. 

	HTTPs Port
	HTTPs communication port, range is 1025~65534, default is 443. 


Note:
· 0～1024, 37780～37880, 1900, 3800, 5000, 5050, 9999, 37776, 39999, 42323 are all special ports. User cannot modify them. 
· Avoid using default port value of other ports. 

5.2.2.2 ONVIF
ONVIF（Open Network Video Interface Forum），this standard describes network video mode, interface, data type and data interaction mode. ONVIF Standard’s aim is to achieve a network video frame agreement and makes the network video products (including video front-end, video equipment, etc.) from different manufacturers completely compatible. 
ONVIF function is on by default. 
[image: image121.jpg]E)/IE]

TEckNOLGY

Camera

Network

>
>
>
>
>
>
>
>
>
>
>
>
>

TCPIP

PPPOE
DDNS

1P Filter
‘SMTP(EMail)
uPnP

SNMP
Bonjour
Multicast
802.1x

aos

HTTPs

Event

Storage

System

Information

Connection

ONVIF

on

OFF

uit





Figure 5-32
5.2.3 PPPoE

The PPPoE interface is shown as in Figure 5-33.
Enter the PPPoE username and password which are provided by ISP (Internet Service Provider), and click “Enable”. The network camera will automatically establish network connection in the mode of PPPoE after it is enabled, after it is successful, the IP of the “IP Address” will be automatically modified into the dynamic IP address of the acquired WAN.  
Note:

After PPPoE dial-up is successful, it needs to log in the device via the IP which is set before dial-up; in the PPPoE setup interface, it will display the registered IP address, and then it can visit the IP address via client. 

Please disable UPnP when clicking PPPoE enable, which is to avoid causing influence to PPPoE dial-up. 
[image: image122.png]@hua

Camera
Network

> TePnp

> Connection
>

> DDNS

> IP Filter

> SMTP(Emai)

> UPnP.
> SNMP

> Bonjour

> Mutticast
>8021x
>Qos
Event
Storage
System
Information




Figure 5-33
5.2.4 DDNS
The DDNS interface is shown as in Figure 5-34.

The DDNS is to set to connect the various servers so that you can access the system via the server. Please go to the corresponding service website to apply a domain name and then access the system via the domain. It works even your IP address has changed. When the device connects to WLAN, you should disable UPnP. 
[image: image123.jpg]@lhua ST T TN ST

Camera
Network
TCPIP Dahua DDNS [l
Connection

PPPOE Mode o Aut Manual

in Nams

1P Filter Usermame
‘SMTP(EMail)
uPnP
SNmP

Bonjour

Multicast
802.1x
aos

HTTPS
Event
Storage
System
Information




Figure 5-34
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Server Type
	You can select DDNS protocol from the dropdown list: CN99 DDNS, NO-IP DDNS, Dyndns DDNS, and Dahua DDNS. The default is Dahua DDNS. 

	Server Address
	DDNS server IP address

CN99 DDNS

Server address: www.3322.org
NO-IP DDNS

Server address: dynupdate.no-ip.com

Dyndns DDNS

Server address: members.dyndns.org

Dahua DDNS

Server address：www.dahuaddns.com

	Mode 
	Default is auto, able to choose manual 

	Domain Name 
	Both auto and manual are “MAC addresss.dahuaddns.com”by default, able to set prefix manually. 

	Username
	The user name you input to log in the server, optional. 



The CN99 DDNS interface is shown as in Figure 5-35. 
[image: image124.png]@lhua —_—————yry————
‘Camera @

Network

;

none

> SMTP(Emai)
> UPnP.
> SNMP

> Bonjour
> Mutticast
>8021x
>Qos
Event
Storage
System
Information




Figure 5-35
	Parameter 
	Function 

	Server Type
	You can select CN99 DDNS protocol 

	Server Address
	Under CN99 DDNS the default server address is “www.3322.org”

	Domain Name
	Self-defined domain name 

	Username
	The user name you input to log in the server.

	Password 
	The password you input to log in the server

	Refresh Period 
	The refresh period of the connection between the device IP and the server, default is 10 minutes. 


5.2.5 IP filter
The IP filter interface is shown as in Figure 5-36.

You can enable IP filter function so that some specified IP/MAC user can access the network camera. 
You can add IP address or IP segment. 

If you do not check the box here, it means there is on access limit. 

Here you can add IP address and MAC address. You must add these addresses before enabling the trusted sites. 

Please note: You must set MAC address in the same network segment. 

[image: image125.png]@hua

Camera IP Filter

Network T

Trusted Sites.
> Connection 1P address /MAC address
> PPPOE

> SMTP(Email)

> Muticast

T Add IPIMAC.

> Qos.

[owaut

Event

Storage
system
Information

[ Remover |





Figure 5-36
5.2.6 SMTP （e-mail)
The SMTP interface is shown as in Figure 5-37.

[image: image126.png]@hua

Camera
Network

> TCPIP

> Connection
> PPPOE

> DONS

> IP Fiter

> UPnP.

> SNMP

> Bonjour

> Mutticast

> 802

>Qos
Event
Storage
system
Information

SMTP(Email)

SMTP Server
Port
Anonymity
Usemame
Password

Sender

Authentication

Tite

Interval

Health Mail

[9] Attachment

0 Second (0~3500)

Update Period Second (1~3600)

Emai Test

[owaut





Figure 5-37
Please refer to the following sheet for detailed information.

	Parameter 
	Function 

	SMTP Server 
	Input server address and then enable this function. 

	Port 
	Default value is 25. You can modify it if necessary.

	Anonymity 
	For the server which supports the anonymity email function, it won’t display the information of the sender. 

	User Name
	The user name of the sender email account.

	Password
	The password of sender email account.

	Sender
	Sender email address.

	Authentication (Encryption mode) 
	You can select SSL, TLS or none. 

	Title (Subject)
	Input email subject here. 



	Attachment 
	System can send out the email of the snapshot picture once you check the box here. 

	Mail receiver
	Input receiver email address here. Max three addresses.



	Interval 
	The send interval ranges from 0 to 3600 seconds. 0 means there is no interval.

Please note system will not send out the email immediately when the alarm occurs. When the alarm, motion detection or the abnormity event activates the email, system sends out the email according to the interval you specified here. This function is very useful when there are too many emails activated by the abnormity events, which may result in heavy load for the email server.

	Health mail enable 
	Please check the box here to enable this function. 



	Email test 
	The system will automatically sent out a email once to test the connection is OK or not .Before the email test, please save the email setup information. 


5.2.7 UPnP
It allows you to establish the mapping relationship between the LAN and the public network.

Here you can also add, modify or remove UPnP item. For UPnP on different routers, you must disable UPnP function. See Figure 5-38.

In the Windows OS, From Start->Control Panel->Add or remove programs. Click the “Add/Remove Windows Components” and then select the “Network Services” from the Windows Components Wizard. 
Click the Details button and then check the “Internet Gateway Device Discovery and Control client” and “UPnP User Interface”. Please click OK to begin installation. 
Enable UPnP from the Web. If your UPnP is enabled in the Windows OS, the network camera can auto detect it via the “My Network Places”.
Under manual mode, you can modify external port. Under auto mode, select idle port for auto port mapping without user modification.
[image: image127.png]@hua

Camera
Network

> TePIP

> Connection
> PPPOE

> DDNS.

> IP Fiter

> SMTP(Emai)

> SNMP

> Bonjour
> Muticast
>8021x
>Qos
Event
Storage
system
Information

Port Mapping List

Router State Mapping Failed

Protocol internal Port
WebService:TCP

PrivSenvice TCP

PrivSenvice:UDP
RTSPSenice TCP.

Extemal Port
Mapping Failed
Mapping Failed
Mapping Failed
Mapping Failed





Figure 5-38
5.2.8 SNMP
The SNMP interface is shown as in Figure 5-39 and Figure 5-40.

The SNMP allows the communication between the network management work station software and the proxy of the managed device. Please install the software such as MG MibBrowser 8.0c software or establish the SNMP service before you use this function. You need to reboot the device to activate the new setup. 

[image: image128.png]@hua

Camera SNMP
Network
> TePnp

NMP

> Connection
> PPPOE

> DONS

> IP Fiter

> SMTP(Email)
> UPnP.

>

> Bonjour

> Mutticast
>8021x
>Qos

Event
Storage
System
Information

Default





Figure 5-39
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	SNMP Version
	· Check SNMP v1, device only process v1 info. 

· Check SNMP v2, device only process v2 info. 

· Check SNMP v3, can set username, password and encryption method. Server calibrate corresponding username, password and encryption method too access device and v1/v2 are unavailable. 

	SNMP port 
	The listening port of the proxy program of the device. It is a UDP port not a TCP port. The value ranges from 1 to 65535. The default value is 161

	Community
	It is a string, as command between management and proxy, , defining a proxy,  and a manager’s authentication. 

	Read community 
	Read-only access to all SNMP targets, default is public.
Note: Only number, letter, _, and – supported. 

	Write community 
	Read/write access to all SNMP targets, default is private.

Note: Only number, letter, _, and – supported.

	Trap address
	The destination address of the Trap information from the proxy program of the device. 



	Trap Address
	Address where to send Trap message.

	Trap Port
	Port which send Trap message, default is 162, range 1~65535.


[image: image129.png]@hua

Camera SNMP

‘SNMP Version SNMP V1 [ ] SNMPV2 [7] SNMPv3

ton SNMP Port 161 (1~65535)
> PPPOE Read Community public
Write Community private
Trap Address
Trap Port
> UPnP
> Read-only Usemame
> Bonjour Authentication Type
> Mutticast Authentication Password
Encryption Type © CBCDES
Encryption Password

Event

Storage Readgwiite Username

system Authentication Type.

Information Authentication Password
Encryption Type © CBCDES

Encryption Password





Figure 5-40
Check SNMP v3 version and SNMP port, read community, write community, Trap address, Trap port are same with SNMP v1 and SNMP v2 versions. Only when SNMP version is SNMP v3, users need to configure parameter in chart.

	Parameter
	Function

	SNMP Version
	SNMP v3

	Read-only Username
	Default is public.
Note:

Name only can be number, letter and underline. 

	Read/Write Username
	Default is private.
Note:

Name only can be number, letter and underline. 

	Authentication
	You may select MD5 or SHA, default is MD5.

	Authentication Password
	Password not less than 8 characters. 

	Encryption 
	Default is CBC-DES.

	Encryption Password
	Password not less than 8 characters. 


5.2.9 Bonjour

The Bonjour interface is shown as below. See Figure 5-41.

Bonjour is based on the multicast DNS service from the Apple. The Bonjour device can automatically broadcast its service information and listen to the service information from other device.
You can use the browse of the Bonjour service in the same LAN to search the network camera device and then access if you do not know the network camera information such as IP address.  
You can view the server name when the network camera is detected by the Bonjour. Please note the safari browse support this function. Click the “Display All Bookmarks: and open the Bonjour, system can auto detect the network camera of the Bonjour function in the LAN. 
[image: image130.png]@hua

Camera Bonjour
Network

> TCPIP

> Connection
> PPPOE

> DONS

> IP Fiter

> SMTP(Email)
> UPnP.

> SNMP

>

> Mutticast
>8021x
>Qos

Event
Storage
System
Information





Figure 5-41
5.2.10 Multicast

The multicast interface is shown as in Figure 5-42.
Multicast is a transmission mode of data packet. When there is multiple-host to receive the same data packet, multiple-cast is the best option to reduce the broad width and the CPU load.  The source host can just send out one data to transit. This function also depends on the relationship of the group member and group of the outer. 
Note:

· Open preview, streaming media protocol, select multicast, and monitor via multicast format. 

· Here you can set multicast address and port. You also need to go to Live interface to set the protocol as Multicast. 
[image: image131.png]@hua

Camera Multicast
Network
> TePnp

Main Stream
Enable

> Connection
Muticast Ad

> PPPOE

> DONS

> IP Fiter

> SMTP(Email)

> UPnP.

> SNMP

> Bonjour

>

>8021x

>Qos

Event

Storage

System

Information

Sub Stream

Enal

Multcas

Port

40002





Figure 5-42
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Enable
	Select to enable multicast function. Main stream and sub stream cannot be used at the same time. 

	Multicast address
	Main/sub stream multicast default address is 224.1.2.4 and its range is 224.0.0.0～239.255.255.255.

	Port
	Multicast port. Main stream is 40000,  sub stream1 is 40016,sub stream2 is 40032 and the range is 1025～65534.


5.2.11 4G

Note: This function is only for series with 4G module. 

4G is a mobile communication system which makes wireless communication integrates with Internet and multimedia communication. 

5.2.11.1 Dialing Setting 

[image: image132.jpg]Dialing Setting | Mobile Settings

e Wireless NetType | Auto [=] @) Enabte

Connection AN 3gnet
5 AuorzeMode  [No =]
1P Fitter Dial-up Number | *9%#

Usemame

Password

Bonjour Update Period 30

Ml

Time Range Setup

Wireless Network State
1P Address

storage Wireless Signal  WCDHA

system FOD-LTE o
Information

Default Refresh





Figure 5-43
	Parameter 
	Function 

	Wireless Net Type
	Auto, WCDMA, FDD-LTE optional, you can select according to the operator.

	Enable
	Check to enable 4G module.

	APN
	It shall be acquired from the operator according to the setup of operator. 

	Authentication Mode
	Auto, PAP and CHAP, the authentication algorithm is different according to different operators. 

	Dial-up Number 
	The number is different according to different operator, it can be acquired from operator. 

	User name
	It shall be acquired from the operator. 

	Password
	It shall be acquired from the operator. 

	Update Period
	It is the period beyond the dial-up period which has been set, it is 30s by default, which means the device can auto acquire 4G dial-up status, cut off dial-up to save flow.

	Time Range 
	It is the time period to set dial-up. It can activate the function via message to enable/disable dial-up function if it is beyond the time period. 

	IP Address
	It will display the acquired IP after the dial-up is successful via 4G.  


5.2.11.2 Mobile Setting 

[image: image133.jpg]Camera Dialing Setting | Mobile settings

wenE ) Message Send ) Message Actiation ] Phone Actvation

Receiver Sender Caller
DON

1P Fiter

SHTP(Email)

UPnP

Bonjour

!
Event Message
Event

storage
system

Default Refresh

Information





Figure 5-44
Please refer to the following sheet for detailed information.
	Parameter 
	Function 

	Message Send
	In the event, after the corresponding message activation is

enabled, it will send message to the phone of receiver when

event happens. It needs to enable the function of message

activation function in the event interface when using the

function. 

	Message Activation
	It can send message ON/OFF to the SIM card inside the device by the mobile phone in the list, which is to enable /disable the dial-up function and send REBOOT to restart the device. 

	Phone Activation 
	You can use the mobile phone in the calling list to call the SIM card inside the device to enable dial-up function. 


5.2.12 WIFI
Note:
Some series devices don’t support WIFI and WPS function.
WPS function is supported by the devices with WPS button. 
5.2.12.1 WIFI

WIFI info can show the name, status, IP information of the Current Hot Spot. Usually please “Refresh” WIFI work information after reconnection to ensure the real-time display of the work status, because it often takes some time to connect WIFI Hot Spot, which depends on the network signal strength. 
[image: image134.jpg]> TCRIP
> Connection
> PPROE

> DDNS

> IP Fitter

> SMTP(EMail)
> UpnP

> SNMP

> Bonjour

> Multcast

>

> 802.1x

> Qos

Event
storage
System
Information

™ Enable

Add SSID

Search SSID

WIFIINFO
Gurrent Hot Spot
1P address
Subnetmask
Default Gateway

Refiesh





                                                         Figure 5-45
WIFI setting method is as follows: 
Step 1  Click [image: image135.png]I~ Enable



，show as [image: image136.png]I Enable



，means enabling WIFI function.
Step 2 Click “ Search SSID”, and shows the wireless network hot spot of the current network camera environment in the list. 
[image: image137.jpg]Camera

> TePIP
> Connection
> PPPOE

> DDNS

> IP Filter

> SMTP(EMail)
> UPnP.

> SNuP

> Bonjour

storage

system

Information

WIFI

¥ Enable

12345
sa70test
TP-LINK_455EFS
Andioidap
TP-LINK 425795
NETGEARG4

TP-LINK_NUIAN

APSKAES

WPANPA2 PSKTKIP+AES

PA-PSK-TKIP+AES

WPA2-PSK-AES

WPANWPAZ-PSK-TKIP+AES

NONE

WPAWPA2-PSK-TKIP+AES

WIFIINFO
Gurrent Hot Spot  unconnected
1P address

Subnetmask

Default Gateway

Refiesh





                                                             Figure 5-46
Step 3    Click “Add SSID” when need to add a wireless network manually pop out an interface in the figure below,  and enter network ID in the dialog box. 
[image: image138.jpg]Add Wireless SSID.

ssD

Ok

Cancel





                                                               Figure 5-47
Step 4   Double click on one can pop out the signal intensity and authentication of the hot spot.
· Please enter the password if necessary.　Please keep the choice of password index number same as the router when entering password.
· Please click “connect” if it is unnecessary to enter the password. 

5.2.12.2 WPS

[image: image139.jpg]Camera
Network

> TePIP

> Connection
> PPPOE

> DDNS

> IP Filter

> SMTP(EMail)
> UPnP.

> SNuP

> Bonjour

> Mulicast

storage

System

Information

WIFI

& Enter PIN
PIN
ssiD

 Button

Status

Connecting

Refiesh





                                                           Figure 5-48
PIN Number and SSID are both acquired from the router, please “refresh” the work information of WIFI to ensure the real-time display of work state after filling in correctly.
5.2.13 802.1x
802.1x (port based network access control protocol) supports manual selection of authentication method to control if device connected to LAN can join the LAN. It well supports authentication, charging, safety and management requirement of network.
[image: image140.png]@Jhua

Camera
Network
> TePnp

> Connection

none

> PPPOE
> DDNS *
> IP Fier Default

> SMTP(Emai)

> UPnP.
> SNMP

> Bonjour

> Mutticast

>

>Qos
Event
Storage
System
Information




Figure 5-49
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Authentication
	PEAP (protected EAP protocol).  

	Username
	It needs the username to login, which is authenticated by the server.

	Password
	Please input password here. 


5.2.14 QoS
The QoS interface is shown as below. See Figure 5-50.
Qos (Quality of Service) is network security mechanism. It is a technology to fix the network delay and jam problem and etc. For the network service, the quality of service includes the transmission bandwidth, delay, the packet loss and etc. We can guarantee the transmission bandwidth, lower the delay, and reduce the loss of the data packet and anti-dither to enhance the quality. 
We can set the DSCP (Differentiated Services Code Point) of the IP to distinguish the data packet so that the router or the hub can provide different services for various data packets. It can select the different queues according to the priority (64 different priority levels) of the packets and select the bandwidth of the each queue. Level 0 is the lowest, and level 63 is the highest. It can also discard at the different ratio when the broad bandwidth is jam. 
[image: image141.png]@hua

Camera
Network

> TePnp

> Connection
> PPPOE

> DDNS

> IP Filter

Default

> SMTP(Email)
> UPnP.

> SNMP

> Bonjour

> Mutticast
>8021x

>

Event
Storage
System
Information




Figure 5-50
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Real-time monitor 
	The value ranges from 0 to 63. The router or the switcher can provide different service for various data packets. 

	Command 
	The value ranges from 0 to 63. The router or the switcher can provide different service for various data packets.

	Enable Wireless QoS
	Check it to enable QoS.


5.2.15 HTTPs 

In the HTTPs setting interface, users can make PC log in normally via HTTPs by creating certificate or uploading signed certificate, which is to guarantee the security of communication data and provide safeguard for user information and device security with reliable and stable technical means. 

Step 1 

Create certificate or upload the signed certificate. 

· Please refer to the following steps if you select “Create Certificate”. 

1. Select “Setup > Network > HTTPs”, and the system will display the interface of HTTPs, which is shown in Figure 5-51. 

[image: image142.jpg]E)/IE]

Camera

Event

Storage

system

Information

Enable HTTPs

Create Certificate

Create.

Request Created
Request Created

Install Signed Certiicate
Cetiicate Path
Certicate Key Path

Certficate Installed
Ceriicate Installed

Attibute.

Downioad.

Browse.

Browse.

Refresh





Figure 5-51
2. Click “Create” and it will pop out the dialog box of “HTTPs”, which is shown in Figure 5-52. 
[image: image143.jpg]HTTPs

County [

IP or Domain name | 10.15.15.211|

Validity Period 365
Province none
Location none
Organization none
Organization Unit | none.
Email

“eg CN

Day'Range :1-5000

Creat

Cancel





Figure 5-52
3. Fill in corresponding “Country”, “Province” and some other information, click “Create” after filling in. It will show the prompt of “Create Successfully”, which means the server certificate has been successfully created. 
Note: 

Make sure the “IP or domain name” is the same as that of the device. 
4. Click “Install” and it will install the certificate on the device end, which is shown in Figure 5-53.

[image: image144.jpg]aJhua

Camera

DoNS
P Fiter
SUTP(EMai)
UPnp

snwp
Bonjour
Micast

802.1x

Event

storage

system

Information

] Enable HTTPs

Create Certificate

Request Created
Request Created

Install Signed Certiicate:
Ceticate Path
Certicate Key Path

Certficate Installed
Cetiicat Installed

Attibute

Issuer. HIP=test, C=CN; ST=test, L=test, O=test,

Download.

Refresh

save





Figure 5-53
5. Click “Download”. It will pop out the dialog box of “Save as”, save the file into the computer. 
[image: image145.jpg]Libraries
Opena library to see your files and arrange them by folder,

» [ Pictures
» H Videos

418 Computer
» & Local Disk (C)
b g DISKLVOL2 (0)
b g DISKLVOL3 (E) #

File name: RootCert.cer

Save astype: [(cer)

(+ Hide Folders




Figure 5-54
6. Double click the downloaded icon of “RootCert.cer”. The system will display the information interface of “Certificate”, which is shown in Figure 5-55. 

[image: image146.jpg]Certificate

General [ etais [ Certfication Path

 J—

This CA Root certificate is not trusted. To enable trust,
install this certificate in the Trusted Root Certification
Authorities store.

Tssued to: test

Tssuedby: test

Validfrom 2016/ 7/ 8to 2020/ 7/ 7

Certifcate...] [ Tssuer Statement

Learn more about ESFEACEIES

o]





Figure 5-55
7. Click “Install Certificate” and it will pop out the interface of “Certificate Import Wizard”, which is shown in Figure 5-56. 
[image: image147.jpg]Certficate Import Wizard L

Welcome to the Certificate Import
Wizard

This wizard helps you copy certificates, certficate trust
lists, and cerfficate revocation sts from your disk to @
certificate store.

A certifcate, which s ssued by a certfication authority, is
 confirmation of your deniity and contains nformaton
used to protect data or o estabish secure network
connections. A certficate store i the system area nhere:
certiicates are kept.

To continue, cick Next.





Figure 5-56
8. Click “Next”. Select “Trusted Root Certification Authorities”, which is shown in Figure 5-57. 
[image: image148.jpg]CertificateStore
Certficate stores are system areas where certficates are kept.

Wiindows can automaticaly select a certficate store, or you can spedify a location for
the certiicate.

Automatically select the certficate store based on the type of certficate
Place al certficates in the following store:

Certficate store:
Trusted Root Certication Authorities

Learn more about certficate stores





Figure 5-57
9. Click “Next”. The system will display the interface of “Completing the Certificate Import Wizard”, which is shown in Figure 5-58. 

[image: image149.jpg]Certificate Import Wiz

Completing the Certificate Import
Wizard

The certficate wil be imported after you clck Finish.

You have spefied the folowing settings:





Figure 5-58
10. Click “Finish”, and it will pop out the dialog box of “Security Warning”, which is shown in Figure 5-59. 
[image: image150.jpg]ity Warming I

You are about to install a certificate from a certfication authority (CA)
claiming to represent:

test
Windows cannot validate that the certficate s actually from "test”. You

should confirm ts origin by contacting "test". The following number
will assist you in this process:

Thumbprint (shal: 6D811FD2 E82313A8 663514ED 2CA36ESB 7D425FAS

Warni
I you installthis root certificate, Windows will automatically trust any
cerificate issued by this CA. Installing a certificate with an unconfirmed
thumbprint is a security risk. f you click "Ves” you acknowledge this
tisk,

Do you want to instal this certificate?

Yo e





Figure 5-59
11. Click “Yes”. It will pop out the dialog box of “Import Successful”, click “Ok” to complete downloading certificate, which is shown in Figure 5-60. 
[image: image151.jpg]Theimport was successful.





Figure 5-60
· Please refer to the following steps if you select “Install Signed Certificate”. 

1. Select “Setup > Network > HTTPs”. The system will display the interface of HTTPs, which is shown in Figure 5-61. 

[image: image152.jpg]E)/IE]

Camera

Network

:
Event

storage

system

Information

Enable HTTPs

Create Certificate

Create.

Request Created
Request Created

Install Signed Certiicate
Ceticate Path
Certicate Key Path

Certficate Installed
Certicate Installed

Attibute

Download.

Browse.

Browse.

Refresh





Figure 5-61
2. Select signed certificate and certificate key path respectively via “Browse”, click “Upload”. 

3. Install root certificate, please refer to step 6~11 for more details. 

4. Check “Enable HTTPs”, click “Save”. The system will display the dialog box of “Need to reboot the device”, and then the config is valid. 

[image: image153.jpg]Reboot

AN e configuration takes effect the device is restarting now, please don'tleave this page or close the browser.





Figure 5-62
5. Use HTTPs

Use HTTPs to log in. 

Enter https://xx.xx.xx.xx in the browser, and it will pop out normal login interface; the browser will prompt certificate error if the certificate is not installed, which is shown in Figure 5-63. 

[image: image154.jpg]- | hitps//101515211/

Fle Edt View Favorites Tools Help
i Favorites | 5 B EWs - ) FITESAE -
{8 Certificate Error: Navigation Blocked T

@ TR R iR W HHG WebTOE's ety eoifiate:

The security certificate presented by this website was not issued by a trusted certificate authority.

Security certificate problems may indicate an attempt to fool you or intercept any data you send to the
server.

We recommend that you close this webpage and do not continue to this website.
© Click here to dlose this webpage.

@ Continue to this website (not recommended).

© More information





Figure 5-63
Note: 

“xx.xx.xx.xx” is corresponding to your IP or domain name. 

5.3 Event 

5.3.1 Video detection
5.3.1.1 Motion Detection
Step 1 

Select “Setup > Event > Video Detection > Motion Detection’ and the system will display the interface of “Motion Detection” which is shown in Figure 5-64. 
[image: image155.jpg]E)/IE]

Camera IMotion Detection] Video Tampering | Scene Changing

Enable

Working Period

Anti-Dither Second (0~100)

Area

Record
Record Delay Second (10-300)
Relay-out

Alarm Delay Second (10-300)
Send Email

storage
Snapshot

system

Information Default Refresh





Figure 5-64
Step 2 

Select “Enable” to enable motion detection function. 

Step 3 

Set motion detection area. 

1. Click “Setup” and the system will pop out the interface of “Setting Area”, which is shown in Figure 5-65
[image: image156.png]cccccc





Figure 5-65
2. Set area name, it is to set valid area of motion detection according to the actual situation, and it can set the value of sensitivity and threshold respectively. 
The bigger the sensitivity is, the easier it is to generate motion detection, the smaller the threshold is, the easier it is to generate motion detection; the entire video image is the valid area of motion detection by default. 

Note: 

· Different colors represent different areas; each area can set different detection area. 

· The red line in the Oscillogram means triggering motion detection; green line means not triggering motion detection. 
3. Click “Save” to complete area setting. 
Step 4
It is to set other parameters of motion detection, please refer to the following sheet for more details. 
	Parameter
	Function 

	Working Period 
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 
· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Anti-dither
	System only memorizes one event during the anti-dither period. The value ranges from 0s to 100s.

	Area 
	1. Click “Setup” and the system will pop out the interface of “Set Area”. 

2. Set the area name, it is to set the valid area of motion detection according to the actual situation, and it can set its sensitivity and threshold respectively. The higher the sensitivity is, the easier it is to trigger motion detect, the smaller the area threshold is, the easier it is to generate motion detect, the whole video image is the valid area of the motion detect by default. 

Note: 

Different colors represent different areas; each area can set different detection areas. 

The red line in the Oscillogram means triggering motion detect, the green line means not triggering motion detect. 

3. Click “Save” to complete area setup. 

	Record
	Check it and so when alarm occurs, system will auto record. You shall set record period in Storage>Schedule and select auto record in record control interface. 

	Record Delay 
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. The value ranges from 10s to 300s. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Audio linkage
	Check it and the system will play alarm audio file when alarm occurs. Users can set the recorded or uploaded audio file in “camera setting ＞ audio ＞ alarm audio”.

	Flash linkage
	Check it and the flash will be on when alarm occurs, and the flash will

be off until the end of alarm after corresponding delay.    

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

· The event type includes: preset, tour and pattern. 

	Snapshot 
	You need to check the box here so that system can backup motion detection snapshot file. You shall set snapshot period in Storage>Schedule.


Step 5 

Click “Save” to complete the setting of motion detection.

5.3.1.2 Video Tampering 

Step 1 

Select “Setup > Event > Video Detection > Video Tampering” and the system will display the interface of “Video Tampering” which is shown in Figure 5-66.

[image: image157.jpg])/ IE]

Camera
Network

Event

ideo Detection

Audio Detection

SmartPlan

Storage

system

Information

Motion Detection |video Tampering] Scene Changing

Enable Video Tampering

Working Period

Record
Record Delay
Relay-out
Alarm Delay
Send Email

Snapshot

Enable Defocus Detection

Setup

Second (10~300)

Second (10~300)

Default

Refresh





Figure 5-66
Step 2 

Set the parameter of video tampering; please refer to the following sheet for more details. 

	Parameter
	Function 

	Enable video tampering 
	You need to check the box to enable video mask detection alarm.

	Enable defocus detection 
	You need to check the box to enable defocus detection alarm.

	Working Period 
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Record
	Check it and so when alarm occurs, system will auto record. You shall set record period in Storage>Schedule and select auto record in record control interface. 

	Record Delay 
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. The value ranges from 10s to 300s. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Audio linkage
	Check it and the system will play alarm audio file when alarm occurs. Users can set the recorded or uploaded audio file in “camera setting ＞ audio ＞ alarm audio”.

	Flash linkage
	Check it and the flash will be on when alarm occurs, and the flash will

be off until the end of alarm after corresponding delay.    

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

· The event type includes: preset, tour and pattern. 

	Snapshot 
	You need to check the box here so that system can backup motion detection snapshot file. You shall set snapshot period in Storage>Schedule.


Step 3 
Click “Save” to complete the setup of video tampering. 
5.3.1.3 Scene Changing 

Step 1 
Select “Setup > Event > Video Detection > Scene Changing” and the system will display the interface of “Scene Changing” which is shown in Figure 5-67. 

[image: image158.jpg]@lhua ST T ST T ST

Camera Motion Detection | Video Tampering [ Scene Changing ®

Network
Event Enable

ction
Working Period

Audio Detecti
SmartPlan Record
s Record Delay Second (10~300)
Relaj-out
Alarm Delay Second (10~300)
Send Email

Snapshot

storage Default Refresh

system
Information





Figure 5-67
Step 2 

Check “Enable” to enable the function of scene changing. 

Step 3 

It is to set the parameters of scene changing, please refer to the following sheet for more details. 

	Parameter
	Function 

	Working Period 
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Record
	Check it and so when alarm occurs, system will auto record. You shall set record period in Storage>Schedule and select auto record in record control interface. 

	Record Delay 
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. The value ranges from 10s to 300s. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

· The event type includes: preset, tour and pattern. 

	Snapshot 
	You need to check the box here so that system can backup motion detection snapshot file. You shall set snapshot period in Storage>Schedule.


Step 4 
Click “Save” to complete the setup of scene changing. 

5.3.2 Audio Detection 
Step 1 

Select “Setup > Event > Audio Detection” and the system will display the interface of “Audio Detection” which is shown in Figure 5-68. 

[image: image159.jpg]E)/E]

Camera
Network

storage
system
Information

Enable Input Abnormal

Enable Intensity Change
=
=

‘Sensitiity

Threshold

Working Period

Anti-Dither

9] Record
Record Delay

] Relay-out
Alarm Delay
SendEmail

] Snapshot

‘Second (0~100)

‘Second (10~300)

‘Second (10~300)

Default

Refresh





Figure 5-68
Step 2 

It is to set the parameters of audio detect, please refer to the following sheet for more details. 

	Parameter
	Function 

	Enable input abnormal
	Select “Enable Input Abnormal” and it will trigger alarm when it detects audio input abnormity. 

	Enable intensity change
	Select “Enable Intensity Change” and it will trigger alarm when it detects that audio intensity change exceeds the threshold. 

	Sensitivity
	It can be judged as audio abnormity when the input volume change exceeds continuous environment volume; users need to adjust according to the actual environment test. 

	Threshold
	It is to set the filtered environment volume intensity, if the environmental noise is too big, then the value needs to be set higher, users can adjust according to the actual environment test. 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Anti-dither
	System only memorizes one event during the anti-dither period. The value ranges from 0s to 100s.

	Record
	Check it and so when alarm occurs, system will auto record. You shall set record period in Storage>Schedule and select auto record in record control interface. 

	Record Delay 
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. The value ranges from 10s to 300s. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

· The event type includes: preset, tour and pattern. 

	Snapshot 
	You need to check the box here so that system can backup motion detection snapshot file. You shall set snapshot period in Storage>Schedule.


Step 3 
Click “Save” to complete the setup of audio detection.

5.3.3 Smart Plan
Smart plan is the master switch for intelligent analysis such as “Face Detection”, “Heat Map”, “IVS”, “People Counting” and so on, the corresponding intelligent functions can be valid after smart plan is enabled. 

Step 1

Select “Setup > Event > Smart Plan”. The system will display the interface of “Smart Plan”, which is shown in Figure 5-69. 
[image: image160.jpg])/ IE]

Camera Smart Plan
Network

Event
> Video Detecti
> Audio Detectic

Smart Plan

> Vs

> Face Detection

> People Counting

Refresh

> HeatMap
> Alam
> Abnormality

storage

system

Information




Figure 5-69
Step 2 

Click “Save” to complete the config of smart plan. 
5.3.4 Intelligence Behavior Analytics 
The basic requirements of scene selection.
· The total proportion of the target shall not exceed 10% of the image. 
· The target size in the image can’t be less than 10 pixel ×10 pixel, the size of abandoned target can’t be less than 15 pixel × 15 pixel (CIF image); The height and width of the target can’t exceed 1/3 of the image; it is recommended that the target height is about 10% of the image height.

· The difference of brightness value between target and background can’t be less than 10 gray levels. 
· Make sure the target appears at least over 2 seconds continuously in the field; the movement distance has to be bigger than the width of the target itself and makes sure it is no less than 15 pixels (CIF image). 

· Try to lower the complexity of the monitoring scene analysis if it is possible; it is not advised to use IVS functions in the environment where the targets are dense and light change is very frequent. 

· Try to keep away from the areas such as glass, ground reflected light, water surface, branch, shadow, mosquito disturbance and etc. try to keep away from the backlight scene to avoid direct light. 
5.3.4.1 IVS
5.3.4.1.1 Tripwire
It will trigger alarm when the target crosses the warning line according to the movement direction which has been set. 

It needs some time from when the target appears to when the target is confirmed, so it has to leave some space on both sides of the warning line when setting the warning line, please do not set the warning line near the obstructions. 
Application scene: it can be applied to the scene with sparse target and there is barely any blocks between the targets, such as the unmanned perimeter defense. 

Step 1 

Select “Setup > Event > IVS > IVS” and the system will display the interface of “IVS”. 

Step 2 

Click “[image: image161.png]


” to set rule name and select the rule type as “Tripwire” which is shown in Figure 5-70. 

[image: image162.jpg]alhua [ e ] payack [ sews | mam [ _iogow ]
Camera @

Hetons Rule Type

Event Tripwire
> Eventhanage
> Video Detection
> Audio Detection
> SmartPlan
Vs - Parameter Setup
> Face Delection o - R Working Period

> People Counting

> HeatMap Direction

> Alarm

> Abnormality Object Track

Storage S

system RecordDelay |10 Second (10-300)

Information Relaj-out

TargetFiter ®) MaxSize 8191 * 8191 1 SecondUin00)

Send Email
MinSze 0 % 0

I Snapshot





Figure 5-70
Step 3 

Click “Draw Rule” to draw tripwire in the monitoring scene, click the right mouse button to complete tripwire setting. 
Step 4 
Click “Draw Target” to draw the size model of filtered target in the video image. 
Step 5 

It is to set the parameters of tripwire, please refer to the following sheet for more details. 

	Parameter
	Function 

	Object tracking 
	Select “Object tracking” to enable the function, refer to “2.4 Video Window Adjustment” for more details. 

Note: 

The function is only supported by some models. 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Direction 
	It is to set the direction of tripwire, you can select A->B、B->A、A<->B.

	Object Track 
	Select “Object Track” and it will enable the function of object track; when the moving object triggers rule and generates alarm, it will select the display mode of object track such as 1P+3 or 1P+5 in the preview interface, the scene of tracking window will follow the moving object till the object leaves the view range of the camera. Please refer to “2.4.8 Fisheye/Trigger Track” for more details.

	Record
	Check it and so when alarm occurs, system will auto record. 

	Record Delay 
	System can delay the record for specified time after alarm ended. 

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Snapshot 
	Select “Snapshot” and the system will auto trigger alarm and snapshot when alarm occurs. 


Step 5 

Click “Save” to complete the setup of tripwire. 

5.3.4.2 Intrusion 
Intrusion includes cross and appears. 
· Cross means that it will trigger alarm when the target enters or exits the area. 

· Appears means that it will trigger alarm when the target appears in the area. 

· As for the report interval of the IVS function in the area, the system will trigger an alarm if it detects same event happened during the interval; the alarm counter will become zero if there is no same event happened in the interval. 

It needs to leave some space for target movement for the area periphery if it is to detect enter & exit event, which is similar to the warning line. 

Application scene; it can be applied to the scene where the target is sparse and there is barely any obstruction between the targets, such as the unmanned perimeter defense. 

Step 1 

Select “Setup > Event > IVS > IVS” and the system will display the interface of “IVS”. 
Step 2 

Click “[image: image163.png]


” to set rule name, select the rule type as “Intrusion” which is shown in Figure 5-71. 

[image: image164.jpg]@lhua [ uve ] payack [ sews | mam | _iogow ]
Camera ®@
Heor

Evel =
Y1 Rulet Intrusion v i@
> EventManage

—
> Ao Detcion
> smatpian

> Face Detection 3 Working Period

Parameter Setup

> People Counting

> Heathiap : 1 Action Appears @ Cross
> Nam t

Direction EnterdExt v
> Abnormaity §
Storage = X‘ Object Track
system : h N Record
Information RecordDelay |10 Second (10-300)
TargetFiter ®) MaxSize 8191 * 8191 Relay-out
10 Second (10-300)
MinSize 0 %0 LY ¢
Send Emall
I Snapsnat





Figure 5-71
Step 3 
Click “Draw Rule” to draw an area in the monitoring image. 
Step 4 
Click “Draw Target” to draw the size model of filtered target in the video image. 
Step 5 

It is to set the parameters of intrusion, please refer the following sheet for more details. 

	Parameter
	Function 

	Object tracking 
	Select “Object tracking” to enable the function, refer to “2.4 Video Window Adjustment” for more details. 

Note: 

The function is only supported by some models. 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Action 
	It is to set the action of intrusion, you can select appears and cross. 

	Direction 
	It is to set the direction of intrusion, you can select enter, exit and enter&exit.

	Object Track
	Select “Object Track” and it will enable the function of object track; when the moving object triggers rule and generates alarm, it will select the display mode of object track such as 1P+3 or 1P+5 in the preview interface, the scene of tracking window will follow the moving object till the object leaves the view range of the camera. Please refer to “2.4.8 Fisheye/Trigger Track” for more details. 

	Record
	Check it and so when alarm occurs, system will auto record. 

	Record Delay 
	System can delay the record for specified time after alarm ended. 

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Snapshot 
	Select “Snapshot” and the system will auto trigger alarm and snapshot when alarm occurs. 


Step 5 

Click “Save” to complete the setup of intrusion. 

5.3.4.3 Abandoned Object 

Abandoned object means the system will trigger alarm if the abandoned object in the monitoring scene exceeds the time which has been set by the users. 
Abandoned object and missing object can be confusing in a situation where both foreground and background are complicated. 
It also triggers alarm as abandoned object if there is pedestrian or vehicle staying still for too long. In order to filter this kind of alarm, generally the abandoned object is smaller than people and vehicle; therefore it can filter the people and vehicle via setting size. Besides, it can avoid the false alarm caused by short stay of people via extending alarm time properly. 
Application scene: It can be applied to the scene where the target is sparse and there is no obvious and frequent light change. False alarm will be increased in the scene with high target density and frequent blocking; false alarm will be increased in the scene where there are more people staying. As for the detection area, try to make it simple, the function is not suitable for the area which is too complicated. 
Step 1 

Select “Setup > Event > IVS > IVS” and the system will display the interface of “IVS”. 

Step 2 

Click “[image: image165.png]


” to set rule name, select the rule type as “Abandoned Object” which is shown in Figure 5-72. 

[image: image166.jpg]E)/IE]

Camera
Network
> Video Detection
> Audio Detection
> SmartPlan

s
> Face Detection
> People Counting
> HeatMap
> Aam
> Abormaity
storage
system
Information

Draw Rule

Target Filter

© MaxSize

Hin Size

Clear

Draw Target

Clear

Parameter Setup.

Working Period

Keep Time

[] Record
Record Delay

[] Relay-out
Alarm Delay

[] Send Email

9] Snapshot

Rule Type
Abandoned Ob[~]

Second (6-3600)

Second (10~300)

Second (10~300)

Default Refiesh





Figure 5-72
Step 3 

Click “Draw Rule” to draw an area in the monitoring area. 
Step 4 
Click “Draw Target” to draw the size model of filtered target in the video image. 
Step 5
It is to set the parameter of abandoned object, please refer to the following sheet for more details. 

	Parameter
	Function 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Keep time 
	It is to set the shortest time from when the object is abandoned to when it triggers alarm.

	Record
	Check it and so when alarm occurs, system will auto record. 

	Record Delay 
	System can delay the record for specified time after alarm ended. 

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Snapshot 
	Select “Snapshot” and the system will auto trigger alarm and snapshot when alarm occurs. 


Step 5 
Click “Save” to complete the setup of abandoned object. 

5.3.4.4 Missing Object 
Missing object means the system will trigger alarm if it exceeds certain amount of time after the target is taken away from the original scene. 
The system can make statistics of the still areas in the foreground area and distinguish if it is missing object or abandoned object according to the similarity between foreground and background, it will trigger alarm when it exceeds the time which is set by users. 
Because the system distinguishes abandoned object from missing object according to the similarity of foreground and background, it may cause error between abandoned object and missing object in a situation where both foreground and background are complicated. 

Application scene: it can be applied to the scene where the target is sparse and there is no obvious and frequent light change. False alarm will be increased in the scene where the target density is high and there is frequent obstruction; the false alarm will be increased in the scene where there are more people staying. As for the detection area, try to make it simple, the function is not suitable for the area which is too complicated. 
Step 1 
Select “Setup > Event > IVS > IVS” and the system will display the interface of “IVS”. 

Step 2 

Click “[image: image167.png]


” to set rule name and select rule type as “Missing Object”, which is shown in Figure 5-73. 

[image: image168.jpg])/ IE]

Camera

Network
Type

t Missing Object[~| i

> Video Detection
> Audio Detection
> SmartPlan

> Face Detection « P Parameter Setup

> People Counting Working Period

> HeatMap

o Keep Time Second (6~3600)

> Abnormality

Storage [ Record

system = Record Delay Second (10-300)

Information [] Relay-out

Draw Rule Clear I Second (10~300)

TagetFiter @ MaxSie DrawTager| | £ SendEmai

9] Snapshot

Hin Size Clear

Default Refiesh





Figure 5-73
Step 3 

Click “Draw Rule” to draw an area in the monitoring scene. 
Step 4 
Click “Draw Target” to draw the size model of filtered target in the video image. 
Step 5 

It is to set the parameters of missing object, please refer to the following sheet for more details. 
	Parameter
	Function 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Keep time 
	It is to set the shortest time from when the object is missing to when it triggers alarm.

	Record
	Check it and so when alarm occurs, system will auto record. 

	Record Delay 
	System can delay the record for specified time after alarm ended. 

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Snapshot 
	Select “Snapshot” and the system will auto trigger alarm and snapshot when alarm occurs. 


Step 5 
Click “Save” to complete the setup of missing object. 

5.3.5 Face Detection 
Step 1 
Select “Setup > Event > Face Detection” and the system will display the interface of “Face Detection”, which is shown in Figure 5-74.

[image: image169.jpg]@lhua ST T ST T ST

Camera Face Detection ®

Network 1 9| Enable
Event ¢ ¥
> Video Detection i Working Period setup

> Audio Detection

Enable Face Enhancement

> SmartPlan S
S Record

L o E Record Delay Second (10-300)
> People Counting \ o

> HeatMap

Alarm Delay Second (10~300)
> Alarm

Send Email
> Abnormality —

[9] Snapshot
Storage

system

Default Refresh

Information

TargetFilter @ Max Size Draw Target

Hin Size Clear





Figure 5-74
Step 2 

Select “Enable” to enable face detection function. 
Step 3 
Click “Draw Target” to draw the size model of target filter in the video image. 
Step 4 

It is to set the parameters of face detection, please refer to the following sheet for more details. 

	Parameter
	Function 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Enable Face Enhancement
	Select “Enable Face Enhancement” and it will guarantee that the face is clear by priority when the stream is set very low.

	Record
	Check it and so when alarm occurs, system will auto record. 

	Record Delay 
	System can delay the record for specified time after alarm ended. 

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Snapshot 
	Select “Snapshot” and the system will auto trigger alarm and snapshot when alarm occurs. 


Step 5 

Click “Save” to complete the setup of face detection. 

5.3.6 People Counting 
5.3.6.1 People Counting 
Step 1 
Select “Setup > Event > People Counting > People Counting” and the system will display the interface of “Entrance/Exit”, which is shown in Figure 5-75. 

[image: image170.jpg]EV/IE]

TecHNoOGY

Camera
Network
Event

> Video Detection

> Audio Detection

> SmartPlan

> s

> Face Detection

> HeatMap
> Alarm
> Abnormality

Storage

System

Information

Draw Rule

Target Fiter

© MaxSize

Hin Size

(950-2560)

Clear

Draw Target

Clear

] Enable

9] Enable 0SD

Working Period

Lens Direction

Rule Name
Direction

Flowrate Alam
Enter Number
Leave Number

Stranded Number

Record
Record Delay
Relay-out
Alarm Delay
Send Email

[9] Snapshot

Zeno

Setup

Inclined

NumberStat
A>B

Second (10~300)

Second (10~300)

Default

Refresh





Figure 5-75
Step 2 
Select “Enable” to enable statistics function of entrance and exit. 

Step 3 

Click “Draw Rule” to draw an area in the monitoring image. 
Step 4 
Click “Draw Target” to draw the size model of filtered target in the video image. 
Step 5 

It is to set the parameters of entrance and exit, please refer to the following sheet for more details. 

	Parameter
	Function 

	Enable OSD 
	Select “Enable OSD” and it will display the people number statistics of entrance and exit in the monitoring image. 

	Working Period
	Note: 

It is to set the alarm period, it can enable alarm event only during the period range which has been set. 

1. Click “Setup” and the system will pop out the interface of “Working Period”. 

2. It is to set working period according to the following methods: 

· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period.   

	Installation Mode
	It is to select wall mount if there is an angle between device lens and horizontal surface, select ceiling if the device lens face downward vertically. 

	Rule Name 
	It is to set the name of rule. 

	Direction 
	It is to select the direction of entrance and exit statistics; A->B or B->A, the arrow direction always means the entrance direction. 

	Flowrate Alarm 
	It is to set the enter number, leave number, stranded number; it will trigger alarm when it exceeds the limited number. 

	Record
	Check it and so when alarm occurs, system will auto record. 

	Record Delay 
	System can delay the record for specified time after alarm ended. 

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. 

	Send Email 
	If you enabled this function, System can send out email to alert you when 
alarm occurs and ends. User can set email address in Network>SMTP.

	Snapshot 
	Select “Snapshot” and the system will auto trigger alarm and snapshot when alarm occurs. 


Step 6 
Click “Save” to complete the setup of people counting. 

5.3.6.2 Report 
It is to generate a form of report according to the number statistics. 
Step 1 

Select “Setup > Event > People Counting > Report “and the system will display the interface of “Report’, which is shown in Figure 5-76. 

[image: image171.jpg]E)/IE]

Camera People Counting
Network

ReportType Day [v]

Event

> Video Dete StartTime | 2016-05-23 00 EndTime |2016-05-23 16 Daily Report Max Range Is 24 Hours
> Audio Dete

S ey Flow Direciion (7] Enters [7] Exits [¥] DisplayNumber  ReporiType @ BarChart () Line Chart

> Vs

Search

> Face Detection

People Counting
> Heathap

> Alam

> Apnormaiity
storage
system
Information





Figure 5-76
Step 2 

It is to set search condition, please refer to the following sheet for more details. 
	Parameter 
	Note 

	Report Type
	Can select day report, month report and year report; also can select bar chart or line chart to display the report.

	Time 
	It can set the begin /end time within the range of statistics time.

	Flow Direction
	It can select enter, leave and display number; the data of checked item can be displayed in the report.

	Search 
	Search flow statistics data according to the setting to form report.

	Export 
	Export the saved report.


Step 3 

Click “Search” to complete report statistics, click “Export” to export the report. 

5.3.7 Heat Map 
5.3.7.1 Heat Map 

It is the density statistics of moving object that can generate report, the color range is from blue to red, blue means the min heat value and red means the max heat value. 

Mirroring, view angle change and heat map original data will be cleared. 

Step 1 

Select “Setup > Event > Heat Map > Heat Map” and the system will display the interface of “Heat Map”, which is shown in Figure 5-77. 

[image: image172.jpg])/ IE]

Camera
Network
Event
storage
system
Information

] Enable

Working Period

Default

Refresh





Figure 5-77
Step 2 

Select “Enable” to enable heat map function. 

Step 3 

It is to set the working period. 

1. Click “Setup” and the system will display the interface of “Working Period”, which is shown in Figure 5-78. 

[image: image173.png]Working Period

Sunday—
- T
- EETTTTT—
an ¥ sunday I~ Monday I™ Tuesday I~ Wednesday I Thursday I Friday [ Saturday
IV Period 1 00 00 00 | -| 23 59 59
I~ Period 2 00 00 00 | -| 23 59 59
I~ Period 3 00 00 00 | -| 23 59 59
I~ Period 4: 00 00 00 | -| 23 59 59
I~ Period 5: 00 00 00 | -| 23 59 59
I~ Period 6: 00 00 00 | -| 23 59 59

[ swe ][ coea ]





Figure 5-78
2. It is to set working period according to the following methods: 
· You can input time numerical value or press the left mouse button to drag on the setup interface to set. 

· There are six periods to be set every day, select the check box in front of the period, and then the period is valid. 

· Select week number (it selects Sunday by default, if it selects the whole week, then it means the setup can be applied to the whole week, you can also select the check box in front of the week number to implement independent setting for some days).

3. Click “Save” to complete the setup of working period. 
Step 4 

Click “Save” to complete the setup of heat map. 
5.3.7.2 Report 
It is to generate the form of report according to the heat map data. 

Step 1 

Select “Setup > Event > Heat Map > Report” and the system will display the interface of “Report”, which is shown in Figure 5-79. 

[image: image174.jpg]E)/IE]

TEckNoLOGY

Camera Heat Map

Network
StartTime | 2016-05-23 EndTime |2016-05-23 *Report Max Range Is 1 Week

Search

storage

System
Information





Figure 5-79
Step 2 

It is to set begin time and end time. 

Step 3 

Click “Search” to complete report statistics, click “Export” to export the statistics report. 

5.3.8 Alarm 
Please note some series products do not support this function.
5.3.2.1 PIR alarm
The alarm activation interface PIR alarm is shown as in figure 5-80.
[image: image175.jpg]@lhua

Camera
Network

> Video Detect
Audio Detect
IVS Analy:
Face Detect
People Counting

Heat Map

Abnormaiity
Storage
System

Information

PIR Alarm
I” Enable

Working Period

Anti-Dither

¥ Flash
Flash Delay
W Record
Record Delay
¥ Relay-out
Alarm Delay

I~ send Email

I~ Audio Linkage

I Snapshot

Relay Activation Flash Set

Second (0~100)

Second (10~300)

Second (10-300)

Second (10~300)

Audio Detect

Default Refiesh





Figure 5-80
Please refer to the following sheet for detailed information. 
	Parameter
	Function 

	Enable 
	After enabled, relay activation will work. Check it and enable the PIR alarm.

	Working period 
	· This function becomes activated in the specified periods. 

· There are six periods in one day. Please draw a circle to enable corresponding period.

· Select date. If you do not select, current setup applies to today only. You can select all week column to apply to the whole week.

· Click OK button, system goes back to motion detection interface; please click save button to exit.
Note: User can set via dragging mouse while not releasing left mouse. 

	Anti-dither
	System only memorizes one event during the anti-dither period. The value ranges from 0s to 100s.

	Flash
	After enabling, the system will automatically turn on the flash when alarm occurs.

	Flash Delay
	· The flash will be off with a period of time delay when the alarm linkage is over.
· Time is calculated by unit of second and ranges from 10s~300s.

	Record 
	System auto activates motion detection channel to record once alarm occurs (working with motion detection function). 

	Record Delay
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  

	Alarm delay
	System can delay the alarm output for specified time after alarm ended. The value ranges from 10s to 300s. 

	Send Email 
	After this function is enabled, system can send out email to alert you when alarm occurs and ends.

	Audio Linkage
	Check it and the system will play alarm audio file when alarm occurs. Users can set recorded audio file in “Camera Setting ＞Audio ＞Alarm Audio”.

	Snapshot 
	After you enabled snapshot, the system will automatically snapshot if alarm occurs. 


5.3.8.1 Alarm Linkage
[image: image176.jpg]Camera
Network

> Video Detect
> Audio Detect
IVS Anal
ot
People Counting

Heat Map

>
Storage

System

Information

PIR Alarm

I~ Enable

Relayin

Working Period

Anti-Dither

I” Flash
Flash Delay
W Record
Record Delay
¥ Relay-out
Alam Delay
I~ send Email
I~ Audio Linkage

T~ snapshot

Relay Activation

Al

Setup

Second (0~100)

Second (10~300)

Second (10~300)

Second (10~300)

Flash Set Audio Detect

SensorType [NO

Default

Reffesh





Figure 5-81
	Parameter
	Function

	Enable
	Check it and enable the alarm linkage.

	Relay-in
	The default alarm 1，some products can choose alarm 2.

	Sensor Type
	Two types of sensor which are normally open (NO) and normally closed (NC), switch from normally open to normally closed, open the alarm. Switch from normally closed to normally open, shut the alarm.

	PTZ
	Check it and set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

The event type includes: preset, tour and pattern and so on.


5.3.8.2 Flash Set
[image: image177.jpg]PIR Alarm

> Video Detect e

> Audio Detect F period 1
> IVS Analy

> Face Detect

> People Counting
> HeatMap

>

> Abnormality
Storage
System
Information

Relay Activation

Weak (&

0

Flash Set Audio Detect

trong

 sunday W Monday W Tuesday W Wednesday W Thursday W Friday W Saturday

00

00

00

23

59 . 59

Default

Refiesh





Figure 5-82
	Parameter
	Function

	ON/OFF
	ON means flash is on; OFF means flash is off.

	Weak-strong bar
	Via dragging this bar to adjust flash brightness.

	All
	Check and it means the day flash works.

	Period
	Period that flash is on.


5.3.8.3 Audio detection
Note: Audio detection won’t work when “Talk” is enabled on the preview interface.

[image: image178.jpg]PIR Alarm Relay Activation Audio Detect
Camera

Network I” Enable

Threshold =]
Video Detect

Working Period

Audio Detect

IVS Analyse ™ Flash

Face Detect Flash Delay Second (10~300)
ople Counting ¥ Record

Heat Map Record Delay Second (10~300)

¥ Relay-out

> Abr lity Alarm Delay Second (10~300)

Storage I” 'Send Email
System

I~ Audio Linkage

Information I Snapshot

Default Refiesh





Figure 5-83
	parameter
	Function 

	Enable
	Check it and enable audio detection.

	Threshold 
	It will trigger alarm and set up a series of linkage when volume exceeds the set threshold.


5.3.9 Abnormity
Abnormity includes No SD Card, Capacity Warning, SD Card Error, Disconnection, IP Conflict and Unauthorized Access. 

Note:

Only device with SD card function has these three statuses: No SD card, capacity warning, and SD card error. Device without SD card function does not have the above three statuses. See Figure 5-84 to
Figure 5-86.

[image: image179.jpg]E)/IE]

TEckNOLGY

Camera SD Card Network Illegal Access

Network
Event Event Type No SD Card [+]

Video Detection Enable
Audio Detection 9] Relay-out
SmartPlan Alarm Delay Second (10~300)

Vs Send Email

Default Refresh

>
>
>
>
>
>
>
>

Storage
system

Information




Figure 5-84
[image: image180.jpg]@lhua TN ST ST T ST

Camera sD card Tllegal Access ©)

Network
Event Event Type SD Card Error [~]

Vic Enable
Audio Detection 9] Relay-out
SmartPlan Alarm Delay Second (10~300)

Send Email

Default Refresh

Heat Map

>
»
>
>
>
»
>
>

Aam

Abnormalty

storage
system

Information




Figure 5-85
[image: image181.jpg])/ IE]

Camera SD Card Illegal Access

Event Type Capacity Warning [+]

Enable
Capacity Limit 9%(0~99)

SmartPlan 9] Relay-out

s Alarm Delay Second (10~300)

Face Detectio Send Email

People Counting

Default Refresh

Heat Map

Aam

Abnormaliy

Storage
system

Information




Figure 5-86
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Enable
	Check to alarm when SD card is abnormal. 

	Relay-out
	Check to enable relay-out alarm. 

	Relay out delay 
	The alarm output can delay for the specified time after alarm stops. The value ranges from 10s to 300s.

Note:

No SD card, capacity warning, SD card error, relay-out delay all start from alarm occurs. 

	Send email 
	After you enabled this function, the system can send out email to alarm the specified user. 

This function is invalid when network is offline or IP conflict occurs.

	SD Card Capacity Limit
	User can set SD card capacity that is left free. When SD card space left is smaller than this limit, alarm occurs. 


When device is offline or IP conflicts, the abnormal alarm is similar with case of SD card error. 
[image: image182.jpg]E)/IE]

TEcHNOLDGY

Camera SD Card, Network Illegal Access

Network
Event Event Type Disconnection =]

] Enable
Audio Detection Record

SmartPlan Record Delay Second (10~300)
s Relay-out

Face Detection Alarm Delay Second (10~300)

Default Reffesh

storage
system

Information




Figure 5-87
When login password keep been wrong for a few times, unauthorized access alarm occurs. This operation is similar to SD card error. Allow login error times as when it exceeds this limit, user account will be locked. 
[image: image183.jpg]alhua T T ST T G

Camera SD Card lllegal Access. ®@
Network

roent ] Enable

Video Detection Login Error time (310)
Audio Detection 9] Relay-out

SmartPlan Alarm Delay Second (10-300)
s Send Email

Face Detection

Default Refresh

People Counting

Heat Map

>
>
>
>
>
>
>
>

Aam

storage
system

Information




Figure 5-88
5.4 Storage Management 
5.4.1 Schedule 
Before schedule setup, user must set record mode is auto or manual. 

Note:

If record mode in record control is off, then device will not snapshot according to schedule. 

5.4.1.1 Record Schedule

Record schedule steps:
Step 1. Click on Record Schedule, see Figure 5-89.

[image: image184.png]@hua

Camera Record Schedule | Snapshot Schedule | Holiday Schedule

Network V] General M [¥] Motion 7] Alarm

Event
0 12 14 16 18 20 2 24

Storage
Sunday
Schedue
Monday
> Destination
> Record Control U=y

SE=D Wednesday

Information Thursday

Friday.
Saturday

Holiday





Figure 5-89
Step 2. From Monday to Sunday select record time, click on setup on the right, see Figure 5-90.
· Set period according to actual need. There are six periods available each day. 

· By checking or unchecking, you can add or delete three types of record schedule: General, Motion, and Alarm. 

Note:

Period setup can be done by dragging in record schedule interface while not releasing left mouse. 
[image: image185.png]setup

™A

Period 1.
Period 2:
Period 3:
Period 4
Period 5

Period

¥ sunday I Monday I~ Tuesday [T Wednesday I Thursday I Friday

00
00
00
00
00
00

00
00
00
00
00
00

00
00
00
00
00
00

23 - 69 : 59 | [ General ¥ Hotion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion
save cancel

W Alarm
™ Alarm
™ Alarm
™ Alarm
™ Alarm
™ Alarm

I saturday I~ Holiday





Figure 5-90
Step 3.  Click on OK, return to record schedule interface. See Figure 5-91. 

· Green color stands for the general record/snapshot.

· Yellow color stands for the motion detect record/snapshot.
· Red color stands for the alarm record/snapshot. 

[image: image186.png]@hua

Camera Record Schedule | Snapshot Schedule | Holiday Schedule

Network V] General M [¥] Motion 7] Alarm

Event
0 12 14 16 18 20 2 24

Storage
Sunday
Schedue
Monday
> Destination
> Record Control U=y

SE=D Wednesday

Information Thursday

Friday.
Saturday

Holiday





Figure 5-91
Step 4. In record schedule interface, click on OK. System prompts it is successfully saved. 
5.4.1.2 Snapshot Schedule
Snapshot setup as:
Step 1. Click on Snapshot Schedule tab, see Figure 5-92.
[image: image187.png]@hua

Camera Record Schedule | Snapshot Schedule | Holiday Schedule

Network V] General M [¥] Motion 7] Alarm

Event
0 12 14 16 18 20 2 24

Storage
Sunday
Schedue
Monday
> Destination
> Record Control U=y

SE=D Wednesday

Information Thursday

Friday.
Saturday

Holiday





Figure 5-92
Step 2. From Monday to Sunday select snapshot time, click on setup on the right. See Figure 5-93.
· Set snapshot period according to actual need. There are six periods available each day. 
· By checking or unchecking, user can add or delete three types of snapshot schedule: General, Motion and Alarm. 

Note: Period setup can be done by dragging in snapshot schedule interface while not releasing left mouse. 

[image: image188.png]setup

™A

Period 1.
Period 2:
Period 3:
Period 4
Period 5

Period

¥ sunday I Monday I~ Tuesday [T Wednesday I Thursday I Friday

00
00
00
00
00
00

00
00
00
00
00
00

00
00
00
00
00
00

23 - 69 : 59 | [ General ¥ Hotion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion
save cancel

W Alarm
™ Alarm
™ Alarm
™ Alarm
™ Alarm
™ Alarm

I saturday I~ Holiday





Figure 5-93
Step 3. Click on OK, return to snapshot schedule interface. See Figure 5-94. 

· Green color stands for the general record/snapshot.

· Yellow color stands for the motion detect record/snapshot.

· Red color stands for the alarm record/snapshot. 

[image: image189.png]@hua

Camera Record Schedule | Snapshot Schedule | Holiday Schedule

Network V] General M [¥] Motion 7] Alarm

Event
0 12 14 16 18 20 2 24

Storage
Sunday
Schedue
Monday
> Destination
> Record Control U=y

SE=D Wednesday

Information Thursday

Friday.
Saturday

Holiday





Figure 5-94
Step 5. In snapshot interface, click on OK. System prompts it is successfully saved. 

5.4.1.3 Holiday Schedule

Holiday schedule can set specific date as holiday. 
Step 1. Click on Holiday Schedule tab, see Figure 5-95.

[image: image190.png]@hua

Camera Record Schedule | Snapshot Schedule | Holiday Schedule

Network
Event

Record [ Snapshot

Storage
Schedue Sun Mon Tue Wed Thu
> Destination 2[{3][s
> Record Control o [0 [11
System 1] [17] [18

Information b=





Figure 5-95
Step 2. Select date to set as holiday. The selected date will be highlighted in yellow. 
Step 3. Check Record/Snapshot, click on Save. System prompts it is successfully saved. 

Step 4. Check Record Schedule/Snapshot Schedule interface, click on setup next to Holiday, and refer to setup of Monday to Sunday. 
Step 5. Complete setup of holiday, then it records/snapshots according to date in holiday schedule. 

5.4.2 Destination 
5.4.2.1 Path
The destination interface is shown as in Figure 5-96.
Path can config record and snapshot storage path. There are three options: Local, FTP and NAS. You can only select one mode. System can save according to the event types. It is corresponding to the three modes (general/motion/alarm) in the Schedule interface. Please check the box to enable the save functions. 
Note: Only device supports SD card has local.

[image: image191.png]@hua

Camera
Network
Event

storage

system
Information

Path

Record
Event Type Scheduled
Local
FTP
NAS

Motion Detect

snapshot
Event Type
Local
FTP
Nas

Scheduled

Motion Detect





Figure 5-96
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Event Type 
	It includes: scheduled, motion detect and alarm.  

	Local 
	It saved in the SD card. 

	FTP 
	It saved in the FTP server. 

	NAS
	It saved in NAS disk. 


5.4.2.2 Local

The local interface is shown as in Figure 5-97. 
Here you can view local Micro SD card or NAS disk information. You can also operate the read-only, write-only, hot swap and format operation. 
[image: image192.png]E)/IE]

Camera
Network
Event
storage

> Schedule

> Record Control

System
Information

Attribute
Read & Wite

Used Capacity/Total Capacity
. | 6297.7MIT578.1M

Read Only

Read & Wiite

Hot Swap

Refiesh





Figure 5-97
5.4.2.3 FTP
The FTP interface is shown as in Figure 5-98.
You need to check the box to enable the FTP function. When network disconnect occurred or there is malfunction. Emergency storage can save the record/snapshot picture to the local SD card. Click the test button to check if the FTP server can be connected for test. 
[image: image193.jpg]@hua

Camera
Network
Event

> Schedule
>

> Record Control

system

Information

Path

Enable.
Server Address

Port 21

User Name ‘anonymity
Password

Remote Directory.

Emergency (Local)

(0~65535)





Figure 5-98
5.4.2.4 NAS

You need to check the box to enable the NAS function. Select NAS storage, fill in NAS server address and corresponding store path, then you can store video file or pictorial information in the NAS server. See Figure 5-99.
[image: image194.png]@hua

Camera Path

Network Enable

Event
Server Address

storage

Remote Directory
> Schedule

> Record Control

system
Information





Figure 5-99
	Parameter
	Function

	Server Address
	Set IP address of NAS server.

	Remote Directory
	Set storage directory, videos and pictures can be stored in to corresponding server directory.


5.4.3 Record control 
The record control interface is shown as in Figure 5-100.
[image: image195.png]@hua

Camera Record Control

e Pack Duration Minute (1~120)

Event
Pre-event Record Second (0-5)
storage

Disk Full Overwiite

> Schedule
Record Mode © Auto Manual
> Destination

Record Stream Main Stream
ecord Control

System Default

Information





Figure 5-100
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Pack Duration
	Here you can select file size within 1min~120min. Default setup is 8 minutes.

	Pre-record 
	Please input pre-record value here. 

For example, system can record the four seconds video in the buffer. The record begins from the fifth second. 

Note:

Configure pre-record time, when alarm or motion detection occurs, if there is no record, system will record the preceding n seconds record.  

	Disk Full
	There are two options: stop recording or overwrite the previous files when HDD is full.

· Stop: Current working HDD is overwriting or current HDD is full, it will stop record. 

· Overwrite: Current working HDD is full; it will overwrite the previous file.

	Record mode 
	There are three modes: Auto/manual/close.  

	Record stream 
	There are two options: main stream and sub stream. 




5.5 System 
5.5.1 General 
The General interface is shown as in Figure 5-101.
[image: image196.png])/ IE]

TecHnoweY

Camera
Network
Event
Storage
System

.

> Account
> PTZSetings
> Defautt
> ImportExport
> Auto Maintain
> Upgrade
Information

General

Language

Video Standard

DatesTime

YZC4DZ02

English

PAL





Figure 5-101
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Device Name
	It is to set device name. 
Note: Different devices have different names. 

	Video Standard
	This is to display video standard such as PAL.



	Language 
	You can select the language from the dropdown list. 

	TV Out 
	You can open or close the function, it can only be supported by the devices with TV Out. 
· Intelligent function will be disabled when TV Out is opened. 

· It will auto close TV Out when intelligent function is enabled. 

· Some devices support SDI and HDCVI. 


The Date & Time interface is shown as in Figure 5-102.
[image: image197.png]@hua

Camera General Date&Time
Network

Date Format Year-Month-Day

Event

Time Format 24-Hour-based System

Time Zone GMT+08:00

Current Time. 2000 - 05

> Account DST Enable.

> PTZ Setings DST Type. © Date

> Default Start Time Jan

> ImporyExport End Time Jan

> Auto Maintain Synchronize with NTP
> Upgrade NTP Server clockisc org

Information Port 123

Update Period 10 Minute (0~30)

[





Figure 5-102
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Date format
	Here you can select date format from the dropdown list.

	Time Format
	There are two options: 24-H and 12-H.

	Time zone 
	The time zone of the device. 

	Current Time 
	It is to set system time. It becomes valid after you set. 

	Sync PC
	You can click this button to save the system time as your PC current time. 

	DST Enable 
	Here you can set both begin and end time of daylight saving time. You can set according to the date format or the week format.

	Synchronize with NTP  
	You can check the box to enable NTP function.  

	NTP server 
	You can set the time server address. 

	Port 
	It is to set the time server port. 

	Update period 
	It is to set the sync periods between the device and the time server. 


5.5.2 Account 

· For user name and user group, the max length is 31 characters, which can be made up of digit, letter, underline, hyphen, dot and @. 
· Password can be 0~32 characters in number and letter only. User can modify other user’s password. 

· User and group can be manually added to 18 and 8 respectively. 

· User management adopts group/user modes. The user name and the group name shall be unique. One user shall be included in only one group. 
· Currently logged in user cannot change his/her own right. 

5.5.2.1 Username 
In this interface you can enable anonymity login, add/remove user and modify user name. See Figure 5-103.

[image: image198.png]@hua

Camera
Network
Event

Storage

System
> General
> PTZ Settings
> Defaut
> Import/Export
> Auto Maintzin
> Upgrade

Information

Account

7] Anonymous Login
User Name
No.

1

Remark

admin’s account

Authority List

Live Playback

Aam Log Search
VideolAudio Schedule/Destination
DefaultimportExport  Conditions.

Record Control
Clear Log
Network

Vs

Backup PTZ
Upgrade Auto Maintain
Abnomality Video Detect

Account
General
PTZ Settings

Add User.





Figure 5-103
Enable anonymity login: Enable anonymity login, and input IP. No username or password is required, you can log in by anonymity (with limited rights). You can click logout to end your session. 
Add user: It is to add a name to group and set the user rights. See Figure 5-86.
Hidden user “default” is for system interior use only and cannot be deleted. When there is no login user, hidden user “default” automatically login. You can set some rights such as monitor for this user so that you can view some channel view without login.

Here you can input the user name and password and then select one group for current user. 
Please note the user rights shall not exceed the group right setup. 

For convenient setup, please make sure the general user has the lower rights setup than the admin. 

[image: image199.png]Add User

UserName
Password

Confirm Password

Group. admin

Remark
Authority List Al

W Live

¥ Playback

¥ Record control

¥ Backup

save

cancel





Figure 5-104
Modify user 
It is to modify the user property, belonging group, password and rights. See Figure 5-103.
Modify password 
It is to modify the user password. You need to input the old password and then input the new password twice to confirm the new setup. Please click the OK button to save. 

Please note, the password ranges from 0-digit to 32-digit. It shall include the number and letter only. For the user who has the account rights, he can modify the password of other users. 

[image: image200.png]Modify User

UserName
T~ Modity Password
Group.
Remark

Authority List

admin -

admin 7

admin 's account

Fa

W Live ﬂ
W Playsack

7 Record contal

¥ Backup

save cancel





Figure 5-105
5.5.2.2 Group

The group management interface can add/remove group, modify group password and etc. 

The interface is shown as in Figure 5-106.

[image: image201.png])/ IE]

Camera
Network
Event

storage

System

> Auto Maintain

Information

Account

Anonymous Login

User Name

administrator group

user group

Authority List
Live

Alam.

Video/Audio
DefaultimportExport

Playback
Log Search
Schedule/Destination
Conditions

Record Control
ClearLog
Network

Vs

Backup P1Z
Upgrade Auto Maintain
Abnormality Video Detect

Account
General
PTZ Settings

Add Group.





Figure 5-106
Add group: It is to add group and set its corresponding rights. See Figure 5-107.
Please input the group name and then check the box to select the corresponding rights. It includes: preview, playback, record control, PTZ control and etc. 
[image: image202.png]Add Group

Group.
Remark
Authority List ™A
™ Live
I Playback

™ Record control
™ Backup

save cancel





Figure 5-107
Modify group 

Click the modify group button, you can see an interface is shown as in Figure 5-108.

Here you can modify group information such as remarks and rights. 

[image: image203.png]Modify Group

Group.
Remark

Authority List

admin

administrator group
W Al

FLve

R Playback

W/ Record control

¥ Backup

save cancel





Figure 5-108
5.5.3 PTZ 
Please note only some series products support this function.
The PTZ settings interface is shown as in Figure 5-109.

[image: image204.png]@hua

Camera PTZ Settings

CEZEn Protocol PELCOD
Event

Adoress 1

Storage
Baudrate

system

Data Bit

> General

Stop Bit

> Account

Parity

sati

> Default

> ImporyExport
> Auto Maintain
> Upgrade

Information





Figure 5-109
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Protocol
	Select the corresponding dome protocol.

	Address
	Set corresponding dome address. Default value is 1. 
Please note your setup here shall comply with your dome address; otherwise you cannot control the speed dome.

	Baud Rate
	Select the dome baud rate. Default setup is 9600.

	Data Bit 
	Default setup is 8. 

	Stop bit 
	Default setup is 1. 

	Parity 
	Default setup is none. 


5.5.4 Default 

The default setup interface is shown as in Figure 5-110.

Please note system cannot restore some information such as network IP address, account and so on.

[image: image205.png]system
> General
> Account
> PTZ Settings

> Import/Export
> Auto Maintain
> Upgrade

Information





Figure 5-110
5.5.5 Import/Export 

The interface is shown as in Figure 5-109.
[image: image206.png]@hua [ e U m | e | e | tam ] i ]
Camera Import/Export @
Network Backup Path
Event

Import

Storage
system

> General
> Account

> PTZ Settings.
> Default

>

> Auto Maintain
> Upgrade
Information




Figure 5‑111
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Import
	It is to import the local setup files to the system.

	Export 
	It is to export the corresponding system setup to your local PC. 


5.5.6 Remote control
Note:
It is supported by some devices.

[image: image207.jpg]Camera
Network
Event

Storage
Arm/Disarm

> General

> Account

> Defautt

> ImportExport
Remale

> Auto Maintain

> Ungrade

Information

Remote -
[Am =] [1os





Figure 5‑112
	Parameter
	Function

	Learn
	Bind remote control or wireless alarm with equipment. 

	Clear
	Unbind remote control with equipment.

	Arm/Disarm
	Start arm after the time you choose, disarm comes into effect immediately.   Note:
With disarm status, all the following alarms won’t trigger: motion detect alarm, masking alarm, local alarm, PIR alarm, wireless alarm.


5.5.6 Auto Maintenance 
The auto maintenance interface is shown as in Figure 5-113.
Here you can select auto reboot and auto delete old files interval from the dropdown list.

If you want to use the auto delete old files function, you need to set the file period. 
[image: image208.png]Network

Event
Storage
system

> General

> Account

> PTZ Settings
> Defautt
> ImportExport
>
> Upgrade
Information

[aniaseoon |

Lretesn s





Figure 5-113
	Parameter
	Function

	Auto Reboot
	Check it and set auto reboot time.

	Auto Delete Old Files
	Check it and set period within 1~31 days.


5.5.7 Upgrade
The upgrade interface is shown as in Figure 5-114.
Please select the upgrade file (file extension is “.bin”) and then click the update button to begin firmware update. 

Important

Improper upgrade program may result in device malfunction!

[image: image209.png]Network

Event
Storage
system

> General

> Account

> PTZ Settings
> Defautt
> ImportExport
> Auto Maintain
>
Information

Upgrade

Upgrade





Figure 5-114
5.6 Information 
5.6.1 Version 
The version interface is shown as in Figure 5-115.

Here you can view system hardware features, software version, release date and etc. Please note the following information is for reference only. 

[image: image210.png]@hua

Camera Version

Network Device Type 1PC-HF3300
Event

Software Version  2.420.0000.0R, build : 2014-03-14
Storage

WES Version 321175653

system

ONVIF Version 241
Information

| e | " Fesm

> Log
Copyright 2011,All Rights Reserved.
> Oniine User





Figure 5-115
5.6.2 Log 

Here you can view system log. See Figure 5-116.

[image: image211.png]@hua

Camera
Network
Event
Storage
system
Information

> Version

Log

> Oniine User

Log

StartTme | 2014 - 03 - 30 |[ 11 - 17

[

End Time.

2014 - 03 - 31

11

Detailed Information

M <11 > MGoTo ]

Clear





Figure 5-116
Please refer to the following sheet for log parameter information. 

	Parameter 
	Function 

	Start time
	Set the start time of the requested log. (The earliest time is 2000/1/1)


	End time
	Set the end time of the requested log. (The latest time is 2037/12/31)

	Type
	Log type.

	Search 


	You can select log type from the drop down list and then click search button to view the list.

You can click the stop button to terminate current search operation. 

	Log information 
	You can select one item to view the detailed information.

	Clear
	You can click this button to delete all displayed log files.  Please note system does not support clear by type.

	Backup 
	You can click this button to backup log files to current PC.


5.6.3 Online User
The online user interface is shown as in Figure 5-117. 

Here you can view current online user, group name, IP address and login time. 
[image: image212.png]@hua

Camera Online User

Network IP Address User Login Time

Event admin 10.18.116.47 2000-05-22 183346

Storage

system
Information
> Version

> Log





Figure 5-117
6 Alarm

Please note some series product does not support this function.
Click alarm function, you can see an interface is shown as in Figure 6-1.

Here you can set device alarm type and alarm sound setup. 

[image: image213.jpg]@hua

Alarm Type i jarm Type

] Motion Detection [ DiskFull

] Disk Error [C] Video Tampering
[] External Alarm ] Wegal Access
[] Audio Detection

] Scene Changing

Operation

] Prompt

Alarm Tone

] Play Alarm Tone





Figure 6-1 

Please refer to the following sheet for detailed information. 

	Type 
	Parameter 
	Function 

	Alarm type 
	Motion detection 
	System alarms when motion detection alarm occurs,

	
	Disk full
	System alarms when disk is full.

	
	Disk Error 
	System records alarm info when disk error happens.

	
	Video Tampering
	System alarms when video is tampering.

	
	External alarm 
	Alarm input device sends out alarm. 

	
	Illegal Access
	System alarms when there is illegal access. 

	
	Audio Detection  
	System alarms when there is audio detect. 

	
	IVS 
	System alarms when IVS event is triggered. 

	
	Scene Changing 
	System will record alarm info when triggering scene changing. 

	Operation 


	Prompt 
	When alarm is triggered, there will be s [image: image214.png]


 inmain menu of alarm interface and system automatically records alarm info. The icon disappears when user click on alarm menu bar. 
Note:
If alarm interface is displayed, when alarm is triggered, there will be no image prompt, but alarm record will be in list on the right. 

	Alarm Tone 
	Play Alarm Tone 
	Click it and select the path of audio file to be played in the tone path, when the subscribed alarm event triggers, it will play the selected audio file to prompt that there is alarm event which has been triggered. 

	
	Tone Path 
	You can customize the storage path of the alarm tone. 


7 Log out

Click log out button, system goes back to log in interface. See Figure 7- 1.
[image: image215.png]@hua

Usemanme: | admin

Password:





Figure 7- 1 
Note:
· This manual is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice.

· All trademarks and registered trademarks mentioned are the properties of their respective 
owners.

· If there is any uncertainty or controversy, please refer to the final explanation of us.

· Please visit our website for more information.


Dahua Vision Technology Co., Ltd

Address：No.1199 Bin’an Road, Binjiang District, Hangzhou, PRC.

Postcode: 310053

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuatech.com

Website: www.dahuasecurity.com

2





3





1





4





1        2      3     4      5       6       7    8      9 





6





3





5





1





4





2








PAGE  
139

_1380970326

_1380970338

_1380970349

_1380970309

