
Speed Dome Web3.0 Operation Manual
Version 1.0.0
Table of Contents
11
Network Config


11.1
Network Connection


21.2
Log in WEB Interface


52
Common Operation


52.1
Live


52.1.1
Encode Setup


62.1.2
Video Window Adjustment


92.1.3
System Menu


92.1.4
Video Window Function Option


102.1.5
PTZ Config


132.2
Playback


132.2.1
Video Playback


142.2.2
Play Function


192.2.3
Picture Playback


223
Setup


223.1
Camera


223.1.1
Conditions


353.1.2
Video


453.1.3
Audio


463.2
Network


463.2.1
TCP/IP


503.2.2
Connection


523.2.3
PPPoE


543.2.4
DDNS


573.2.5
IP filter


583.2.6
SMTP （e-mail)


603.2.7
UPnP


613.2.8
SNMP


643.2.9
Bonjour


653.2.10
Multicast


653.2.11
802.1x


663.2.12
QoS


683.3
PTZ Setting


683.3.1
Protocol


693.3.2
Function


813.4
Event


813.4.1
Video detect


863.4.2
Audio Detect


883.4.3
IVS Plan


893.4.4
Intelligent Behavior


963.4.5
Face Detection


973.4.6
Heat Map


983.4.7
Alarm


993.4.8
Abnormity


1033.5
Storage Management


1033.5.1
Schedule


1063.5.2
Destination


1103.5.3
Record control


1113.6
System


1113.6.1
General


1133.6.2
Account


1173.6.3
Peripheral


1193.6.4
Default


1193.6.5
Import/Export


1203.6.6
Auto Maintenance


1213.6.7
Upgrade


1223.7
Information


1223.7.1
Version


1223.7.2
Log


1243.7.3
Online User


1254
Alarm


1275
Log out




Important
The following functions are for reference only. Some series products may not support all the functions listed below. 

1 Network Config
1.1 Network Connection

Speed dome and PC connection mainly has two ways. See Figure 1‑1 and Figure 1‑2.

[image: image1.png]‘ Network Cable

=

Network Speed Dome Computer




Figure 1‑1
[image: image2.png]Router or Switch

Network Speed Dome

Computer




Figure 1‑2
Note: 

The models presented in the figure are for reference only. 

The IP address of all the intelligent speed domes is 192.168.1.108 by default when they are delivered out of factory; it needs to plan available IP segment reasonably according to practical network environment in order to make intelligent speed domes get access to network smoothly. Users can modify IP address via quick config tool in the disk, please refer to  <<Quick Configuration Tool User manual>> for more details. 
1.2 Log in WEB Interface
Step 1 

Open IE browser and input IP address of network speed dome in the address bar, then press “Enter”. It will display the interface shown in Figure 1-3 below after it is successfully connected. 
[image: image3.png]



Figure 1‑3
Step 2 

Please input your user name and password, and then click “Login”. Default factory username is admin and password is admin. 

Step 3 

The system will pop out “Modify Password” prompt box shown in Figure 1-4, please modify administrator password in time and save it properly. 
[image: image4.png]First login, suggest updating password.

Newpassword ]

Confirm Password

Don't show me again





Figure 1‑4
Step 4 
WEB interface will be displayed in Figure 1-5 below after successful login. 
[image: image5.jpg]IP PTZ Dome mmm
Protoco! IR DEO® O

PTZ Control

000
0600
000
speea(t-o) KA
=0
© =0
-0

PTZSetings  Menu

© Please cick here 1 downipad and instal the plug-in

& i

= Fo E@




Figure 1‑5
Step 5 

Install or load plug-in according to the system prompt. 

The plug-in installation interface will close automatically after installation is completed, the WEB client will refresh automatically and you can see the image, as it is shown in Figure 1-6. 
[image: image6.png]e T T T YT T
[iainsisamall suosvean i | swsvean: LRe R N 2BO0Io
3876Kbps. 192ﬂ WB PTZControl  Joystick.
©00
0o®o0
000

speea(t-o) KA
=0
Q =0
e -0
PTZ Settings. Menu

IP PTZ Dome

& oo R EE @ 4o B O




Figure 1-6
Click “Logout” to log out, it needs to log in again for entrance. 
Note: 

The interface shown above is for reference only; please refer to the exact model for more details.
2 Common Operation
2.1 Live 
Users can implement several operations on the “Live” interface upon the realtime monitoring image, such as live, snapshot, record and etc, you can also implement simple PTZ operation. 
Click “Live” to display “Live” interface which is shown in Figure 2‑1.
[image: image7.jpg]IP PTZ Dome

o 1}- e 210}

PTZControl  Joysiick

000
o000
000
speeact-o) [EIKN
=0
©~0
e-o

P1Z Setings

Frotoco T

=l
SIP PTZ Doma =
< =

oo & & %o Fl@ ©





Figure 2‑1
	SN
	Note

	1 
	Encode setting column

	2 
	Video window adjust column 

	3 
	System menu column 

	4 
	Video window function option column

	5 
	PTZ config column


2.1.1 Encode Setup 
Note: 

Some models don’t support two sub streams
The encode setup interface is shown as in Figure 2‑2.
[image: image8.png]protocol IS





Figure 2‑2
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Main stream 
	Streaming media protocol connection, under main stream config, monitor video or not. Generally for storage and monitor.  

	Sub (Extra) stream 1
	Streaming media protocol connection, under sub stream 1 config, monitor video or not. When network bandwidth is insufficient, it substitutes main stream for monitoring. 

	Sub (Extra) stream 2
	Streaming media protocol connection, under sub stream 2 config, monitor video or not. When network bandwidth is insufficient, it substitutes main stream for monitoring.

	Protocol 
	You can select stream media protocol from the dropdown list. 

There are three options: TCP/UDP/Multicast


2.1.2 Video Window Adjustment
[image: image9.png]i





Figure 2‑3
	Parameter
	Note 

	1. Image Adjustment 
	Click “Image Adjustment” and it will display image adjustment interface on the right of the live interface, then you can adjust image brightness, contrast and etc. 

	2. Original Size
	Click the button and it will display the actual size of video stream. 

	3. Full Screen 
	Click it to go to full-screen mode.  Double click the mouse or click the Esc button to exit the full screen.

	4. Width and Height Ratio
	Click it to adjust image to original ratio or suitable window. 

	5. Fluency Adjustment
	There are three levels of fluency for you to select (real-time, normal, fluent). The default is normal. 

	6. Rules Info 
	Click it to enable preview page to display intelligent rules, the default is enabled. 

	7. PTZ
	Click it to enable live page to display PTZ config item. 

	8. Panorama PTZ
	Click the button to display panorama window in the live interface, it can realize quick position, call preset, tour and other operations in the window.


Image Adjustment 
[image: image10.png]Image Adjustment




Figure 2‑4
	Parameter
	Note 

	
[image: image11.png]



	It is to adjust monitoring image brightness. 

	
[image: image12.png]



	It is to adjust monitoring image contrastness.

	
[image: image13.png]



	It is to adjust monitoring image hue.

	
[image: image14.png]



	It is to adjust monitoring image saturation.

	[image: image15.png]Reset




	Restore brightness, contrastness, saturation and hue to system default setup.


Note: 
The function can only be used to adjust the brightness, contrast, hue and saturation of the monitoring image in WEB client. It needs to go to “Setup > Camera > Conditions” to set the brightness, contrast, hue and saturation of the device. 
Panorama PTZ 

[image: image16.jpg]



Figure 2‑5 
Use left mouse button to draw a box to operate positioning in the window, the live page will display the positioned location and zoom in. Click “Refresh”, and the camera will rotate horizontal 0°～360°, vertical 6°～75°to regain panorama image. Drag the picture ratio bar [image: image17.png]


 to adjust the size of panorama picture. 
[image: image18.jpg]



Figure 2-6
User can use the corresponding preset on the right of the window, please refer to “3.3.2.1 Preset” for more details about preset setting. 
[image: image19.jpg]



Figure 2-7

User can use the corresponding tour on the right of the window, please refer to “3.3.2.2 Tour” for more details about tour setting.
2.1.3 System Menu 
Click each item to enter corresponding interface. 
[image: image20.png]| uve | papack [ sewp [ aam [ vLogout |




Figure 2‑8
2.1.4 Video Window Function Option 

[image: image21.png]



Figure 2‑9
Please refer to the following sheet for detailed information. 

	SN
	Parameter
	Function 

	1
	Regional focus
	Click it and use mouse to select any area in the video image, and it can make the device auto focus in the selected area. 

	2
	Remark 
	Click it and then select pen color, you can write down the mark information on the preview interface. 

	3
	Gesture Control
	Click it and control the PTZ via dragging left mouse button in the preview interface, mouse roller can control zoom rate. 

	4
	Relay out 
	Click the button to trigger alarm. The light becomes red/gray when there is relay out/cancel. 

	5
	Digital Zoom
	· When the video is in the original status, click it you can select any zone to zoom in. In the non-original status, you can drag the zoom-in zone in specified range. Right click mouse to restore previous status. 

· Click it; you can use the middle button of the mouse to zoom in/out the video size.  

	6
	Snapshot
	Click the button to snapshot, save picture to path in Chapter 3.1.2.5

	7
	Triple snap
	Click it, system can snap at 1f/s. All images are saved to path in Chapter 3.1.2.5.

	8
	Record
	Click it, system can record. All images are saved to path in Chapter 3.1.2.5.

	9
	Manual track
	Click the button and then drag left mouse button to select any area in the video window, the speed dome will make smart tracking to the object in the area. 

Note: It needs to check “Event >IVS Plan> Rule Config>Smart Track”, otherwise, it will be invalid. 

	10
	Audio output
	Turn on or off audio when you are monitoring.

	11
	Talk
	Click it to start or end bidirectional talk.

	12
	Help
	Click it to open help file.


2.1.5 PTZ Config 
You can control PTZ via PTZ control or virtual joystick; also you can enable the functions of preset, scan and etc in the PTZ setting area. 
PTZ Control 

Note: 

Users have to set PTZ protocol first before using PTZ control, please refer to “Setup > PTZ Settings > Protocol” for more details. 
[image: image22.png]



Figure 2-10
	Parameter
	Note

	[image: image23.png]


Quick Position
	Use mouse to draw a box in monitoring video, PTZ will rotate, focus and quickly position the scene. 

	PTZ direction
	PTZ supports eight directions: left/right/up/down/upper left/upper right/bottom left/bottom right.

	Speed
	It controls rotation speed. The longer the step length, the higher the speed. Step length control PTZ, zoom, focus and iris. 

	Zoom/focus/iris
	Click [image: image24.png]


 to increase value and click [image: image25.png]


 to decrease value. 
Note: 

Some cameras don’t support iris, please refer to the actual devices for details. 


Virtual Joystick 

The virtual joystick interface is shown as below. See Figure 2‑11.
This function allows you to control the button in then center to simulate the joystick operation. You can use it to control device movement. 

[image: image26.png]PTZControl  Joystick




Figure 2‑11
The setting method of speed, zoom, focus and iris is the same as that of the PTZ control. 
PTZ Setting
Note: 

Please refer to “3.3 PTZ Setting” for more details.
Click [image: image27.png]» start




 to start some certain PTZ function, and at this moment the “Start/Stop” button becomes[image: image28.png]Stop



, click the button to stop the PTZ function. 

[image: image29.png]PTZ Settings

Scan

Preset
Tour
Pattem
Assistant
Pan

Go to





Figure 2‑12
Please refer to the following sheet for PTZ setup information. 

	Parameter 
	Function 

	Scan 
	· Select Scan from the dropdown list, click Start button, you can begin scan operation. Default SN is 1.

	Preset 
	· Input the preset value and then click View button, the camera turns to the corresponding position of the preset. 

	Tour 
	· Select Tour from the dropdown list and then click Start button, you can begin tour. 

	Pattern 
	· You can select Pattern from the dropdown list and then click Start button to begin PTZ movement.

	Assistant 
	· Reserved assistant function. 

	Pan
	Select Pan from the dropdown list and then click “Start” button and it can realize horizontal rotation of the PTZ.

	Go to
	· It is the accurate positioning function. Please input corresponding horizontal angle, vertical angle and zoom speed and then click “Go to” button to go to a specified position.  

· One unit of the horizontal angle or vertical angle stands for 0.1 degree. 


2.2 Playback

You can playback the saved video or picture in the “Playback” interface. 

Note: 

It needs to set record, snapshot period, storage method, record control and other parameters in “3.5 Storage” before implementing playback operation. 

Click “Playback” item, and the system will display “Playback” interface, which is shown in Figure 2-13. 

[image: image30.png]RecordType [ Al [ General W [ Event® @ Aamm [ Manual M




Figure 2-13

2.2.1 Video Playback 
Select file type as “dav” and the system will display the interface which is shown in Figure 2-14. 
[image: image31.jpg][ J©)
T

Dasc 5B

Sun Mon Tue Wen Thu Fri Sat
1]2

ala]s]e]7]e]e
10111213 14

17|18 19|20 | 21
2425 26 27 28 29| 30

00000 O =@ CIKIEEETEFD

00:00 0200 04:00 06:00 08:00 1000 1200 1400 1600 1800 20.00 2200 2400

Stop. RecordType @ Al [ General M (4 Event M [ Alamn Manual





Figure 2‑14
	SN
	Note

	1
	Play function column

	2
	Record type column

	3
	Assistant function column

	4
	Playback file column 

	5
	Playback clip column 

	6
	Progress bar time format column 

	7
	Progress bar 


2.2.2 Play Function
[image: image32.png]00000 O =0

De@®® 6 ® O




Figure 2‑15
	Parameter
	Function

	1 Play
	When you see this button, it means pausing or not playing record. Click this button and switch to normal play status. 

	2 Stop
	Click this button to stop playing video. 

	3 Next frame
	Click this button to go to next frame. 

Note: 

You shall pause playback when you use this function. 

	4 Slow
	Click this button to play slowly. 

	5 Fast
	Click this button to play fast. 

	6 Mute
	When this button displays, it means audio is silent. Click this button to switch back to normal. 

	7 Volume
	Left click mouse to adjust volume. 

	8 Rules info 
	Click the button to display intelligent rules after enabling playback video. 


2.2.2.1 Record Type

Check record file type, the only selected file will be displayed in progress bar and file list. See Figure 2-16.

[image: image33.png]RecordType @Al [General ® [@Motion @ [MAlarm  [dManual B




Figure 2-16

2.2.2.2 Assistant Function

Video playback assistant function is shown in Figure 2-17.

[image: image34.png]



Figure 2-17
	Parameter
	Function

	[image: image35.png]


 Digital Zoom 
	· Click it, you can zoom in any area when then playback video is in original status. In non-original status, you can zoom in specified zone, Right click mouse to restore its original size. 

· Click this button; you can scroll to zoom in/out video. 

	[image: image36.png]


 Snapshot
	Click this button; you can snapshoot video under playback status. Snapshot will be saved to path in chapter 3.1.2.5.


2.2.2.3 Playback File
In calendar, the date with blue shading means the current date having video record or snapshot file. See Figure 2-18.
[image: image37.png]e e
ouscv: RG]
< |

‘Sun Mon Tue Wed Thu Fri Sat

1|2

8 9

3.4 5 6 7

10 11 12 13 14




Figure 2-18
	Parameter
	Function

	File Type
	· Select “dav” and it means record video playback. 

· Select “jpg” and it means picture playback. 

	Data Source
	It is SD card by default. 


Click the date in blue, time axis displays record file progress bar in color. While, green represents normal record, yellow represents motion detect record, red represents alarm record, and blue represents manual record. 
Click certain time location on the progress bar, playback of the record file starts from this time spot.  See Figure 2-19.

[image: image38.png]



Figure 2‑19
Step 1. Click file list[image: image39.png]


, the file of selected date will be displayed in the list. 
Step 2. Double click the file in list, playback this file and display file size, start time and end time. 
See Figure 2-20.
[image: image40.png][0 00 00 25 co 5 [N
Download Format # dav @ mp4.

Start Time

z
E

13,0820

1 "0
2 130422 W Q
3 130458 W ©Q
4 131208 W Q
5 132122 W ©Q
6 132244 W ©Q
7 132412 W Q
8 132432 W Q
9 1332 B QO
10 134518 W Q
1 144014 W Q
DREEAC] ' [»]

Begin Time: 2013-12-18 13:03:20
EndTime: 20131218 13.03:55
FileSize:  16868(KB)




Figure 2‑20
	Parameter
	Function

	[image: image41.png]


Search
	It means searching all the record files between start time and end time of selected date. 

	Record Download Format
	There are two formats:  dav, mp4.

	[image: image42.png]


Download
	· Record type is “dav”, click “Download” button to download file to local. 

· Record type is “mp4”, click “Download” button and download file to path in Chapter 3.1.2.5.

Note:

System does not support download and playback of MP4 file at the same time. 

	[image: image43.png]


Back
	Click “Back” button to go back to calendar interface, then you can select time again to operate.  


2.2.2.4 Playback Clip
Note:

The record file which is being playbacked will pause automatically when using playback clip function, which means that playback clip and playback can’t be implemented at the same time. 

[image: image44.png]



Figure 2-21

Step 1 
Click start time to clip on time axis. This time must be within progress bar range. 

Step 2 
Move mouse above the clip icon[image: image45.png]


, and “Select Start Time” will show up in the lower right corner. 
Step 3 
Click clip icon [image: image46.png]


 and complete the setting of start time for playback clip. 

Step 4 

Click end time of playback clip on time axis, and the time must be within progress bar range. 
Step 5 
Move mouse above clip icon[image: image47.png]


, and “Select End Time” will show up in the lower right corner. 
Step 6 
Click clip icon [image: image48.png]


, and complete the setting of end time of playback clip. 

Step 7 

Click “Save” button to edit file in the path of “Playback Clip” set in the “3.1.2.5 storage path”. 
2.2.2.5 Progress Bar Time Format

[image: image49.png]



Figure 2-22

	Parameter
	Function

	[image: image50.png]


24 hours
	Click it and the progress bar will display in 24-hour mode. 

	[image: image51.png]


2 hours
	Click it and the progress bar will display 2 hours of the video. 

	[image: image52.png]


1 hour
	Click it and the progress bar will display 1 hour of the video. 

	[image: image53.png]30min




30 min
	Click it and the progress bar will display 30 minutes of the video.


2.2.3 Picture Playback
Select file type as “jpg”, and the system will display the interface shown in Figure 2-23. 

[image: image54.jpg]e
e

af«]s]e]7]=]
10[11]12] 13 14 [15]) 16
17[18] 19|20 | 21| 2|2
242526 27 28|20 | 30

(>)

SnapshotType [ Al [ General Event Alarm





Figure 2-23

	SN
	Note

	1
	Play function column

	2
	Playback file column

	3
	Snapshot type column


2.2.3.1 Play Function
The play button is shown as below. See Figure 2-24.
[image: image55.png]


     [image: image56.png]



Figure 2-24
Default icon is[image: image57.png]


, and it means pauseing or not playing picture.
· Click play button to switch to normal play status. Icon becomes[image: image58.png]


. 
· Click it to pause and switch it to pause status. 
2.2.3.2 Playback File

Step 1: Click file list[image: image59.png]


, ans the selected snapshot file will be displayed in the list.
Step 2: Double click file in list to playback this snapshot. 
[image: image60.png]rie e [N
ouscv: TG
< |

‘Sun Mon Tue Wed Thu Fri Sat

1|2]a]s]s]e]7

8 9 10 11 12 13 14



    [image: image61.png])

StatTime.
120340
120341
130342
120343
120344
120345
120842
120643
120644
120445
120845
120647
130448

LRRVENS

z
E

[]
[]
[]
[]
[]
[]
[]
[]
[]
[]
[]
[]
[]
»

IOOOOOOOOOOOOO




Figure 2‑25            Figure 2‑26
Please refer to the following sheet for detailed information. 

	Parameter
	Function

	[image: image62.png]


 Search
	It means all snapshot files within the start time and end time of selected date.

	[image: image63.png]


Download
	Click download button to download snapshot file to local. 

	[image: image64.png]


 Back
	Click back button to return to calendar interface and re-select time to operate. 


2.2.3.3 Snapshot Type

It will only display the selected type of file in the list after checking the snapshot type. It can also select the snapshot type to be displayed via the drop-down box above the file list. 

[image: image65.png]Snapshot Type Al [General @ Motion @ [MAarm




Figure 2‑27
3 Setup
You need to set the camera, video and audio conditions of the network intelligent speed dome in order to guarantee normal monitoring of the device. 

3.1 Camera 
3.1.1 Conditions
Select “Setup > Camera > Conditions” and the system displays “Conditions” interface, which is shown in Figure 3‑1. The interface is mainly to set the camera conditions, adjust image parameters to realize best preview effect. 
[image: image66.png]T s | —sour ] _som | o]
Camera Conditions  |Profile Management}

= Profile| Day
> Video
> Audio

Brightness (]
Network » Exposure ]
PTZ Settings Backlight Saturation =)
Event ; RE Chroma CNT (=] ()
Storage BN Sharpness (=
System = ZeomEec Sharpness CNT (=]—(}
Information i IR Light
Gamma (=]
e Fiip  180°
EISC ON © OFF

Picture Freeze () ON (© OFF





Figure 3‑1
Please refer to the following sheet for detailed information. 
	Parameter
	Note 

	Config file
	It can select normal, day or night mode, it can set and check config and effect of corresponding mode after selecting mode. 



	[image: image67.png]


 Full screen test
	Click the [image: image68.png]


 on the interface to implement full screen test.

	Direction button
	It supports 4 directions, which as up, down, left and right. 

	Zoom,

focus, iris
	Click [image: image69.png]


 and the corresponding value becomes bigger, click [image: image70.png]


 and

the corresponding value becomes smaller. 

	Speed 
	It is used to operate speed, the bigger the step is, the faster the speed becomes. The speed is valid to PTZ direction control, zoom, focus and iris adjustment. 


Note: 

· The function is depended on the product. 
· The camera parameter may be different due to different models, please set parameter according to the actual product. 

· It needs to click “OK” to save the config on the camera condition interface, and the image effect can be valid immediately. 

3.1.1.1 Image 
Step 1 
Select “Setup > Camera > Conditions > Image”. The system displays the interface of “Image” which is shown in Figure 3-2. 
[image: image71.png]T s | —sour ] _som | o]
Camera Conditions  |Profile Management}

= Profile| Day
> Video
> Audio

Brightness (]
Network » Exposure ]
PTZ Settings Backlight Saturation =)
Event ; RE Chroma CNT (=] ()
Storage BN Sharpness (=
System = ZeomEec Sharpness CNT (=]—(}
Information i IR Light
Gamma (=]
e Fiip  180°
EISC ON © OFF

Picture Freeze () ON (© OFF





Figure 3-2
Step 2 
Configure each parameter info according to the actual needs; see the following sheet for more details. 

	Parameter 
	Note 

	Brightness
	It is to set the overall brightness of image, the bigger the value is, the brighter the image becomes. The value ranges from 0 to 100. 

	Contrast 
	It is to set the image contrast, the bigger the value is, the bigger the image bright contrast becomes. The value range is from 0 to 100. 

	Saturation
	It is to set the image color purity, the higher the purity is, the brighter it becomes, the lower the purity is, the darker it becomes. The range is from 0 to 100. 

	Chroma CNT
	It is to set the suppression level upon the image color, the bigger the value is, the more obvious the suppression becomes. The value range is from 0 to 100. 

	Sharpness
	It is to adjust the sharpness level of the image edge. The bigger the value is, more obvious the edge becomes; it is opposite if the value becomes smaller. It is easy for the image to generate noise if the value is set too high. The value range is from 0 to 100. 

	Sharpness CNT
	It is to adjust the sharpness suppression level of the camera, the bigger the value is, the stronger the sharpness suppression becomes. The value range is from 0 to 100. 

	Gamma 
	The threshold is mainly used to change image brightness via nonlinear adjustment mode and improve the dynamic display range of the image. The bigger the value is, the brighter the image becomes. The value range is from 0 to 100. 

	Flip
	The function can be used to change the direction of video surveillance image. It can select normal and flip. It is normal by default. 

	EIS
	It can realize EIS function via image difference value comparison algorithm, which can effectively solve the problem of image jittering during use and make HD image even clearer. It is disabled by default. 

	Picture freeze
	After the image is frozen, it directly displays the preset when calling preset. 


Step 3 
Click “Save” to make config valid. 

3.1.1.2 Exposure 

Step 1 
Select “Setup > Camera > Conditions > Exposure”. 
The system displays “Exposure” interface, which is shown from Figure 3-3 to Figure 3-6. 

[image: image72.png]IP PTZ Dome

Camera

> Video

Conditions  [Profile Management]

> Audio
Network

PTZ Settings
Event
Storage
System
Information

Profile Day M

» Picture Mode | Auto M

» Backiight stow Exposure (5]}

»we GainLimit (5] 0

b By Slow Shutler &) ON () OFF

» ZoomFocus Shutter Limit| 130 M

» IR Light AE Recovery | 15Minute v

» Defog 2DNRC) ON © OFF
Advanced NR @) ON () OFF
Advanced 3D (=] 0

Advanced 2D (5] 0





Figure 3-3

[image: image73.png]IP PTZ Dome T TR S —rr
Camera Conditions  [Profile Management] ®@

- Profile. Day
> Video
> Audio b FELT Mode | Aperture Priority
Network , e E—0
PTZ Settings Backiight Exposure Comp (5] 0
Event s - RE Siow Exposure (1}
Storage Cave Nioht GainLinit (]
system ZoomFocus
Information

IR Light

Slow Shuteer & ON
Shutter Limit
Detog AE Recovery
DR ON
Advanced NR @) ON

Advanced 30 (5]

Advanced 20 (5]





Figure 3-4

[image: image74.png]IP PTZ Dome

Camera

> Video

Conditions  [Profile Management]

> Audio
Network

PTZ Settings
Event
Storage
System
Information

Profile. Day
» Picture Mode | Shutter Priority
» Backiignt Exposure Comp (5] 0
oUB Siow Exposure (=]}
» Day & Night

Gain Limit (5] 0

» ZoomFocus

AE Recovery | 15Minute
DI 2DNR ON @ OFF
pEEEY Advanced NR @/ ON OFF

Advanced 3D (5] 0

Advanced 2D (5] 0





Figure 3-5

[image: image75.png]IP PTZ Dome

Camera

> Video
> Audio
Network
PTZ Settings
Event
Storage
System
Information

JProfie Management}

Profile| Day

> e Mode | Manual

Stuter 130000

» Backignt can G0
»we s 0
» Day & Nignt 5

AE Recovery | 15Minute

» ZoomFocus ST S —

[ Advanced NR @/ ON OFF

» Deto
3 Advanced 3D (5]

oo

Advanced 20 (5]

@





Figure 3-6

Step 2 

Configure each parameter info according to actual needs, see the following sheet. 

	Parameter 
	Note 

	Mode 
	It is to set camera exposure mode. It includes: auto/manual/aperture priority/shutter priority. The default is auto mode. 

· For the auto exposure mode, the image overall brightness will auto adjust according to different scene brightness in the normal exposure range. 

· For manual exposure mode, it can manually adjust gain value and shutter value; it supports long exposure. 
· For aperture priority mode, fixed aperture is the set value, it can auto realize best brightness according to priority drive exposure time to drive gain mode. 

· For shutter priority mode, the image overall brightness can auto adjust according to the adjustment shutter range priority according to different scene brightness in normal exposure range. If the image brightness is still improper and gain has reached upper and lower limit of the range, then it can auto adjust gain value again to make image normal.

	Exposure Comp
	It is to set the value of exposure compensation; value range is from 0 to 100. 

	Slow Exposure
	It is to set the exposure adjustment speed; the value range is from 0 to 100. 

	Gain limit
	It is to set the gain upper limit of exposure, the value range is from 0 to 100. 

	Slow Shutter
	It can capture image via extending auto exposure time in the low illuminance environment, which can effectively reduce image noise, but it may generate smear for moving objects. 

	Shutter limit 
	It is to restrict the min shutter value of the camera.

	Auto exposure recovery
	After manually adjusting “Iris + or Iris –”, it will recover to the exposure mode before adjustment regularly. 

	2D NR
	The threshold is used to suppress noise, the higher the level is, the smaller the noise becomes, and the image appears more blurry then before. The value range is from 0 to 100. 

	Advanced NR
	It can realize noise suppression effect via 3D and 2D video filtering method.

	Advanced 3D
	It is to set 3D grade, the value range is from 0 to 100. 

	Advanced 2D
	It is to set 2D grade, the value range is from 0 to 100. 


Step 3 
Click “OK” to make the config valid. 

3.1.1.3 Backlight 

Backlight mode includes backlight compensation, WDR and HLC. 
· Backlight compensation: backlight compensation can make the dark part of the main shooting object clear in the backlighting environment. 

· WDR: Enable WDR to suppress the over bright area and compensate over dark area, which can make the whole image a quite clear status. 

· HLC: HLC is used to weaken the highlight area, which can be applied in the areas such as toll gate, entrance and exit of parking lot, etc. As for extreme light, it can snapshot human face in dark environment and realize quite good effect of license plate details. 
Step 1 
Select “Setup > Camera > Conditions > Backlight”. The system displays “Backlight” interface, which is shown in Figure 3-7. 

[image: image76.png]1P PTZ Dome T T .
Camera Conditions  [Profile Management] ®@

Profile | Day
> Video
> Audio » Picture Node| OFF
Network » Exposure
PTZ Settings

Event e

» B
storage

5 » Day &Night
System

: » ZoomFocus.
Information

» IR Light

» Defog





Figure 3-7

Step 2 

Select backlight mode. 

Step 3 

Click “OK” to make config valid. 

3.1.1.4 White Balance 
White balance is used to restore white objects, after setting white balance mode; it can make the white object display white status in different environments. 
Step 1 

Select “Setup > Camera > Conditions > WB”. 

The system displays the interface of “WB”, which is shown in Figure 3-8. 

[image: image77.png]IP PTZ Dome

Camera Conditions  [Profile Management] ®@

Profile | Day
> Video

> Audio » Picture ot Auto
Network » Exposure

PTZ Settings

» Backight
Event

storage <

» Day &Night
System

. » ZoomFocus.
Information » IR Light

» Defog





Figure 3-8

Step 2 

Select “WB” mode. 

As for WB mode, it can select auto, manual, 

Step 3 

Click “OK” to make config valid. 
3.1.1.5 Day & Night 
The function can be used to set the conversion between color mode and B/W mode, which can effectively guarantee that it can still monitor clear image even in dark environment for the intelligent speed dome. 

Step 1 

Select “Setup > Camera > Conditions > Day & Night”. 

The system displays the interface of “Day & Night” mode, which is shown in Figure 3-9. 

 [image: image78.png]IP PTZ Dome

Camera Conditions  [Profile Management] ®@

Profile| Day
> Video

> Audio

P Type O Electical © ICR
Network ! s Mode | Auto
PTZ Settings. .

e Sensitity | Middle
Event

» B
storage

System : » ZoomFocus.
Information

» IR Light

» Defog





Figure 3-9

Step 2 

Configure each parameter info according to the actual needs; refer to the following sheet for more details. 

	Parameter 
	Note 

	Type 
	Day/night switch mode can select electrical and ICR, it is ICR by default. 

· ICR: Mechanical day/night switch uses filter for day & night switch. 

· Electrical: It uses image processing mode for day & night switch. 

	Mode 
	It is to set image color and b/w mode, which is not influenced by the selection of config profile. It is auto mode by default. 

· Color：The camera will only output color image.
· Auto：It can select to output color or black & white image according to the environment adaptation. 
· Black & White：The camera will only output black & white image.

	Sensitivity
	It is used to adjust the sensitivity of switch between color and black & white. It can select high, middle and low, it is middle by default. 

Note: 

It can set sensitivity only when day/night mode is auto. 

	Delay 
	It is used to adjust the delay value of switch between color and black & white. The value range is 2s～10s. 

Note: 

It can set delay only when the day/night mode is auto. 


Step 3 
Click “Save” to make config valid. 

3.1.1.6 Zoom & Focus 
Digital zoom means zooming in part of the image, the bigger it zooms in, the more blurry it becomes. 
Step 1 

Select “Setup > Camera > Conditions > Zoom & Focus”. 

The system displays the interface of “Zoom & Focus”, which is shown in Figure 3-10. 

[image: image79.png]T s | —sour ] _som | o]
Camera Conditions  |Profile Management}

# Profile Day
n

2T DRI Digital Zoom () ON (&) OFF
Network » Exposure 2o0m speed

PTZ Settings. i D =R Mode | Semi Auto
Event Ui DD Focus Linit| Auto
Storage o g D By sensitvity| Default
system %

:
Information £

» IR Light

AFTracking ® ON () OFF





Figure 3-10

Step 2 

Configure each parameter info according to the actual needs; please refer to the following sheet. 

	Parameter 
	Note 

	Digital zoom 
	It is used to set if it is to enable digital zoom function, it is off by default. 

	Zoom speed 
	It is to set camera zoom speed, the bigger the value is, the faster the zoom speed becomes.

	Mode
	It is to control the trigger mode of focus, you can select semi-auto, manual and auto. 

· Semi auto: it will actively trigger focus when detecting zoom, ICR switch and etc.

· Auto: It will actively trigger focus when detecting scene change and zoom, ICR switch and etc. 

· Manual: The users can adjust focus location by themselves; the device won’t trigger focus actively. 

	Focus limit 
	It is to set the nearest distance of focus, and focus on the object beyond the distance, the auto option will make it select proper nearest distance automatically according to the different zoom value. 

	Sensitivity 
	It is to set the steady ability or anti-interference capability of focus, the lower the value is, the steadier it becomes, the higher the value is, the stronger the anti-interference capability becomes. 

	AF tracking
	The image becomes relatively clear during zoom if the function is enabled. If the function is disabled, then the zoom speed becomes relatively fast during zoom. 

	Lens initialization
	Click the button and it will implement lens initialization automatically, at this moment, it will realize correction of zoom and focus for the camera. 


Step 3 
Click “Save: to make config valid. 

3.1.1.7 IR Light 
Step 1 
Select “Setup > Camera > Conditions > IR Light”. 

The system displays the interface of “IR Light”, which is shown in Figure 3-11. 
[image: image80.png]IP PTZ Dome

Camera Conditions  [Profile Management] ®@

Profile| Day
> Video

> Audio » Picture

Mode | ZoomPrio
Network

PTZ Settings
Event

» Exposure Correction (=]
» Backight

» B
storage

System
Information

» Day & Night
» ZoomFocus
» RLight

» Defog





Figure 3-11

Step 2 

Configure each parameter info according to the actual needs, please refer to the following sheet for more details. 

	Parameter 
	Note 

	Mode
	It is used to set the mode of IR light, you can select zoom priority, manual and off. It is zoom priority by default. 

· Zoom priority: The intelligent speed dome can auto adjust IR light brightness according to the actual zoom rate. 

· Manual: It can set the brightness value of IR light manually. 

	Light compensation
	It is used to compensate the brightness of IR light; the value range is from 0 to 100. 

	Near light
	It is used to set the brightness of near light; the value range is from 0 to 100. 

	Near light angle
Note: The function is only supported by some models.
	It is used to set the angle value of the near light; the range is from 0 to 100. 



	Far light
	It is used to set the brightness value of far light; the range is from 0 to 100. 

	Far light angle
Note: 

The function is only supported by some models.
	It is used to set the angle value of far light; the range is from 0 to 100. 


Step 3 
Click “Save” to make config valid. 

3.1.1.8 Defog 

The image quality may become weak if the camera is in the environment with fog or haze, the image can realize auto correction in the auto mode; it can also select different intensity manually according to the fog concentration, which is to adjust the image definition. 
Step 1 

Select “Setup > Camera > Conditions > Defog”. 

The system displays the interface of “Defog”, which is shown in Figure 3-12. 

[image: image81.png]T s | —sour ] _som | o]
Camera Conditions  |Profile Management}

Profile| Day
> Video

> Audio » Picture

Mode | OFF
Network

PTZ Settings
Event

» Exposure
» Backight

» B
storage

System
Information

» Day & Night
» ZoomFocus

» IR Light





Figure 3-12

Step 2 

Configure each parameter info according to the actual needs, please refer to the following sheet for more details. 

	Parameter 
	Note

	Mode 
	It is used to set the defog mode of the intelligent speed dome, you can

select auto, manual and off, it is off by default. 


Step 3 
Click “Save” to make config valid. 

3.1.1.9 Profile Management 

It can select three modes of profile management, such as normal, full time and schedule. 
· When it selects “Normal”, the video will be monitored according to the normal config of the camera. 
[image: image82.png]IP PTZ Dome

Camera

Frre A S -
Profile Management @
>

* Nomal ) Full Time

> Video
> Audio
Network
PTZ Settings

Event
Storage
System
Information




Figure 3-13

· When it selects “Full time”, it can select day or night, which is corresponding to the config file of the camera conditions for day or night.
[image: image83.png]IP PTZ Dome IS T TR TR ST
[Profile Management

@

al @ Fu
> Video

Camera Conditions

> Audio
Network
PTZ Settings
Event

Storage
System
Information




Figure 3-14

When it selects “Schedule”, you can select one period as day config, the other period as night config. If the config profile management is displayed according to schedule, you can set 0:00～12:00 as day config, and 12:00～24:00 as night config. 

[image: image84.png]IP PTZ Dome

Camera

> Video

> Audio
Network
PTZ Settings
Event
Storage
System

Information

Conditions _ |Profile Management]

Profile Management () Normal () FullTime @ Schedule

Period seting ]

000 400

© Day @ Night

oo





Figure 3-15

3.1.2 Video 
You need to implement config upon the camera video stream, snapshot, overlay, ROI and path. 
3.1.2.1 Video 
Step 1 

Select “Setup > Camera > Video > Video Stream”. 
The system displays the interface of “Video stream” , which is shown in Figure 3-16. 

[image: image85.png]IP PTZ Dome

> Conditions
>

> Audio
Network

PTZ Settings
Event
Storage
System
Information

Video

Main Stream

Encode Mode
Resolution

Frame rate(FPS)
BitRate Type
Reference Bit Rate
BitRate

| Frame Interval
sve

Watermark Settings.

‘Snapshot

H264H
1080P (1920*1080)
30

CBR
1280-8192K0/S
409

60

1(off)

Watermark Character | DigitalCCTV

(30~150)

Path

Sub Stream
I Enable
Encode Mode
Resolution
Frame rate(FPS)
BitRate Type
Reference Bit Rate
BitRate
| Frame Interval

sve

Sub Stream 1
H264H
DI (704°480)
30
CBR
192-4096KbIS
1024
60
1(off)

(30~150)





Figure 3-16

Note: 

Different bit stream may be corresponding to different default value, please refer to the actual interface for more details.
Step 2 

Configure each parameter info according to the actual needs, please refer to the following sheet for more details. 

	Parameter 
	Function 

	Sub Stream Enable 
	· Please check the box here to enable extra stream function. This function is enabled by default. 

	Code-Stream Type 
	· ACF function, use different frame rates to record, use high frame rate to record for important event and low frame rate for scheduled event. The frame rate of motion detect record and alarm record can be set separately. 

· Main stream includes general, motion and alarm three code-stream types, sub stream can only support one type of stream as general stream. You can select different coding stream for different recording events. 
Note: 

Some WEB interfaces don’t support motion and alarm stream setting. 

	Encode mode 
	There are seven options: H.264, H.264H, H.264B, H.265, MJPEG and MPEG4.
· H.264: Main Profile encode mode. 
· H.264H: High Profile encode mode. 
· H.264B: Baseline Profile encode mode. 
· H.265: Main Profile encode mode. 
· MJPEG: In this encode mode, the video needs to enlarge bit stream to guarantee the video definition. You can use the max bit stream value in the recommended bit to get the better video output effect.

	Resolution 
	There are multiple resolution types. You can select from the dropdown list. 

For each resolution, the recommended bit stream value is different.

	Frame Rate (FPS)
	PAL: 1～25f/s，NTSC: 1～50f/s.. 

The frame rate may vary due to different resolutions. 

	Bit Rate Type 
	There are two options: VBR and CBR. 

· Please note, you can set video quality in VBR mode. 

· In MJPEG encode mode, only CBR is available. 

	Reference  Bit Rate  
	Recommend a reasonable bit rate value range according to the resolution and frame rate you have set. 

	Bit Rate 
	· In VBR, the bit rate here is the max value.  
In CBR, the value is fixed. 

·  Refer to “Reference Bit Rate”, bit rate value can provide best reference range. 

	I Frame interval 
	Here you can set the P frame amount between two I frames, the range varies according to the frame rate, the max is 150, it is recommended to set twice as big as the frame rate. 

	SVC
	Frame rate can realize layered coding, it is a scalable encoding mode in time domain, and it is 1 by default, which is not layered. It can set 2, 3, 4 layer coding setting.

	Watermark Settings
	By calibrating watermark, to see if video is modified. Select Watermark function. Default watermark is Digital CCTV. 

Watermark character can only be number, letter, _, - within 128 characters. 


Step 3 
Click “Save: to make config valid. 

3.1.2.2 Snapshot 
The snapshot interface is shown as in Figure 3‑17.
[image: image86.png]IP PTZ Dome

Camera Video Snapshot Overlay
> Conditions.

Snapshot Type General

Image Size 1080P (1920
> Audio

Quality 5
Network

. Interval 15
PTZ Settings

Event

Storage
System
Information





Figure 3‑17
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Snapshot type 
	There are two modes: general (schedule) and Event (activation).

	Image size 
	It is the same with the resolution of snapshot (main stream or sub stream). 

	Quality 
	It is to set the image quality. There are six levels. 

	Interval 
	It is to set snapshot frequency. The value ranges from 1s to 7s or customized.  


3.1.2.3 Video Overlay 
The video overlay interface is shown as in Figure 3‑18 to Figure 3-25.
[image: image87.png]IP PTZ Dome

Camera Snapshot Path
> Conditions

> Audio » Channel Titie SN

Network » Time Tite

PTZ Settings » OSD info

Bt » Font
Storage
System

Information

» Picture Overlay
» Abnormal

» GPS Position





Figure 3‑18
[image: image88.png]IP PTZ Dome T T TR ST

Camera Snapshot Path
> Conditions

Network » Time Titie IP PTZ Dome.

» Privacy Mask © Enable ) Disable

PTZ Settings » OSD info
Event : » Font

Storage » Picture Overlay
System
Information

» Abnormal

» GPS Position





Figure 3‑19
[image: image89.png]IP PTZ Dome

Camera Snapshot Path

> Conditions
ideo » Privacy Mask © Enable ) Disable

> Audio » Channel Titie @I Week Display

v

PTZ Settings » 0SD info
Event » Font
Storage

System

Information

» Picture Overlay
» Abnormal

» GPS Position





Figure 3‑20
[image: image90.png]IP PTZ Dome

Camera Snapshot Path
> Conditions
ideo » Privacy Mask Preset © Enable ) Disable

> Audio » Channel Titie

Temperature Disable
Network ime Title

PTZ Settings
Event

Goordinates Disable
» Font
storage

System

» Picture Overlay Disable

» Abnormal
o o Disable
Information » GPS Position

Disable

Disable

Text Overlay Disable

Input Text

TextAlign
Right





Figure 3‑21
[image: image91.png][BFTz fome [ [ romsa | sor | om [ o]

Path

Camera Snapshot

> Condifons
o » PrivacyMask  Color
> Audio ) Chamnel e  Size 10
Network
PTZ Settings » 0SDnfo
Event

storage

» Picture Overlay
System <

» Time Titie Row Height 1.0

» Abnormal
Information <

» GPS Position





Figure 3-22
[image: image92.png][P F1z Bome [ e [ opwms [ s | aam ] ogon ]

Camera Snapshot Path
> Conditions.

deo » Privacy Mask Enable (® Disable
> Audio » Channel Tt Picture Preview:
Network » Time Title
PTZ Settings
Event

» OSD info

» Font

Storage Picture Overiay

System
Information

» Abnormal

» GPS Position

Upload Picture

Requirement for picture upload
1. Max size s 16k

2. Max resolution is 128128 pixels.
3.256 colors, bmp format.





Figure 3-23
[image: image93.png][P P1Z Dome [ e [ e | cewr | sam [ iosou ]

Path

Camera Snapshot
> Conditions.

> Audio » Channel Titie

» Privacy Mask Enable (@) Disable

Network » Time Tite
PTZ Settings » OSD info
=l 3 ' » Font

Storage : » Picture Overlay

o o b= ]

Information » GPS Position





Figure 3-24

[image: image94.png][P P1Z Dome [ e [ e | cewr | sam [ iosou ]

Path

Camera Snapshot
> Conditions.

> Audio » Channel Titie

» Privacy Mask GPS Position () Enable (® Disable

Network » Time Tite
PTZ Settings » OSD info
=l » Font

Storage » Picture Overlay
System <
Information

» Abnormal

» GPS Position





Figure 3-25
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Privacy mask 
	· Click “Draw” to draw privacy mask in the image preview area. 

· Click “Delete” to delete corresponding privacy mask.

· Click “Clear” to clear all the privacy mask areas.

· Set “Privacy Mask SN”, click “Go to” to check the corresponding privacy mask area of the “Privacy Mask SN”. 

	Channel Title 
	· Check “Enable” to display channel title in the video monitoring window; check “Disable” not to display.

· You can use the mouse to drag the channel tile to adjust the position of channel title.

	Time Title
	· Check “Enable” to display time title in the video monitoring window; check “Disable” not to display. 

· You can drag “Time Title” box to adjust the position of time title.
· Check “Display Week” to display week info on the time title.   

	OSD
	· Check the corresponding “Enable” button, and it will display preset, temperature, PTZ coordinates, zoom, north and text overlay in the video monitoring window; check corresponding “Disable “button and it won’t display. 
· Click “Set North” to set the current location as north.

· You can adjust preset, temperature, PTZ coordinates, zoom, north and text overlay via dragging “OSD Info” box. Alignment include align left and align right.

	Font
	· It is set the font of channel title, time title, OSD info, it can set color, size and row height. 

	Picture Overlay
	· You can enable this function to display overlay picture. Click disable to turn it off. 

· Click Upload Picture to overlay local picture into monitoring window. You can drag the yellow box to move it. 

Note:

You cannot enable OSD info and picture overlay at the same time.  

	Abnormal 
	· It is to set if it will display abnormity in the monitoring picture. 

	GPS position 
	· It is to set if it will display longitude and latitude in the monitoring image. 




Step 3 

Click “Save” to make config valid. 

3.1.2.4 ROI 
You can set the key monitoring area as the ROI and set image quality upon the area. 

Step 1 

Select “Setup > Camera > Video > ROI”. 

The system will display the interface of “ROI”, which is shown in Figure 3-26.

[image: image95.png]IP PTZ Dome

Camera

Snapshot Overlay

Path
> Conditions

© Enable ) Disable
> Audio

Image Quality |6
Network

PTZ Settings
Event
Storage
System
Information

IP PTZ Dome

Renoveal, .. oo, (et

[T | | —





Figure 3-26
Note: 

Some models don’t support ROI function. 

Step 2 

Select “Enable “to enable ROI function. 

Step 3 

Press the left mouse button and draw area on the video image. It can set max 4 areas. Click “Delete” to delete corresponding area. Click “Remove all” to remove all the areas. 
Step 4 
Set the image quality of the corresponding ROI. 

Step 5 

Click “Save” to make config valid. 

3.1.2.5 Path 
The storage path is activated with snapshot and record in the live interface, which can set the storage path of monitoring snapshot and monitoring record respectively. 
The storage path is activated with snapshot, download and clip in the playback interface, which can set the storage path of playback snapshot, record download and playback clip respectively. 

Step 1 
Select “Setup > Camera > Video > Path”. 

The system will display the interface of “Storage Path”, which is shown in Figure 3-27. 

[image: image96.png]IP PTZ Dome

Camera
Network

PTZ Settings
Event
Storage
system
Information

Video

Live Snapshot
Live Record

Playback Snapshot
Playback Download

Video Clips

Snapshot

C:\Users\2:
Ci\Users\2
C:\Users\2:

C:\Users\2:

Overlay

load\LiveSnapshot

load\LiveRecord

load\PlaybackRecord

load\VideoClips

Path

Browse,
Browse,
Browse,
Browse,

Browse,





Figure 3-27

Step 2 

Set the corresponding storage path. 
· The default monitoring snapshot path: C:\Users\admin\WEBDownload\LiveSnapshot.
· The default monitoring record path：C:\Users\admin\WEBDownload\LiveRecord.
· The default playback snapshot path: C:\Users\admin\WEBDownload\PlaybackSnapshot.
· The default playback download path: C:\Users\admin\WEBDownload\PlaybackRecord.
· The default playback clip path: C:\Users\admin\WEBDownload\VideoClips.
Note:

Admin is locally logged in PC account. 

Step 3 

Please click the “Save” button to make config valid. 
3.1.3 Audio 

Note: 

Some models don’t support audio function. 

Step 1 

Select “Setup > Camera > Audio”. 

The system will display the interface of “Audio”, which is shown in Figure 3-28. 

[image: image97.png]IP PTZ Dome

Camera
> Conditions

Video

PTZ Settings
Event
Storage
System
Information

Audio

Encode
Main Stream
Enable
Encode Mode GT11A

‘Sampling Frequency | 16k

Sub Stream
Enable Sub Stream 1
Encode Mode GT11A

‘Sampling Frequency | 16k

Aftibute
Audioln Type Lineln
Noise Filter Enable

Microphone Volume ()

=

‘Speaker Volume

100





Figure 3-28

Step 2 

Please configure info of each parameter according to the actual needs, refer to the following sheet for more details. 
	Parameter 
	Function 

	Audio enable 
	· Check Enable: the stream is A/V composite stream, otherwise it contains video only. 

Note: 

Audio can be enabled only when video is enabled. 

	Encode mode 
	The encode mode includes G.711A and G.711Mu. 

The default is G.711A. 
Note: 

The audio encode mode set here can make both audio stream and bidirectional talk valid at the same time.  

	Sampling frequency 
	It includes 8k and 16k. It is 8K by default. 

	Audio in type
	It is to set audio input type, it is LineIn by default. 

	Noise filter
	It is to set if it is to enable noise filter function, it is enable by default. 

	Microphone volume
	It is to adjust the volume of the microphone; the value range is from 0 to 100. 
Note: 

The function is only supported by some models

	Speaker volume
	It is to adjust the volume of the speaker; the value range is from 0 to 100. 
Note: 

The function is only supported by some models. 


Step 3 

Click “Save” to make config valid. 

3.2 Network 

3.2.1 TCP/IP
3.2.1.1 TCP/IP 
You need to configure the IP address and DNS server of the intelligent speed dome, make sure it can be mutually connected to other devices in the networking. 

Note: 

Please confirm the intelligent speed dome has connected to network correctly before setting network parameters. 

· Please distribute IP address of the same network segment if there is no router in the network. 

· It needs to set corresponding gateway and subnet mask if there is no router in the network. 

Step 1 
Select “Setup > Network > TCP/IP”.

The system displays the interface of “TCP/IP”, which is shown in Figure 3-29. 

[image: image98.png]T [ [ romsa | sor | om [ o]

Camera TCPIP Easy4ip ®@
Network

Connection Ethemet Card Wire(DEFAULT)
PPPOE

Mode @ Satc () DHCP
DONS

MAC Address
P Filler

1P Version
SMTP(EMail)

1P Address 1

] Subnet mask

Bonjour Default Gateway

[
[
Multcast Preferred DNSServer| 8 . 8 . 8
4

802.1x Altenate NS Server| 8 . 8

>
>
>
>
>
> UPnP
>
>
>
>
>

aus . Enavle ARFIFng o et adaress s
PTZ Settings

oo
Event

Storage
System
Information





Figure 3‑29
Step 2 

Configure TCP/IP parameter, refer to the following sheet. 
	Parameter 
	Function 

	Host Name 
	It is to set current host device name. It supports max 15 characters.  

	Ethernet Card
	Please select the Ethernet port. Default is wired. 

Please note you can modify the default Ethernet card if there is more than one card. .

Please note the device needs to reboot to activate the new setup once you modify the default setup. 

	Mode
	There are two modes: static mode and the DHCP mode. Select DHCP mode, it auto searches IP, and you cannot set IP/subnet mask/gateway. Select static mode, you must manually set IP/subnet mask/gateway. 

	Mac Address 
	It is to display device Mac address. 

	IP Version 
	It is to select IP version. IPV4 or IPV6.

You can access the IP address of these two versions.

	IP Address 
	Please use the keyboard to input the corresponding number to modify the IP address and then set the corresponding subnet mask and the default gateway. 

	Subnet mask 
	It has to set according to the actual situation, the prefix of subnet is number, input from 1 to 255, the prefix of subnet identifies a specific network link, and usually it includes a layering structure. 

	Default gateway
	Make sure it has to be in the same segment with IP address according to the actual situation.
	Note: 

It inputs 128 bit for IP address, default gateway, preferred DNS, alternate DNS of IPv6 version, it can’t be null. 

	Preferred DNS
	DNS server IP address.
	

	Alternate DNS 
	Alternate IP address of DNS server. 
	

	Enable ARP/Ping to set device IP address service. 
	Check, you can use ARP/Ping command to modify or set the device IP address if you know the device MAC address. 

When it is enabled by default, ping packet can set device IP via specific length within 2 minutes during device reboot, the service is off 2 minutes later, the service will be closed immediately after setting IP successfully. Ping packet can’t set IP if it is not enabled.  


Step 3 

Click “Save” to complete setting. 
An example of setting device IP via ARP/Ping

Step 1 

Get an unoccupied IP address, and make sure the device and PC are in the same LAN. 

Step 2 

Get the physical address of the device from the label.

Step 3 

Input the following commands in the PC. 
	System 
	Command 

	Windows syntax
	Arp  -s  <IP Address>  <MAC>

Ping  -l  480  -t  < IP Address >

Example：
Arp  -s  192.168.0.125  11-40-8c-18-10-11

Ping  -l  480  -t  192.168.0.125

	UNIX/Linux/Mac syntax
	Arp  -s  <IP Address>  <MAC>

Ping  -s  480  < IP Address >

Example：
Arp  -s  192.168.0.125  11-40-8c-18-10-11

Ping  -s  480  192.168.0.125

	Win7 syntax
	netsh  i  i  show  in

netsh  -c  “i i”  add  neighbors  ldx  <IP Address>  <MAC>

ping  -l  480  -t  < IP Address >

Example：
netsh  i  i  show  in

netsh  -c  “i i”  add  neighbors  12  192.168.0.125  11-40-8c-18-10-11

ping  -l  480  -t  192.168.0.125


Step 4 
Power off and reboot the device or reboot the device via network. 

Step 5 

Check the similar info like “Reply from 192.168.0.125…” from the PC command line, then it can set successfully; you can close the command line. 

Step 6 

Open the browser and then input http://<IP address>. Click the Enter button, you can access now.
3.2.1.2  Easy 4ip 
Easy 4ip module is mainly used to visit device via serial number, it doesn’t need to set IP address, plug and play, and scan QR code to log in the device.

[image: image99.png]IP PTZ Dome

Camera Easyaip
Network

Connection
PPPOE
DDNS

1P Filler
SMTP(Email)
UPnP.
SNMP
Bonjour
Mulicast
802.1x

Qos
PTZ Settings
Event
Storage
System
Information





Figure 3-30
3.2.2 Connection 
3.2.2.1 Connection 
You can configure the device with max connection port and each port value on this interface. 
Step 1 

Select “Setup > Network > Connection > Connection”. 

The system will display the interface of “Connection”, which is shown in Figure 3-31. 
[image: image100.png]IP PTZ Dome

Camera

Network
SMTP(Email)
e
-
QoS
PTZ Settings
Event

>
>
>
>
>
>
>
>
>
>

Storage
System
Information

Connection

Max Connection
TCP Port

UDP Port
HTTP Port
RTSP Port
HTTPs.

HTTPs Port

Default





Figure 3‑31
Step 2 

Configure each port value of the device; refer to the following sheet for more details. 
	Parameter 
	Function 

	Max connection 
	It is the max Web connection for the same device. The value ranges from 1 to 20. Default connection amount is 10.

	TCP port 
	Port range is 1025~65534. The default value is 37777. You can input the actual port number if necessary. 

	UDP port 
	Port range is 1025~65534. The default value is 37778. You can input the actual port number if necessary.

	HTTP port 
	Port range is 1025~65524. The default value is 80. You can input the actual port number if necessary.

	RTSP port
	· The default value is 554. Please leave blank if use default. User uses QuickTime or VLC can play the following formats. BlackBerry can play too. 

· Real-time monitoring URL format, please require real-time RTSP media server, require channel no., bit stream type in URL. You may need username and password. 

· User uses BlackBerry need to set encode mode to H.264B, resolution to CIF and turn off audio. 

URL format is:

rtsp://username:password@ip:port/cam/realmonitor?channel=1&subtype=0

username/password/IP and port. 

The IP is device IP and the port default value is 554. You can leave it in blank if it is the default value. 

Follow standard RTP protocol and when encode mode is MJPEG; the max resolution only supports 2040*2040. 

	HTTPs Enable 
	Check HTTPs enable, login as https://ip:port. Protect data. Default port is https://ip . It is disabled by default. 

	HTTPs Port
	HTTPs communication port, range is 1025~65534, default is 443. 


Note:
Except “max connection”, it needs to reboot the device to make it valid after modifying other parameter config. 
Step 3

Click “Save” to complete setting. 
3.2.2.2 ONVIF

ONVIF (Open Network Video Interface Forum) describes network video model, port, data type and data switch modes. ONVIF standard targets to create a network video frame protocol that communicates network video products from different manufacturers. 
Step 1 

Select “Setup > Network > Connection > ONVIF”. 

The system will display the interface of “ONVIF”, which is shown in Figure 3-32.
[image: image101.png]{F 1z Bome [ e [ e | e | aem | iosou ]
Camera ®@

Network
TCPIP

PPPOE
DDNS

1P Filler
SMTP(Email)
UPnP.
SNMP
Bonjour
Mulicast
802.1x

Information




Figure 3‑32
Step 2 

Set “Authentication” as “Enable”. 
Step 3 

Click “Save” to complete setting. 
3.2.3 PPPoE

It can set up network connection via enabling PPPoE (Point-to-Point Protocol over Ethernet) dial mode; the device will acquire a dynamic IP address of WAN. Please gain the PPPoE username and password provided by ISP (Internet service provider). 

Step 1 

Select “Setup > Network > PPPoE”. 

The system will display the interface of “PPPoE”, which is shown in Figure 3-33. 

[image: image102.png]{# 1z Bome T T T ST T
[

Camera

Network
> TcPIP
> Connection
>
DDNS
1P Filler
SMTP(Email)
UPnP.

>
>

>

>

> SNMP
> Bonjour
> Mulicast

> 8021

> Qos

PTZ Settings
Event
Storage
System
Information




Figure 3‑33
Step 2 

Check “Enable” and input PPPoE username and password. 

Step 3 

Click “Save” to complete PPPoE config. 

The system will prompt that it has been successfully saved and realtime display the acquired IP address of WAN, which is shown in Figure 3-34, users can visit the device via the IP address. 

[image: image103.jpg]7] Enable
Username hzhz01902107
Password .
1P Address 15 199 252 . 240

202 . 101 . 172 . 35

bemun_] [ retesn_|[L_sae





Figure 3-34

3.2.4 DDNS

DDNS (Dynamic Domain Name Server) can be used to update the relationship between domain name on the DNS server and IP address dynamically in the situation where the device IP address changes frequently, which is to guarantee the users to visit device via domain name. 

Note: 

· Please confirm if the device supports the type of DNS before config and log in the website of DDNS service provider to register domain name and other info. 

If DDNS type is Dahua DDNS, then it doesn’t need to register domain name. 
If DDNS type is other type, then it needs to log in corresponding DDNS website to register username, password, domain name and etc. 

· Users can check info of all the connected devices after they successfully registered in DDNS website and logged in. 
Step 1 

Select “Setup > Network > DDNS”. 

The system will display the interface of “DDNS”, which is shown in Figure 3-35. 
[image: image104.png]{# 1z Bome T T T ST T
[

Camera

Network
> TcPIP
> Connection
> PPPoE
main Name
1P Filler Usemame
SMTP(Email)
UPnP.
SNMP
Bonjour
Mulicast
802.1x

Qos
PTZ Settings
Event
Storage
System
Information




Figure 3‑35
Step 2 

Check “Server Type” and configure relevant parameter of DDNS according to the actual situation. 
· Please refer to the following sheet for configuring DDNS parameter if users select DDNS type as “Dahua DDNS”. 
	Parameter 
	Function 

	Server Type
	The default “Server Address” of Dahua DDNS is “www. Quickddns.com”. 

	Server Address
	DDNS server IP address. 

· CN99DDNS
Server address: www.3322.org

· NO-IPDDNS
Server address: dynupdate.no-ip.com

· DyndnsDDNS
Server address: members.dyndns.org

· QUICK DDNS
Server address: www.quickddns.com

	Mode 
	It is auto by default, manual is optional. 

	Domain Name 
	It is “MAC address. quickddns.com” by default in both auto and manual mode, users can set prefix by themselves. 

	Test 
	It is to test if the domain name is available. The parameter appears only when selecting “Mode” as “Manual”. 

	Username
	The user name you input to log in the server. It is optional. 


1. After filling in the interface, click “Test” to confirm if the domain name can be successfully registered. If it is successful, please continue to 2, if not, please check if the domain name info is correct and clear browser cache.
2. Click “Save”.

3. Input complete domain name in the PC browser and press enter. 

It means successful config if it can display the device WEB interface; it means config failure if it fails to display the interface, please configure again. 

· Please refer to the following sheet to configure parameter if it selects other types of DDNS. 
[image: image105.png]{# 1z Bome T T T ST T
[

Camera

Network
> TcPIP Dyndns DDN
> Connection
PPPOE main Name
1P Filler
SMTP(Email)
UPnP.
SNMP
Bonjour
Mulicast
802.1x

Qos
PTZ Settings
Event
Storage
System
Information




Figure 3‑36
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	DDNS Type
	The names and addresses of DDNS server provider are listed below: 
· Dyndns DDNS address is：members.dyndns.org

· NO-IP DDNS address is：dynupdate.no-ip.com

· CN99 DDNS address is：members.3322.org

PRIVATE DDNS address is：www.privateddns.com, as for private DDNS server, the port number can be configured according to its actual situation, which can realize device access with the mode of domain name + port number 

	Server address
	

	Domain Name
	It is the domain name that the users register on the website of DDNS server provider. 

	Username
	Input the username and password which are acquired from DDNS server provider. The users need to register account on the website of DDNS server provider (including username and password)

	Password
	

	Update Period
	After the designated DDNS is updated and enabled, it will launch the interval of update requirement regularly, the unit is minute. 


1. Click “Save” after filling in the interface. 

2. Input domain name in the PC browser and press “Enter” button. 

It means successful config if it can display the device WEB interface; it means config failure if it doesn’t display. 

3.2.5 IP filter

It can set the users who are allowed to visit the device via IP filter. 
· Trusted Sites: It is to add the users’ IP/MAC which can log in the device. If the users check the trusted sites, then only the users’ IP/MAC which is in the list can log in the device; if trusted sites in not checked, then there is no restriction for the users who visit the device. 
· Banned List: It is to add the users’ IP/MAC which refuses to log in the device. If the users check the banned list, then except the IP/MAC in the banned list, the other users’ IP/MAC can log in the device. 

· Users are not allowed to set the device IP/MAC as trusted sites.

· MAC verification can be valid only when the device IP and PC’s IP are in the same LAN. 

Note: 

· MAC verification can only be restricted according to the router’s MAC when visiting WAN. 
· Some models don’t support banned list, please refer to the actual product for more details. 

Step 1 

Select “Setup > Network > IP Filter”. 

The system will display the interface of “IP Filter”, which is shown in Figure 3-37. 
[image: image106.png]IP PTZ Dome

v [T (ST (PR

Camera IP Filter ®

Network
> TcPIP Trusted Sites

o e
> SMTP(Email)
e
A
- i
PTZ Settings

oeraut ||

Event
storage
system
Information





Figure 3‑37
Step 2 

Check corresponding check box to enable trusted sites and banned list. 

· Check “Trusted Sites”, click it and add trusted sites. 

1. Click “Add IP/MAC”, and configure IP address info according to the following sheet. 
	Parameter
	Note

	IP address
	Input the IP address of host which is to be added.

	IP segment
	Input the start address and end address of the segment which is to be added. 

	IPv4
	IP address adopts IPv4 format, such as 172.16.5.10。

	IPv6
	IP address adopts IPv6 format, such as aa:aa:aa:aa:aa:aa:aa:aa。

	MAC
	Input the MAC address of host which is to be added. 


2. Click “Save” to make config valid. 

Use the IP host in the trusted sites to log in device WEB interface, which can realize successful device login. 

· Check “Banned List”, click it to add banned list. 
1. Complete adding banned list according to the sheet above. 
Note: 

Add banned list, but it doesn’t support adding MAC address. 

2. Click “Save” to make config valid. 

Use the IP host in the banned list to log in the device WEB interface. The system prompts that it has been added into banned list, and it fails to log in.  
3.2.6 SMTP （e-mail)

By setting SMPT, it will send email immediately when alarm, video detection and abnormity happen. 

When alarm, video detection and abnormity trigger, it can send email to the server of the receiver via SMPT server. The receiver can receive the email when logging in the server. 

Step 1 

Select “Setup > Network > SMPT”. 

The system will display the interface of “SMPT”, which is shown in Figure 3-38. 

[image: image107.png]IP PTZ Dome

Camera SMTP(Email)
Network
e SMITP Server
Connection Port
PPPOE Anonymity
DDNS Usemame anonymity
1P Filter Password

Sender

UPnP.

SNMP Authentication
Bonjour Title I Atiachment
Mulicast Mail Receiver

1
[
PTZ Settings
Event

Interval 0 ‘Second(0-3600)

Health Mail Update Period 60 ‘Second(1-3600)
Storage

System Email Test

Information ‘

oo





Figure 3‑38
Step 2 

Configure info of each parameter according to the actual needs. 
	Parameter 
	Function 

	SMTP Server 
	Conform to SMTP protocol; send the IP address of email server.

	Port 
	Conform to SMTP protocol; send the port number of email server, it is 25 by default. 

	Anonymity 
	For the server supports the anonymity function. You can auto login anonymously. You do not need to input the user name, password and the sender information.  

	User Name
	The user name of the sender email account.

	Password
	The password of sender email account.

	Sender
	Sender email address.

	Authentication (Encryption mode) 
	You can select SSL, TLS or none. 

	Title
	Email title, and it can be customized.


	Attachment 
	System can send out the email of the snapshot picture once you check the box here. 

	Mail receiver
	Input receiver email address here. Max three addresses.

	Interval 
	The send interval ranges from 0 to 3600 seconds. 0 means there is no interval.

Please note system will not send out the email immediately when the alarm occurs. When the alarm, motion detection or the abnormity event activates the email, system sends out the email according to the interval you specified here. This function is very useful when there are too many emails activated by the abnormity events, which may result in heavy load for the email server.

	Health mail enable 
	Please check the box here to enable this function. 

	Email test 
	The system will automatically sent out an email once to test the connection is OK or not .Before the email test, please save the email setup information. 


3.2.7 UPnP

It allows you to establish the mapping relationship between the LAN and the public network.

Here you can also add, modify or remove UPnP item. For UPnP on different routers, you must disable UPnP function.
Enable UPnP, network cameras support UPnP protocol. In Windows Xp or Windows Vista system, if system UPnP is enabled, then the network camera can auto search it in the network neighborhood of Windows. 
Please refer to the following steps to install UPnP network service in the Windows system: 

Step 1 

Open control panel, and select “Add or Remove Programs”. 

Step 2 

Click the “Add/Remove Windows Components”
Step 3 

Select the “Network Services” from the Windows Components Wizard. Click the Details button
Step 4 

Check the “Internet Gateway Device Discovery and Control client” and “UPnP User Interface”. Please click OK to begin installation.
The operation steps of UPnP config is shown as follows: 
Step 1 

Select “Setup > Network > UPnP”. 

The system will display the interface of “UPnP”, which is shown in Figure 3-39. 

[image: image108.png]IP PTZ Dome

Camera

Network

> TcPP Mode | Manual v Router State Mapping Failed
> Connection el

> PPPoE

WebSenice:TCP Mapping Failed
> DDNS

PrivSenvice:TCP. Mapping Failed
> IP Filler

PrivService:UDP. Mapping Failed
> SMTP(EMail)

> SNMP.

RTSPService:TCP. Mapping Failed

> Bonjour
> Muttcast
> 8021x

> Qos

PTZ Settings
Event
Storage
System
Information





Figure 3-39
Step 2 

Check the box and enable UPnP function. 

Step 3 Selection mode

There are two mapping modes for UPnP which are auto and manual. As for manual mapping mode, it allows users to modify external port; as for auto mapping mode, it completes port mapping automatically without occupying the port, and users don’t need to modify mapping. 

Step 4 

Click “Save” to make config valid. 

3.2.8 SNMP

The SNMP (Simple Network Management Protocol) provides framework of bottom-layer network management for network management system. It can control SNMP function in the network service setting. It can gain the relevant config info after connecting to device via relevant software tool. It needs to satisfy the following conditions if it is to use SNMP function: 

· Install SNMP device monitoring and management tool, such as MIB Builder and MG-SOFT MIB Browser. 

· Get two MIB documents which are corresponding to the current version from technical personnel. 

Step 1 

Select “Setup > Network > UPnP”. 

The system will display the interface of “SNMP”, which is shown in Figure 3-40 and Figure 3-41

[image: image109.png]IP PTZ Dome

Camera
Network

> TCPIP

> Connection
> PPPOE

> DDNS

> IP Fiteer

> SMTP(EMail)
> UPnP.

N

> Bonjour

> Mulcast

> 802.1x
> Qos

PTZ Settings
Event
Storage
System

Information

SNMP Version

SNMP Port
Read Community
Wite Community
Trap Address
Trap Port

Keep Alive

SNMP V1 SNMP V2

161
public

private

SNMP V3

Default





Figure 3‑40
[image: image110.png]IP PTZ Dome

Camera
Network
TCPIP
Connection
PPPOE
DDNS

P Filler
SMTP(EMail)
uPnP

Bonjour
Multcast

802.1x

>
>
>
>
>
>
>
>
>
>
>
>

[
PTZ Settings
Event
Storage
System
Information

SNMP Version SNMP v

SNMP Port 161
Read Community | public
Wite Communiy | private
Trap Address

Trap Port

Keep Alive

Read-only Usemarme  public
Authentication Type (® MDS

Authentication
Password

EncryptionType  (®) CBC-DES.

Encryption Password

Read&write private
Username

Authentication Type (® MD5

Authentication
Password

EncryptionType  (®) CBC-DES.

Encryption Password

SNMP 2 < SNMPV3

(1~65535)

SHA

‘The minimum pass phrase length is & characters

‘The minimum pass phrase length is & characters

SHA

‘The minimum pass phrase length is & characters

‘The minimum pass phrase length is & characters

Reresn_ ][5





Figure 3-41

Step 2 

Configure info of each parameter according to the actual needs.  

	Parameter 
	Function 

	SNMP Version
	· Check SNMP v1, device can only process v1 info. 

· Check SNMP v2, device can only process v2 info. 

· Check SNMP v3, can set username, password and encryption method. Server calibrate corresponding username, password and encryption method too access device and v1/v2 are unavailable. 

	SNMP port 
	The listening port of the proxy program of the device. It is a UDP port not a TCP port. The value ranges from 1 to 65535. The default value is 161

	Community
	It is a string, as command between management and proxy, , defining a proxy,  and a manager’s authentication. 

	Read community 
	Read-only access to all SNMP targets, default is public.

Note: Only number, letter, _, and – supported. 

	Write community 
	Read/write access to all SNMP targets, default is private.

Note: Only number, letter, _, and – supported.

	Trap address
	The destination address of the Trap information from the proxy program of the device. 



	Trap
	SNMP trap is a proxy message sent to admin as important event notice or status change.

	Trap Port
	Port which send Trap message, default is 162, range 1~65535.

	Read-only Username
	Default is public.

Note:

Name only can be number, letter and underline. 

	Read/Write Username
	Default is private.

Note:

Name only can be number, letter and underline. 

	Authentication
	You may select MD5 or SHA, default is MD5.

	Authentication Password
	Password not less than 8 characters. 

	Encryption 
	Default is CBC-DES.

	Encryption Password
	Password not less than 8 characters. 


Step 3 
Click “Save” to make config valid. 
3.2.9 Bonjour

Bonjour, known as zero config networking, can auto discover the PC, device and service on the IP network. Bonjour uses the IP protocol with industrial standard to allow the device to discover each other automatically without inputting IP address or configuring DNS server. 

After Bonjour function is enabled, the network cameras will be auto detected in the operating system and client which support Bonjour. When the network camera is auto detected by Bonjour, it will display the “Server Name” which is configured by users. 

Step 1 

Select “Setup > Network > Bonjour”. 

The system will display the interface of “Bonjour”, which is shown in Figure 3-42. 

[image: image111.png][BFTEbome ST T TR TN T

Camera Bonjour ®@
Network

TCPIP
Connection ame 1E03BSTPANO
PPPOE
DDNS
1P Filler

UPnP.
SNMP

>
>

>

>

>

> SMTP(EMail)
>

>

>

> Mulicast

> 8021

> Qos

PTZ Settings
Event
Storage
System
Information




Figure 3-42
Step 2 

Check “Enable” to set server name. 

Step 3 

Click “Save” to make config valid. 

In the operating system and client which support Bonjour, it can visit network camera WEN interface via Safari browser, the steps are as follows: 
Step 1 

Click Safari browser “Display All Bookmarks”.

Step 2 

Open “Bonjour”, and it can auto detect the network camera which enables Bonjour function in the LAN. 

Step 3 

Click it to visit corresponding WEB page. 

3.2.10 Multicast

Preview video image via network device visit, it will fail to preview video image if it exceeds the visit upper limit of the device, at this moment you can solve the problem by adopting multicast protocol visit via setting multicast IP to the device. 
Step 1 
Select “Setup > Network > Multicast”. 

The system will display the interface of “Multicast”, which is shown in Figure 3-43. 

[image: image112.png]{# 1z Bome T T T ST T

Camera Multicast ®@
Network
fR— Main Stream Sub Stream

> Connection v E

DDNS 49400 5-65

1P Filter
SMTP(Email)
UPnP.
SNMP

Bonjour

802.1x

Qos
PTZ Settings
Event
Storage
System
Information




Figure 3-43
Step 2 
Check “Enable” to enable multicast. 

Step 3 

Input multicast address and port. 

Step 4 

Click “Save” to complete config. 

3.2.11 802.1x

802.1x (port based network access control protocol) supports manual selection of authentication method to control if device connected to LAN can join the LAN. It well supports authentication, charging, safety and management requirement of network.
Step 1 

Select “Setup > Network > 802.1x”. 

The system will display the interface of “802.1x”, which is shown in Figure 3-47. 

[image: image113.png]{# 1z Bome T T T ST T
[

Camera

Network
TCPIP

Connection

>
>
>
> DDNS
> IP Fitter

> SMTP(EMail)
> UPnP.

> SNMP

> Bonjour

> Mulicast

>

> Qos

PTZ Settings
Event
Storage
System
Information




Figure 3‑47
Step 2 

Check “Enable” to enable 802.1x function. 

Step 3 

Select authentication mode, set username and password. 
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Authentication
	PEAP (protected EAP protocol).  

	Username
	It needs the username to login, which is authenticated by the server.

	Password
	Please input password here. 


Step 4 
Click “Save” to complete config. 

3.2.12 QoS

Qos (Quality of Service) is network security mechanism. It is a technology to fix the network delay and jam problem and etc. For the network service, the quality of service includes the transmission bandwidth, delay, the packet loss and etc. We can guarantee the transmission bandwidth, lower the delay, and reduce the loss of the data packet and anti-dither to enhance the quality. 

We can set the DSCP (Differentiated Services Code Point) of the IP to distinguish the data packet so that the router or the hub can provide different services for various data packets. It can select the different queues according to the priority (64 different priority levels) of the packets and select the bandwidth of the each queue. Level 0 is the lowest, and level 63 is the highest. It can also discard at the different ratio when the broad bandwidth is jam. 
Step 1 

Select “Setup > Network > QoS”. 

The system will display the interface of “QoS”, which is shown in Figure 3-48. 
[image: image114.png]{# 1z Bome T T T ST T
Camera ®@

Network
> TcPIP
> Connection
PPPOE
DDNS
1P Filler

SMTP(Email)
UPnP.
SNMP
Bonjour
Mulicast
802.1x

PTZ Settings
Event
Storage
System
Information




Figure 3‑48
Step 2 

Set realtime monitor and operation command. 

Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Realtime monitor 
	Data packet of network video monitoring, the value ranges from 0 to 63. 

	Command 
	Configure or inquire non-monitoring data packet for the device, the value ranges from 0 to 63. 


Step 3 
Click “Save” to complete config

3.3 PTZ Setting 

3.3.1 Protocol 
3.3.1.1 Network PTZ 
Step 1 

Select “Setup > PTZ Setting > Protocol > Network PTZ”. 

The system will display the interface of “Network PTZ”, which is shown in Figure 3-49. 

[image: image115.png]IP PTZ Dome

Camera Network PTZ Analog PTZ
Network

PTZ Settings
>

> Function
Event
Storage
System
Information





Figure 3-49
Step 2 

Set PTZ protocol. 

Step 3 

Click “Save” to complete config. 

3.3.1.2 Analog PTZ 

Step 1 

Select “Setup > PTZ Setting > Protocol > Analog PTZ”. 

The system will display the interface of “Analog PTZ”, which is shown in Figure 3-50. 

[image: image116.png]IP PTZ Dome

Camera
Network
PTZ Settings

> Functon
Event
Storage
System

Information

Network PTZ Analog PTZ

Address
Baudrate
Data Bit
StwopBit

Parity

@




Figure 3-50

Note: 

Some models don’t support analog PTZ function. 

Step 2 

Configure info of each parameter according to the actual needs; refer to the following sheet for more details.

	Parameter 
	Function 

	Address
	It is to set corresponding device address.
Note: 

Make sure the address is the same as that of the device, otherwise it will fail to control the device.   

	Baud Rate 
	It is to select the baud rate used by he device.

	Data Bit 
	It is 8 by default. 

	Stop Bit 
	It is 1 by default. 

	Parity 
	It is none by default.


Step 3 
Click “Save” to complete config.

3.3.2 Function 
3.3.2.1 Preset 
Preset means the current environment in which the camera is located, users can adjust the PTZ and camera to the environment quickly via calling preset. 
Step 1 

Select “Setup > PTZ Setting > Function > Preset”. 

The system will display the interface of “Preset”
Step 2 

Add and call preset according to the method shown in Figure 3-51. 
[image: image117.jpg]IP PTZ Dome

Playback

Camera Function
Network

Vhih Presett

> Protocol

Event

2 Preset2 in
3 Preset3 (1

Storage
system : § » Przspecd
Information % > 1ol Motion

» PowerUp

» PTZLimit
» TmeTask |

» PTZRestart

» CameraRestart

A
=) 2 64 » Defaut

—) Forws (4

—) ms (4





Figure 3-51
Note: 

· Double click “Preset title” to modify the title to be displayed on the monitoring screen for the preset. 
· Click [image: image118.png]


to delete the preset. 

· Click “Clear” to remove all the preset. 
3.3.2.2 Tour

Tour can realize auto movement according to the set preset. 

Note: 

It needs to set several presets in advance. 

Step 1 

Select “Setup > PTZ Setting > Function > Tour”. 

The system will display the interface of “Tour”.

Step 2 

Add tour according to the method shown in Figure 3-52. 

[image: image119.jpg]IP PTZ Dome

Function

Storage
System
Information

Zoom

Focus.

s

o

=

™

» Preset

» Patiem
» Pan

» PTZSpeed

» Idle Motion

» Powerlp

» PTZLimit

» Time Task

» PTZRestart

» Camera Restart

» Default

Playback





Figure 3‑52
Note: 

· Double click “Tour Name” to modify the name of the tour. 

· Double click “Duration” to set duration for each preset. 
Step 3 
Click “Start” to start tour. 

Note: 

The device will stop tour if PTZ is operated during tour. 

Step 4 

Click “Stop” to stop tour. 

3.3.2.3 Scan 
Scan means the speed dome scanning back and forth within the left and right limit with a certain speed. 

Step 1 

Select “ Setup > PTZ Setting > Function > Scan”. 

The system will display the interface of “Scan”, which is shown in Figure 3-53.

Step 2 

Set scan according to the method shown in Figure 3-53. 
[image: image120.jpg]IP PTZ Dome

Playback

Camera Function

Network
» Presst ScanNo. | 1 v

» Tour Teed O ©

PTZ S
> Protocol

Event

Storage » Pan

system =l o i » PTZSpeed

Information 3 £ > ldle Motion

[setRignt Limi{

» Powerlp
f » PrzLimit

» Time Task
» PTZRestart

» CameraRestart

A = 206 » Default
< >
—) Focus (4
¥
—) ms (+





Figure 3‑53
Step 3 

Click “Start” to start scan.

Step 4 

Click “Stop” to stop scan. 

3.3.2.4 Pattern 
Pattern can continuously record the operations implemented upon the device, such as pan, tilt, zoom, call preset and etc. You can directly call the pattern after it is saved completely.
Step 1 

Select “Setup > PTZ Setting > Function > Pattern”. 

The system will display the interface of “Pattern”. 

Step 2 

Set pattern according to the method shown in Figure 3-54. 

[image: image121.jpg]IP PTZ Dome

Playback

Camera Function

Network
» Preset Pattern No. | 1 b2

PTZ Settings
» Tour
> Protocol

Event
storage
system . B » PTZSpeed

Information 5 gl » (dle Motion

» PowerUp

» PTZLimit

» Time Task
» PTZRestart

» Camera Restart
A b, 016 » Default

—) Fowws (4

—) ms (4





Figure 3-54

Step 3 

Click “Start” to start pattern. 

Step 4 

Click “Stop” to stop pattern. 

3.3.2.5 Pan
Pan means the speed dome rotating continuously 360°horizontally with a certain speed. 

Step 1 

Select “Setup > PTZ Setting > Function > Pan”. 

The system will display the interface of “Pan”, which is shown in Figure 3-55. 

[image: image122.png]R Rz Bome [ e ras [ s [ mam [ osou ]

Camera Function [©)
Network
PTZ Settings
> Protocol

Event 5
storage , ! =N
System » PTZSpeed

Information » Idle Motion

Preset

Pan Speed (3

» Tour

» Scan

» Pattem

» Powerlp
» PTZLimit

» Time Task

» PTZRestart

» Camera Restart

» Default





Figure 3-55

Step 2 

Select pan speed, it is 5 by default. 

Step 3 

Click “Start” to make the PTZ rotate horizontally with the speed you just set. 

Step 4 

Click “Stop” to stop pan. 

3.3.2.6 PTZ speed 
PTZ speed means the rotation speed of the device. 

Step 1 

Select “Setup > PTZ Setting > Function  > PTZ Speed”. 

The system will display the interface of “PTZ Speed”, which is shown in Figure 3-56. 

[image: image123.png]7 Tz Bome AT ST TR TN AT

Camera Function [©)
Network
PTZ Settings
> Protocol

Event

» Presst PTZSpeed () Low @ Middle () High

» Tour
» Scan
» Patem
Storage 3 o Fem
System » PTZSpeed
Information » ldle Motion

» Powerlp

» PTZLimit

» Time Task

» PTZRestart

» Camera Restart

» Default





Figure 3-56
Step 2 

Select PTZ speed, it is “Middle” by default. 

The system will make the PTZ rotate with the speed you just set. 

3.3.2.7 Idle Motion
Idle motion means the device implementing the behavior which is set in advance when it is not receiving any valid command within the set time. 
Note: 

It needs to set preset, tour, scan and pattern in advance. 

Step 1

Select “Setup > PTZ Setting > Function > Idle Motion”. 

The system will display the interface of “Idle Motion”.

Step 2 

Set idle motion according to the method shown in Figure 3-57. 

[image: image124.jpg]IP PTZ Dome

Playback

Camera Function

Network

Preset Ton

PTZ

» Tour Tde Molon & Presel ) Tour
> probocal el -

] » patem Numper 1 Preseti g e
s » Pan iate Time [ 10 Minute

Storage
T = » PTZSpeed

Information
» Powerup

& Przumt

» Time Task

» PTZRestart

» Camera Restart

A
D) e » Default

—) Forws (4

—) ;s (+





Figure 3‑57
Step 3 

Click “Save” to complete config.

3.3.2.8 Power up
It means the motion which is auto operated by the device after it is powered up. 
Note: 
It needs to set preset, tour, scan and pattern in advance. 

Step 1 

Select “Setup > PTZ Setting > Function > Power up”. 

Step 2 

Set power up according to the method shown in Figure 3-58. 

[image: image125.jpg]IP PTZ Dome

Playback

Camera Function

Network

» Presst Enable ¥
PTZ

» Tour FowerUp ® Presel ) Tour
> Protocol

b & Scan Patierm
on Auto

» Patiem

Event Number | 1: Preset! v

a » Pan

System o ~ * @ § » PTZSpeed

Information

» Time Task
» PTZRestart

» CameraRestart

A et » Default
<« T
) Fouus (+
v
) s (4





Figure 3‑58
Note: 
When selecting “Auto”, the system will operate the last motion before power off. 

Step 3 

Click “Save” to complete config. 

3.3.2.9 PTZ Limit
PTZ limit function is used to set the movement area of the device, which makes the device move within area. 

Step 1 

Select “Setup > PTZ Setting > Function > PTZ Limit”. 

The system will display the interface of “PTZ Limit”. 

Step 2 

Set up line and down line according to the method shown in Figure 3-59. 

[image: image126.jpg]IP PTZ Dome Playback

Function

» Preset Enable . i

- » Tour
> Profocol o
[rTETE— » Soan
: » Patiem
Event
Storage » Pan Please enable Coordinates at Overlay ncton
system fl » F1zspecs o see PTZ Coordinates on video
Information > 1aie Motion 4
[ :

» Time Task
» PTZRestart
» CameraRestart

A
=) 2 6 » Defaut

Y Focus (+

—) ;s (4





Figure 3-59

Note: 

Click “Live” to preview the up line and down line which are already set. 
3.3.2.10 Time Task 
Time task is to implement relevant movements within the set period. 
Note: 

It needs to set preset, tour, scan and pattern in advance. 

Step 1 

Select “Setup > PTZ Setting > Function > Time Task”. 

The system will display the interface of “Time Task”. 

Step 2 

Set time task according to the method shown in Figure 3-60. 

[image: image127.jpg]IP PTZ Dome

Playback

Camera Function

Network

Preset Enable

PTZ

» Tour u
> Protocol 2 Time Task num 1 2
- b Sxm ISR
Event E 4 Lo Time Taskaction ©) Presel () Tour
Storage 2 g e » Pan Scan O Patiem e
f |
System & » PTZSpeed action number | 1: Prest ¥
Information 5 » wemoton | |[“AuoHome 30 Second o
0 Bzl [Treroasemn 5
» PTZLmit | Copy To TaskNo. [1

» PTZRestart

» CameraRestart

A
=)z 6 » Default

—) Forws (4

—) ms (4





Figure 3-60

Auto home time means the time it needs to take to auto recover time task when manually calling PTZ and interrupting time task. 
Note: 
· Select new time task number in the dropdown box of “Copy To Task No.”, and click “Copy” to copy the time task info to corresponding time task number. 

· Click “Clear All” to remove all the settings. 

3.3.2.11 PTZ Restart 
Step 1 

Select “Setup > PTZ Setting > Function > PTZ Restart”. 

The system will display the interface of “PTZ Restart”, which is shown in Figure 3-61. 

[image: image128.png]7 Tz Bome AT AT AT SR AT

Camera Function [©)
Network

PTZ Settings

> Protocol

Patier
Event

Preset

Tour

Pan

storage
System PTZSpeed

Information % Idle Motion

» Default





Figure 3-61

Step 2 
Click “PTZ Restart” and the system will restart PTZ. 

3.3.2.12 Camera Restart 
Step 1 
Select “Setup > PTZ Setting > Function > Camera Restart”.

The system will display the interface of “Camera Restart”, which is shown in Figure 3-62. 

[image: image129.png]7 Tz Bome AT AT AT SR AT

Camera Function [©)
Network

PTZ Settings

> Protocol

Patier
Event

Preset

Tour

Pan

storage
System o PTZSpeed

Information ¢ Idle Motion
PowerUp
PTZLimit
Time Task.
PTZRestart

» CameraRestart

» Default





Figure 3-62

Step 2 

Click “Camera Restart” and the system will restart the camera.

3.3.2.13 Default 
Attention: 

The function will delete all the PTZ config made by users, please operate after confirmation. 

Step 1 

Select “Setup > PTZ Setting > Function > Default”.

The system will display the interface of “Default”, which is shown in Figure 3-63. 
[image: image130.png]7 Tz Bome AT AT AT SR AT

Camera Function [©)
Network

PTZ Settings

> Protocol

Patier
Event

Preset

Tour

Pan

storage
System PTZSpeed

Information Idle Motion
PowerUp
PTZLimit
Time Task.
PTZRestart
» CameraRestart

» Default





Figure 3‑63
Step 2 
Click “Default” to recover all the default settings. 

3.4 Event 

3.4.1 Video detect
Video detect is a type of traffic information detection technology which is based on computer vision and image processing technology. It can collect traffic images via the cameras installed above the road, and apply computer vision and image processing technology to deal with image data to acquire realtime, rich and dynamic traffic information which can be used for traffic signal control, information release and etc. 
Video detect includes motion detect and video tamper. 
3.4.1.1 Motion Detect 
Step 1 

Select “Setup > Event > Video Detect > Motion Detect”. 

The system will display the interface of “Motion Detect”, which is shown in Figure 3-64. 

[image: image131.jpg]IP PTZ Dome

Playback

Camera Motion Detect | Video Tamper

Network
PTZ Settings Emite

Working Period

Anti-Dither Second (0~100)
> Audio Detect

Area

> VSPlan

Manual Control
> Intelligence Behavior

Excluded
> Face Defection
> HeatMap @ Record
> Aam Record Delay 0 Second (10-300)
> Aonormaty Gt B
e e Atarm Delay 10 Second (10-300)
System Send Email
Information

Piz

Snapshot

bemun__] [ retesn__|[L_sae





Figure 3-64

Step 2 

Click “Enable” and configure info of each parameter according to the actual needs. 

· Set working period. 

Click “Setup”, and it will display the interface of working period in Figure 3-65. 

[image: image132.jpg]Working Period

Sunday
Monday
Tuesday
Wednesday
Thursday
Friday

Saturday

Al Sunday (] Monday [ Tuesday () Wednesday () Thursday (] Friday [ Saturday
@i Periogt | 00: 00: 00 |-[ 23: 59 : 59
Perod2 [ 00: 00: 00 |-[ 23: 69 : 69
Period3 [ 00: 00: 00 |-[ 23: 69 : 69
Period4 [ 00: 00: 00 |-[ 23: 69 : 69
Periods | 00: 00: 00 |-[ 23: 69 : 69
Period | 00: 00: 00 |-[ 23: 69 : 69





Figure 3-65

· Set alarm period, it can enable alarm event within the range of set period. 

· There are totally six periods to set every day, click the check box in front of the period, and then the period can be valid. 

· Select week number (default is Sunday, if users select the whole week, it means setting can be applied to the whole week; users can also select the check box in front of the day to make separate setting for some days).
· Click “Save” button after setup, return to motion detect page. 
Note: 

You can also set working period via pressing left mouse button and dragging it on the setup interface.
· Set area 
Click “Setup” and you can set area in the interface shown in Figure 3-66. 
Different colors represent different areas. Each area can set different detection zones. Detection zone can be irregular and discontinuous.
[image: image133.jpg]Area

Region

Regiont

Name

Sensity (=

@5

Threshold (=)0

B
R

i

]

LSRN 2|

“““““ TRy
]
TR

i

(OrRightclick )

=z
5
&

Cancel

save





Figure 3-66
	Parameter
	Function 

	Name
	Default area name includes Region 1, Region 2, Region 3, Region 4 and custom. 

	Sensitivity 
	It is sensitivity of brightness as motion detection is more possible to be trigger with high sensitivity. You can set up to four areas. The range is 0~100. The recommenced value is 30~70. 

	Area threshold
	It is to check target object area related to detection area. The lower the area threshold, the easier to trigger motion detection. You can set up to four areas. The range is 0~100. The recommenced value is 1~10.

	Waveform
	Red means motion detect is triggered. Green means motion detect is not triggered.

	Delete all
	Clear all areas. 

	Delete
	Delete selected area. 


· Other parameters

	Parameter
	Function 

	Anti-dither
	System only memorizes one event during the anti-dither period. The unit is second, the value ranges from 0s to 100s.

	Record
	Check it and so when alarm occurs, system will auto record. You shall set record period in Storage>Schedule and select auto record in record control interface. 

	Record Delay 
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out 
	Check it to enable alarm activation output port, it can activate corresponding alarm output device when alarm occurs. 

	Alarm delay
	It means alarm delays a period of time to stop after alarm ends, the unit is second, and the value ranges from 10s to 300s. 

	Send Email 
	If you enabled this function, System can send out email to alert you when alarm occurs. User can set email address in Network>SMTP.

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

· The event type includes: preset, tour and pattern. 

	Snapshot 
	You need to check the box here so that system can backup motion detection snapshot file. You shall set snapshot period in Storage>Schedule.


3.4.1.2 Video Tamper 

Step 1 

Select “Setup > Event > Video Detect > Video Tamper”. 
The system will display the interface of “Video Tamper”, which is shown in Figure 3-67. 

[image: image134.jpg]IP PTZ Dome

Playback

Camera Motion Detect | Video Tamper

Network

PTZ Settings B

Working Period

[ vigeopetet |
> Audio Detect Record
> VS Plan Record Delay. Second (10-300)
> Intelligence Behavior R
» FEEiTE Alam Delay. Second (10-300)
> HeatMap

Send Email

> Alam o

> Abnomality
Snapshot

Storage
system Default Refresn

Information





Figure 3-67
Step 2 

Click “Enable” and configure info of each parameter according to the actual needs. 

Note: 

Please refer to “3.4.1.1 Motion Detect” for more details about parameter config. 

Step 3 

Click “Save” to complete config. 

3.4.2 Audio Detect 
Step 1 

Select “Setup > Event > Audio Detect”. 

The system will display the interface of “Audio Detect”, which is shown in Figure 3-68. 

[image: image135.png]IP PTZ Dome

Camera Audio Detect
Network
PTZ Settings Enable Input Abnormal

Enable Inensity Change
Sensitvty =
Threshold =

Intelligence Behavior

Face Detection

HeatMap
Alam
Abnormaiity

Storage

Working Period
System

Anti-Dither Second (0~100)
Information

Record

Record Delay. 10 ‘Second (10~300)

Relayout DB
10

Alarm Delay. Second (10~300)
‘Send Email
P1Z

‘Snapshot

Default





Figure 3-68
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter
	Function 

	Enable Input Abnormal 
	Check “Enable Input Abnormal” and it will activate alarm when detecting audio input abnormal.

	Enable Intensity Change
	Check “Enable Intensity Change” and it will activate alarm when detecting audio intensity change surpass threshold. 

	Sensitivity 
	Levels range from1 to100 and adjustable, the smaller the value, it means the input sound volume change surpassing continuous environment volume and it can be judged as audio abnormal, users can adjust according to the actual environment testing.    

	Threshold 
	Levels range from 1 to 100 and adjustable, which is used to set filtered environment sound intensity. If the environment noise is bigger, users need to set the value higher. Besides, users can adjust it according to the actual environment testing. 


Note: 
Please refer to “3.4.1.1 Motion Detect” for description of other parameters. 

Step 3 

Click “Save” to complete config.

3.4.3 IVS Plan 
Users need to select first to decide if heat map and preset-related intelligent functions can coexist at the same time. Each preset can set different function; the corresponding setting can be valid only when it selects exact intelligent functions. 
Note: 

· It needs to set preset in advance, please refer to “3.3.2.1 Preset” for more details about preset setting. 

· Heat map and intelligent function added by preset can’t be enabled at the same time. 

Step 1 

Select “ Setup > Event > IVS Plan”. 
The system will display the interface of “IVS Plan”, which is shown in Figure 3-69.

[image: image136.png]IP PTZ Dome

Camera IVS Plan
Network

PTZ Settings
Event

> Video Detect

> Audio Detect

>

> Intelligence Behavior
> Face Detection

> HeatMap

> Alam

> Abnommality
Storage

System
Information





Figure 3-69
Step 2 
Click “Add Plan”, select preset in the dropdown box and add intelligent plan, which is shown in Figure 3-70. It is selected when the icon is yellow. 
Note: 

It has to disable heat map to set intelligent plans. 

[image: image137.jpg]IP PTZ Dome Live Playback setup Alarm Logout

Camera IVS Plan
Network

PTZ Settings L o |

Event

> Video Detect Add Plan

> Audio Detect

[ vsPan | Presett x

> Intelligence Behavior

> Face Detection
> HeatMap
> Alam

> Abnomality

storage
Preser2 x

System
Information

Preset3 #





Figure 3-70

Step 3 
Click Save: to complete config. 

3.4.4 Intelligent Behavior 
Basic requirements for scene selection:
· The total proportion of the target shall not exceed 10% of the image. 

· The target size in the image shall not be less than 10 pixel ×10 pixel, the size of abandoned target shall be less than 15 pixel ×15 pixel (CIF image); the height and width of target shall be less than that of the image; it is recommended that the target height shall be around 10% of the image. 
· The brightness value of target and background shall be no less than 10 gray levels. 

· It has to guarantee that the target has to appear at least more than 2 seconds in the view, the movement distance exceeds the width of the target itself and it is no less than 15 pixels (CIF image).

· Try to lower the complexity of the monitoring analysis scene if possible; It is no t recommended to use intelligent analysis functions in the scene where the target is very crowded and light changes frequently. 
· Try to keep away from glass, ground reflect light, water surface and other areas; try to keep away from branches, shadow and mosquito interference area; try to keep away from backlight scene, avoid direct light. 

3.4.4.1 Config 
Step 1 
Select “Setup > Event > Intelligent Behavior > Config”. 

The system will display the interface of “Rule Config”. 

Step 2 

Add intelligent rule according to the method shown in Figure 3-71

[image: image138.jpg]IP PTZ Dome

Playback

Camera

Network
PTZ Settings

Event No. Rule Name Rule Type.

> Video Detect

Preset Preset3 v . Select prese

> Audio Detect
> VSPlan

R
Storage
System
Information




Figure 3-71

Enter “Config” interface, the unlock function is enabled automatically, the locking time is 180s, during this period, other control modes are invalid except manual control over the speed dome. You can click “Unlock” to unlock it. 
Note: 

It needs to configure preset in advance, please refer to “3.3.2.1 Preset” for more details.

3.4.4.1.1 Tripwire
It will trigger alarm when the target crosses the warning line according to the set movement direction. 

It needs some time and space to confirm the target after it appears, so it needs to leave some space on both sides of the warning line when setting warning line, please don’t set it near the obstacle. 

Application scene: it can only be applied to the scene where the target is sparse and it is basically blocked between targets, such as the perimeter protection area without security guard.  
Set tripwire rule according to the method shown in Figure 3-72. 

[image: image139.jpg]IP PTZ Dome

Camera
Network
PTZ Settings
Event
> Video Detect
> Audio Detect
> VS Plan
> Face Detection
> HeatMap
> Alarm
> Abnomality
Storage
System
Information

A003KB08

Target Filter

Unlock(127s)

© MaxSize

Min Size

8191

0

8191

0

Playback

Parameter Setup

Working Period

Direction A<>B

@ Mam Tack
Track Time 30

@ Recoa
Record Delay | 30

v romros (A2
Alarm Delay 10
-

Snapshot

Rule Type

Tripwire v

Second (5~300)

Second (10~300)

Second (10~300)

)





Figure 3-72

Note: 
The method to draw rules: Press the left mouse button to draw rule on the monitoring image, click right mouse button to finish rule drawing. 

Please refer to the following sheet for more details about the parameter setting. 
	Parameter
	Note 

	Working period
	Note: 

Set alarm period, it can enable alarm event within the range of set period.

Click “Setup” to pop out setting interface of “Working Period”. 

· You can input time number value or press the left mouse button, and drag on the setting interface directly. 

· There are totally six periods to set every day, click the check box in front of the period, and then the period can be valid. 

· Select week number (default is Sunday, if users select the whole week, it means setting can be applied to the whole week; users can also select the check box in front of the day to make separate setting for some days).
Click “Save” button after setup is completed, return to rule config setting interface, click “Save” to complete period setting of tripwire.  

	Direction
	It is set the direction of tripwire, you can select A→B、B→A、A↔B. 

	Alarm track
	Check it and it will generate alarm track when the target triggers intelligent

rules. 

	Track time
	It is to set the time of track time. 

	Record
	Check it and so when alarm occurs, system will auto record. You shall set
record period of alarm in “Storage>Schedule” and select auto record in
record control interface. 

	Record delay
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay out
	Check it to enable alarm activation output port, it can activate corresponding alarm output device when alarm occurs. 

	Alarm delay
	It means the alarm delays a period of time to stop after alarm ends, the unit is second, and the value ranges from 10s to 300s.

	Send Email
	Check it and the system can send out email to inform users when alarm occurs. User can set email address in “Network>SMTP (email)”. 

	PTZ
	Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.

The event type includes: preset, tour and pattern.

	Snapshot
	Check it and the system can realize alarm snapshot automatically when alarm occurs, meanwhile it needs to set snapshot period of alarm in “
Storage>Schedule”.

	Target filter
	Check some intelligent rule, click “Draw Target”, then you can draw the model of filter target according to the rule in the scene. Click “Clear” to delete all the target filter models which have been drawn. 


Note: 
Double click “Rule Name” to modify the rule name. 

Click the “Clear” which is on the right of “Draw Rule” to remove all the rules which have been drawn.

3.4.4.1.2 Intrusion 
Intrusion includes cross area and in area functions. 
· Crossing area means that it will trigger alarm when target entering or leaving area. 

· In area function means that it will trigger alarm when certain amount of target appear in the designated area within specific time. In area function is only responsible for the statistics of the target quantity in the detection area without considering whether the target is the same or not. 

· As for the report interval of the in area function, the system can detect if the same event will occur within the interval after triggering the first alarm, the alarm counter will clear itself if no same event occurs during the period. 
As it is similar to the warning line, it also needs to leave some movement space for the target out of the area line if it is to detect enter and leave events
Application scene: It can only be applied to the scene where the target is sparse and it is basically mutually blocked between targets, such as the perimeter protection area without security guard. 
Please set the rule of intrusion according to the method shown in Figure 3-73. 
[image: image140.jpg]IP PTZ Dome

Playback

Camera
Network

PTZ Settings
Event

> Video Detect 1 Intrusion

> Audio Detect

> VSPlan

> Face Detection
Parameter St
> HeatMap

Working Periox

> Alam
> Abnomality

Storage Action ¢/ Appears || Cross
System | Alarm Track
Liiommation: TrackTime |30 Second (5~300) ete
: A I Record
TargetFilter  (®) MaxSize 8191 * 8191 Record Delay | 30 Second (10-300)

MinSze 0+ 0

o remar  [1)[2
Alarm Delay 10 Second (10~300)
U

Send Email

Snapshot





Figure 3-73

Please refer to the following sheet for parameter setting. 
	Parameter
	Note

	Action 
	It is to set the actions of intrusion, you can select appear and cross. 

	Direction
	It is to set the direction of cross area, you can select enter, leave, enter&leave.


Note: 
Please refer to “3.4.4.1.1 Tripwire” for more details about other parameters.

3.4.4.1.3 Abandoned Object 
It means triggering alarm when the abandoned object exceeds the time set by users in the monitoring scene. 
Abandoned and missing object detection can be confusing in a situation where foreground and background are both very complex. 

It will trigger alarm when pedestrians or vehicles stay still for a long time, which is considered as abandoned object. In order to filter this kind of alarm, generally the abandoned object is smaller than person and vehicle, therefore it can filter person and vehicle via setting filter size. Besides, it can avoid false alarm of short stay for people via extending alarm time. 
Application scene: It can be applied to the scene where the target sparse and there is no obvious and frequent light change. As for the scene with high target density and frequent blocking, alarm leakage will increase; as for the scene with more people to stay, false alarm will increase. As for the detection area, it is required to be simple; it can’t be applied to complicated areas. 

Please draw abandoned object rule according to the method shown in Figure 3-74. 

[image: image141.jpg]IP PTZ Dome

Playback

Camera
Network

PTZ Settings
Event Rule Type.

Preset Preset3

> Video Detect 1 Abandoned Ot v ||
> Audio Detect
> VS Plan

> Face Detection
Parameter Setup

> HeatMap

> Alam Working Period

> Abnomaiity
Keep Time Second (6~3600)
Storage

System @I Alam Track
Information Track Time Second (5~300)
I Record

TargetFilter (®) MaxSize 8191 * 8191 Record Delay |30 Second (10~300)

Minses 0 |- 0 @ rearot ()2

AamDelay | 10 Second (10~300)
Send Email

Snapshot





Figure 3-74

Please refer to the following sheet for the parameter setting. 
	Parameter 
	Note 

	Duration 
	It is to set the shortest time from object abandoned to trigger alarm. 


Note: 
Please refer to “3.4.4.1.1 Tripwire” for other parameter description. 

3.4.4.1.4 Missing Object
It means it will trigger alarm after the target in the scene is taken and exceeds a certain period of time. 
The system will make statistics for the still areas in the foreground area, and distinguish whether it is missing object or abandoned object according to the similarity between foreground and background, it will trigger alarm when it exceeds the time set by users. 
It may cause mistake when distinguishing abandoned object from missing object when both foreground and background are very complex. 
Application scene: It can be applied to the scene where the target sparse and there is no obvious and frequent light change. As for the scene with high target density and frequent blocking, alarm leakage will increase; as for the scene with more people to stay, false alarm will increase. As for the detection area, it is required to be simple; it can’t be applied to complicated areas. 

Please set the rule of missing object according to the method shown in Figure 3-75.
[image: image142.jpg]IP PTZ Dome Playback

Camera

Network AG09KS g Preset Preset3
PTZ Settings

Fvent No. Rule Name Rule Type.

> Video Detect ¥ 1 Rule Missing Objec v ||

> Audio Detect

> VSPlan . 3 I Pt L

> Face Detection
Farameter Setup

> HeatMap
o Working Perio
> Abnomality
KeepTime |10 Second (6~3600)
Storage
System | Alarm Track
BT TrackTme |30 Second (5~300) - .
I Record
TargetFiter (©) MaxSize | 8191 * 8191 Record Delay | 30 Second (10~300)
Minsze 0 |*0 < Relayout  [1][2
AamDelay | 10 Second (10~300)
Unlock(10s)
Send Email
Snapshot





Figure 3-75

Please refer to the following sheet for parameter setting. 

	Parameter
	Note

	Duration
	It is to set the shortest time from object disappears to trigger alarm.


Note: 
Please refer to “3.4.4.1.1 Tripwire” for more details about other parameters.

3.4.5 Face Detection
You can detect face and make snapshot in complex environment via face detection function.
Step 1 
Select “Setup > Event > Face Detect”. 

The system will display the interface of “Face Detect”. 

Step 2 

Please set face detection according to the method shown in Figure 3-76.
[image: image143.jpg]IP PTZ Dome

Camera
Network
PTZ Settings

> Video Detect
> Audio Detect
> VSPlan

> Inteligence Behavior

> HeatMap
> Alam

> Abnormality
Storage
System
Information

Face Detection

Playback

Enable 1. Clicl

Working Period

Enable Face Enhancement

Record

Record Delay 10 Second (10~300)

Reout @k
10

Alarm Delay

Second (10~300)
Send Email
P1Z

I Snapshot

bemn__] [ retesn__JL_sae





Figure 3-76

Please refer to the following sheet for parameter setting. 
	Parameter
	Note

	Enable Face Enhancement
	The detected face becomes clearer after clicking enable. 


Note: 
Please refer to “3.4.1.1 Motion Detect” for other parameter description.
3.4.6 Heat Map 

It is used to detect the activity level of moving object in the scene within a certain period of time. 
3.4.6.1 Heat Map 
Step 1 
Select “Setup > Event > Heat Map”. 

The system will display the interface of “Heat Map”. 

Step 2 
Set heat map according to the method shown in Figure 3-77. 

[image: image144.jpg]IP PTZ Dome

Playback

Camera Heat Map
Network

PTZ Settings

Event

Heat Map
Enable
> Video Detect
Working Period
> Audio Detect
> VSPlan
> Intelligence Behavior

> Face Detection

EETT—

> Alam

> Abnomality
Storage
System

Information
Zoom (+

s (4





Figure 3-77

Note: 

You can set the image of the heat map via direction buttons and zoom, focus and iris buttons which are under the image. 

Step 3 
You can search heat map according to the method shown in Figure 3-78. 

[image: image145.jpg]IP PTZ Dome

Playback

Camera Heat Map

Network
PTZ Settings Start Time | 2016-04-19 ®| 00 00 00 End Time | 2016-04-19 | 1 00 00 | |*Report Max Range Is 1 Week:
Event Heat Map 1 . 1. Set search conditions

> Video Detect

s

> Abnormality
Storage
System
Information





Figure 3-78

The darker the color is in the image, the more frequent the area activity becomes. 
Note: 

Click “Export” to export the statistics chart of the heat map. 

3.4.7 Alarm 

Step 1 
Select “Setup > Event > Alarm”. 

The system will display the interface of “Alarm”, which is shown in Figure 3-79.

[image: image146.png]IP PTZ Dome

Camera
Network
PTZ Settings
Event
 visaooe
3 Ao
3 s sonin
> Abnormality
Storage
system
Information

Relay Activation

Enable

Relay-n

Working Period

Anti-Ditner

e
Record Delay 10
Riood @2
Alarm Delay 10
senazna

-

Snapshot

Second (0~100)

SensorType [NO

Second (10-300)

Second (10-300)

Default





Figure 3-79
Step 2 

Configure info of each parameter according to the actual needs; refer to the following sheet for more details. 

	Parameter
	Function 

	Enable 
	After enabled, relay activation will work.

	Relay-in 
	Select relay-in, it can select 7-channel relay in.

	Sensor type 
	There are two options: NO/NC. Switch from NO to NC means enabling alarm; Switch from NC to NO means disabling alarm. 


Note: 
Please refer to “3.4.1.1 Motion Detect” for description of other parameters. 

Step 3 
Click “Save” to complete config. 

3.4.8 Abnormity 
Abnormality includes six alarm events which are no SD card, capacity warning, SD card error, disconnection, IP conflict and illegal access. 
3.4.8.1 SD Card 

Step 1 
Select “Setup > Event > Abnormity > SD Card”. 

The system will display the interface of “SD Card”, which is shown from Figure 3-80 to Figure 3-82. 

[image: image147.png]7T Dome IS TR T SN T

Camera SD Card Network llegal Access (€]
Network
PTZ Settings
Event Enable

> Video Defect | Relay-out @2
> Audio Detect Alarm Delay 10

EventType No SD Card

Second (10~300)

> VSPlan Send Email

> Intelligence Behavior
Default

3 e e
S o
Storage
system
Information





Figure 3-80

[image: image148.png]R FTz Dome IS TR T SN T

Camera sD Card Network llegal Access (€]
Network

PTZ Settings BT
Event Enable

> Video Defect | Relay-out @2
> Audio Detect Alarm Delay 10

SD Card Error.

Second (10~300)

> VSPlan Send Email
> Intelligence Behavior

Default
> Face Detection

> HeatMap

S o
Storage
system
Information





Figure 3-81

[image: image149.png]BTz Bome ST T TR ST AT

Camera SD Card Network llegal Access )
Network
PTZ Settings Event Type Capacity Waming

- e

> Video Detect Capacity Limit 10 %(0~99)

> Autio etet Reayut DB
10

> VS Plan Alarm Delay Scenduoss)
> Intelligence Behavior Send Email

> Face Detection

Default
> HeatMap

S o
Storage
system
Information





Figure 3-82
Step 2 

Configure info of each parameter according to the actual needs; refer to the following sheet for more details. 

	Parameter 
	Function 

	Enable
	Check to alarm when SD card is abnormal. 

	SD Card Capacity Lower Limit
	User can set SD card capacity percentage which is left. When SD card space left is smaller than this, alarm occurs. 


Note: 

Please refer to “3.4.1.1 Motion Detect” for description of other parameters. 

Step 3 

Click “Save” to complete config. 

3.4.8.2 Network 
Step 1 

Select “Setup > Event > Abnormity > Network”. 

The system will display the interface of “Network”, which is shown in Figure 3-83 and Figure 3-84. 

[image: image150.png][FF1z Dome T T T ST T

Camera SD Card llegal Access. ®@
Network

[ — Event Type Disconnection

et Enable

> Video Detect Record

> Audio Detect Record Delay 10 Second (10-300)

> v Pan Relayout DB
> Intelligence Behavior Alarm Delay 10

Second (10~300)
> Face Detection

(e [_reten
> Heatap

S o
Storage
system
Information





Figure 3-83

[image: image151.png][FF1z Dome T T T ST T

Camera SD Card Network llegal Access. ®@
Network

[ — Event Type IP Conflct

et Enable

> Video Detect Record

> Audio Detect Record Delay 10 Second (10-300)

> v Pan Relayout DB
> Intelligence Behavior Alarm Delay 10

Second (10~300)
> Face Detection

(e [_reten
> Heatap

S o
Storage
system
Information





Figure 3-84

Step 2 

Configure info of each parameter according to the actual needs; refer to the following sheet for more details. 

	Parameter
	Note

	Enable
	Click it to enable network abnormity alarm. 


Note: 

Please refer to “3.4.1.1 Motion Detect” for description of other parameters. 

Step 3 

Click “Save” to complete config. 

3.4.8.3 Illegal Access 

When the login password error reaches a certain number of times, it will generate illegal access alarm. 
Step 1 

Select “Setup > Event > Abnormity > Illegal Access”. 

The system will display the interface of “Illegal Access”, which is shown in Figure 3-85. 

[image: image152.png]{# 1z Bome T T T ST T

Camera SD Card Network lllegal Access ®@
Network

PTZ Settings BzED

time (3~10)

Event Login Error

3
> Video Defect ! Relay-out @2
> Audio Defect Alam Delay 10 Second (10~300)
> VSPlan Send Email

> Intelligence Behavior

Default

3 e e
S o
Storage
system
Information





Figure 3-85

Step 2 

Configure info of each parameter according to the actual needs; refer to the following sheet for more details. 

	Parameter
	Note

	Enable
	Click it to enable illegal access alarm. 

	Login error


	It will trigger alarm of illegal access after entering wrong password for some certain times, and the account will be locked. 


Note: 

Please refer to “3.4.1.1 Motion Detect” for description of other parameters. 

Step 3 

Click “Save” to complete config. 

3.5 Storage Management 
Here you can set schedule, storage mode and record control. 
3.5.1 Schedule 
Before schedule setup, user must set record mode in record control as auto status. 

Note:

If record mode in record control is off, then device will not record or snapshot according to the schedule. 

3.5.1.1 Record Schedule

Step 1 

Select “Setup > Storage > Schedule > Record Schedule”. 

The system will display the interface of “Record Schedule”, which is shown in Figure 3-86. 
[image: image153.png]IP PTZ Dome [ [ romsa | sor | om [ o]

Camera Record Schedulesnapshot schedule] Holiday Schedule: ®@
Network

PTZ Settings
Event 14 16 18 0 2 M

@I General M ¥/ Motion (¢ Alam

G Sunday

Schedule Monday

> Destination Tuesday

> Record Control Wednesday
system
Information

Thursday
Friday
Saturday

Holiday

11331331





Figure 3‑86
Step 2 

From Monday to Sunday select record time, click “Setup” on the right. See Figure 3-87 for more details.
· Set period according to actual needs. There are six periods available each day. 

· By checking or cancel, you can add or delete three types of record schedule: General, Motion, and Alarm. 

Note:

Period setup can be done by dragging in record schedule interface while not releasing left mouse. 
[image: image154.png]setup

™A

Period 1.
Period 2:
Period 3:
Period 4
Period 5

Period

¥ sunday I Monday I~ Tuesday [T Wednesday I Thursday I Friday

00
00
00
00
00
00

00
00
00
00
00
00

00
00
00
00
00
00

23 - 69 : 59 | [ General ¥ Hotion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion

23 - 69 : 59 | [ General I Motion
save cancel

W Alarm
™ Alarm
™ Alarm
™ Alarm
™ Alarm
™ Alarm

I saturday I~ Holiday





Figure 3‑87
Step 3 

Click “Save”, return to record schedule interface. See Figure 3-88. 

· Green color stands for the general record.

· Yellow color stands for the motion detect record.
· Red color stands for the alarm record. 

[image: image155.jpg]IP PTZ Dome

Camera
Network
PTZ Settings
Event

> Destination
> Record Control
System
Information

Playback

Record ScheduleSnapshot Schedule] Holiday Schedule

@I GeneralM ¥/ Moton (¢ Alam M

Sunday
Monday
Tuesday
Wednesday
Thursday
Friday
Saturday

Holiday

Default Reffesn





Figure 3‑88
Step 4 
Click “Save” on the “Record Schedule” interface, the system prompts “Successfully Saved”, and the record schedule is completed. 

3.5.1.2 Snapshot Schedule
Step 1 

Select “Setup > Storage > Schedule > Snapshot Schedule”. 

The system will display the interface of “Snapshot Schedule”, which is shown in Figure 3-89. 
[image: image156.png]IP PTZ Dome [ [ romsa | sor | om [ o]

Camera Record Schedule [snapshot Schedule] Holiday Schedule ®@
Network

PTZ Settings
Event 14 16 18 0 2 M

@I General M ¥/ Motion (¢ Alam

G Sunday

Schedule Monday

> Destination Tuesday

> Record Control Wednesday
system
Information

Thursday
Friday
Saturday

Holiday

11331331





Figure 3‑89
Step 2 

Set the snapshot period according to step 2 and 3 of “3.5.1.1 record Schedule”. 

Step 3 

Click “Save” and the system will prompt “Successfully Saved’, then snapshot schedule is completed. 

3.5.1.3 Holiday Schedule

Holiday schedule can set specific date as holiday. 
Step 1 

Select “Setup > Storage > Schedule > Holiday Schedule”. 

The system will display the interface of “Holiday Schedule”, which is shown in Figure 3-90. 
[image: image157.png]T [ [ romsa | sor | om [ o]

Camera Record Schedule [snapshot SchedulefHoliday schedute] ®@
Network
P12 settings Record (| Snapshat

Event
Storage
o
system
Information





Figure 3‑90
Step 2 

Select date to set as holiday. The selected date will be highlighted in yellow. 

Step 3 

Check “Record/Snapshot”, click “Save”. System prompts it is successfully saved.

Step 4 

Check “Record Schedule/Snapshot Schedule” interface, click setup next to “Holiday”, and refer to setup of “Monday to Sunday”. 

Step 5 

Complete setup of “Holiday”, then it records/snapshots according to date set in holiday schedule.

3.5.2 Destination 
3.5.2.1 Path
Path can configure storage path of device record and snapshot. There are three options: Local, FTP and NAS. You can only select one mode. System can save according to the event types. It is corresponding to the three modes (general/motion/alarm) in the schedule interface. Please check the box to enable the save functions. 
Note: Only some devices support NAS storage, please refer to the actual device.
Step 1 

Select “Setup > Storage > Destination > Path”. 

The system will display the interface of “Path”, which is shown in Figure 3-91. 

[image: image158.jpg]IP PTZ Dome

Camera
Network

PTZ Settings
Event

> Schedule
> Record Control
System
Information

Path

Record
Event Type
Local
FTP

Scheduled
v

Motion Detect
v

Default

Reffesn

Playback

NAS

Snapshot
EventType Scheduled
Local v
FTP

Motion Detect
v




Figure 3‑91
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter 
	Function 

	Event Type 
	It includes: scheduled, motion detect and alarm.  

	Local 
	It is saved in the SD card. 

	FTP 
	It is saved in the FTP server. 

	NAS
	It is saved in NAS server. 


3.5.2.2 Local

Here it can display kinds of information of local SD card in the local storage list. You can also realize several operations such as read-only, read & write, hot swap and format. 
Select “Setup > Storage > Destination > Local”, the system will display the interface of “Local” shown in Figure 3-92. 
[image: image159.jpg]IP PTZ Dome

Playback

Camera
Network

PTZ Settings
Event

Used Capacity/Total Capacity

> Schedule
> Record Control
System
Information

Reagony ] _Rease e |[_Hotswap ][ Retesn





Figure 3‑92
· Click “Read only” to set the SD card as read only. 

· Click “Read & Write” to set the SD card as read & write. 

· Click “Hot Swap” to realize hot swap upon the SD card. 

· Click “Format” to realize formatting upon the SD card. 

3.5.2.3 FTP
You need to check the box to enable the FTP function. When network disconnect occurred or there is malfunction, emergency storage can save the record/snapshot to the local SD card. 
Step 1 

Select “Setup > Storage > Destination > FTP”, the system will display the interface of “FTP” shown in Figure 3-93.  
[image: image160.jpg]IP PTZ Dome

Camera
Network

PTZ Settings
Event

> Schedule
> Record Control
System
Information

Path

Enable
Server Address

Port 21

User Name anonymity
Password

Remote Directory

Emergency (Local)

(0~65535)

Default

Playback





Figure 3‑93
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter
	Note

	Enable FTP
	Click it to enable FTP function

	Server Address
	FTP server address

	Port
	FTP server port

	User name 
	User name used to log in FTP server. 

	Password 
	Password used to log in FTP server.

	Remote Directory
	Store it to the directory of FTP server. 

	Emergency (Local)
	Click it and it will store to local SD card when FTP storage abnormity occurs. 


Step 3 

Click “Save” to complete config. 

3.5.2.4 NAS 

When it selects NAS storage mode, NAS function can be enabled. You can store file to NAS server when selecting NAS storage. 
Step 1 

Select “Setup > Storage > Destination > NAS”, the system will display the interface of “NAS” shown in Figure 3-94. 

[image: image161.jpg]IP PTZ Dome

Camera Path
Network

PTZ Settings

Event

Enable.
Server Address

Remote Directory.

> Schedule
> Record Control
System
Information

Playback





Figure 3-94
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter
	Note

	Enable NAS
	Click it to enable NAS function.

	Server Address
	NAS server address.

	Remote Directory 
	Store it to the directory of the NAS server. 


Step 3 
Click “Save” to complete config. 
3.5.3 Record control 
Step 1 

Select “Setup > Storage > Record Control”, the system will display the interface of “record Control” shown in Figure 3-95. 
[image: image162.png]IP PTZ Dome

Camera

Network

PTZ Settings

Event

Storage
Schedule

> Destination
Record Control

System

Information

Record Control

Pack Duration
Pre-event Record
Disk Full

Record Mode

Record Stream

30

Ovenwrite
© Auto O Manual

Main Stream

Minute (1~120)

Second (0-5)

Default





Figure 3‑95
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter 
	Function 

	Pack Duration
	It is to set the pack duration of each record file; it is 30 mins by default. 

	Pre-event Record 
	It is to set pre-event record time. 

For example, when it inputs 5, then the system will read the record video of first 5 seconds of the internal storage and record it into the file. 

Note:

Configure pre-event record time, when alarm record or motion detection record occurs, if there is no record, system will record the preceding n seconds video data into the record file.  

	Disk Full
	There are two options: stop recording or overwrite the previous files when HDD is full.

· Stop: Current working HDD is overwriting or current HDD is full, it will stop record. 

· Overwrite: Current working HDD is full; it will overwrite the previous file.

	Record mode 
	There are three modes: Auto/manual/close. It starts recording when selecting manual mode, it records within the range of schedule when selecting auto mode. 

	Record stream 
	There are two options: main stream and sub stream. 




Step 3 
Click “Save” to complete config. 
3.6 System 
3.6.1 General 
3.6.1.1 General 
Step 1 

Select “Setup > System > General”, the system will display the interface of “General” shown in Figure 3-96. 
[image: image163.png]e IS 7 I ST TR

Camera Date&Time ®
Network
PTZ Settings. Device Name 1E03BS7PAN00001

Event Language English
Storage VideoStandard | NTSC

System Tvout ON

eneral

> Account

Default

> Default
> ImportExport
> Auto Maintain
> Upgrade

Information





Figure 3‑96
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter 
	Function 

	Device Name
	It is to set device name. 
Note: 

Different devices may have different names. 

	Language
	You can select the language from the dropdown list.

	Video Standard 
	It is to display the video format of device, such as 50Hz. 

	TV Output 
	It can select on or off, the device which only supports TV output can have this function. 

Note: 

It will disable the intelligent functions when it is confirmed to enable TV output. 
It will disable TV output when it is confirmed to enable intelligent functions.

Some models support SDI, HDCVI functions. 


Step 3 
Click “Save” to complete config. 

3.6.1.2 Date & Time 

Step 1 

Select “Setup > System > General > Date & Time”, the system will display the interface of “Date & Time” shown in Figure 3-97. 
[image: image164.png]T [ [ romsa | sor | om [ o]

Camera General Date&Time ®@
Network

PTZ Settings
Event Time Format 24-Hourbased System ¥

Date Format Year-Month-Day v

storage Time Zone GMT+08:00

System Current Time 2016-05-06

> Account DSTType

D=t Start Time. Jan

D LI End Time Jan

> Auto Maintain .

Synchronize with NTP
> Upgrade

NTP Server clock isc.org
Information

Port 123

Update Period 30 Minute (0-30)

e [L_Retesn





Figure 3‑97
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter 
	Function 

	Date format
	Here you can select date format from the dropdown list.

	Time Format
	Here you can select the corresponding time format which needs to be displayed. 

	Time zone 
	The time zone of the device. 

	System time 
	It is to set system time. It becomes valid after you set. 

	DST
	Here you can set begin time and end time of DST. You can set according to the date format or according to the week format. 

	NTP  
	You can check the box to enable network time sync function

	NTP server 
	You can set the address of time serve. 

	Port 
	It is to set the port of time server. 

	Update period 
	It is to set the sync interval between the device and the time server. 


Step 3 

Click “Save” to complete config. 

3.6.2 Account 
3.6.2.1 Account 
Only when users have the right of account management then can it realize account management operation. 
· For the character in the following user name or the user group name, system max supports 15-digits. The valid string includes: character, number, and underline. 

· Password can be 0~32 characters in number and letter only. User can modify other user’s password. 

· The user amount is 18 and the group amount is 8 when the device is shipped out of the factory. 
· User management adopts group/user modes. The user name and the group name shall be unique. One user shall be included in only one group. 
· Currently logged in user cannot change his/her own right. 
· There is one default user admin during initialization. Admin belongs to high right user by default when it is out of factory.
3.6.2.1.1 User Name 
You can enable anonymity login, add/remove user and modify user name and etc in “Setup > System > Account > Account > User name. See Figure 3-98. 
[image: image165.png]IP PTZ Dome

Camera
Network

PTZ Settings
Event
Storage
System

Account

Anonymous Login

User

Group Name.
admin

Remark.
admin's account

Authority List
Live

Alam

Video/Audio
DefaultimportExport

Playback
Log Search
Schedule/Destination
Conditions

Record Control
ClearLog
Network

Vs

Backup
Upgrade
Abnormality

P1Z Account
Auto Maintain General
Video/Audio Detect PTZ Setings

Add User,





Figure 3‑98
Note: 

The version info and other icons in the live interface except alarm have no right control temporarily. 
Anonymous login: Enable “Anonymity Login”, and input IP. No username or password is required, you can log in by anonymity (with limited rights). You can click logout to use other users to log in the device. 
Add user
It is to add user in group and set the right control of the user.

The highest right user admin can’t be deleted by default.
Step 1 

Click “Add User” and the system will pop out the interface of “Add User”, which is shown in figure 3-99. 
[image: image166.jpg]



Figure 3‑99
Step 2 

Input user name and password, select group and check authority list.
Note: 

· Once the group is selected, then the user right can only be subset of the group which can’t surpass the right of the group. 

· Users are recommended to make it lower than senior users when defining general users in order to make user management convenient. 

Step 3 

Click “Save”. 

Modify user 
Step 1 

Click the [image: image167.png]


 icon which is corresponding to the users which needs to be modified. 
The system will pop out the interface of “Modify User” which is shown in Figure 3-100. 

[image: image168.jpg]



Figure 3-100

Step 2 
Modify the user information according to actual needs.
Step 3 
Click “Save”. 

Modify password 
Step 1 

Select the check box of “Modify Password”. 
Step 2 
Input old password, input new password and confirm it. 

Step 3

Click “Save”. 

Delete User 

Click the [image: image169.png]


 icon of corresponding user which needs to be deleted, and the you can delete the user. 
3.6.2.1.2 Group

You can realize add group, delete group, modify password and other operations in “Setup > System > Account > Group”. You can refer to Figure 3-101 for more details. 
[image: image170.png]T [ [ romsa | sor | om [ o]

Camera Account ®@
Network

PTZ Settings Anonymous Login
e User Name Group.

No. Group Name Remark
Storage
1 admin administrator group
system S
2 user user group
> General

Authority List

Live Playback Record Control Backup P1Z Account
Alam Log Search ClearLog Upgrade Auto Maintain General
Video/Audio Schedule/Destination  Network Abnormality Video/Audio Detect PTZ Setings
DefaultimportExport  Conditions IR Light Control Wiper Control Vs





Figure 3‑101
Add Group: 

Please refer to “3.6.2.1.1 User Name” for more details. 

Modify Group 

Please refer to “3.6.2.1.1 User Name” for more details. 

Delete Group 

Please refer to “3.6.2.1.1 User Name” for more details. 

3.6.3 Peripheral 
Note: 

Only some models support peripheral management, please refer to the actual device for more details. 

You can make settings for wiper. 

Step 1 

Select “Setup > System > Peripheral > Wiper”.

The system will display the interface of “Wiper” which is shown in Figure 3-102 and Figure 3-103. 

[image: image171.jpg]IP PTZ Dome

Camera
Network
PTZ Settings
Event
Storage
Systen
> General
> Account

Perip

> Default

> ImportExport

> Auto Maintain

> Upgrade

Information

Wiper

Mode
Interval Time

Period

10
00

00

Second (0~255)
00

23

59

59

Default

Reffesn

Playback





Figure 3-102
[image: image172.jpg]IP PTZ Dome

Playback
Camera Wiper

Network

PTZ Settings Mode Manual v

Event Interval Time 10 Second (0-255)

Storage ]

system

stop

g

Information





Figure 3-103

Step 2 

Configure info of each parameter according to the actual needs. 

	Parameter 
	Note

	Mode 
	It is to set wiper mode, you can select timing and manual, it is timing by default. 

· In timing mode, it needs to set the period of enabling wiper.

· In manual mode, it needs to enable wiper by manual operation.  


	Interval Time
	It is the interval from when the wiper stops to when the wiper is enabled.  

	Period
	Click it to set the period when the wiper is enabled in timing mode. 

	Start, Stop, Once 
	In manual mode: 
· Click “Start”, and the wiper operates regularly according to the interval which has been set. 

· Click “Stop”, and the wiper stops. 

· Click “Once” and the wiper operates once. 


Step 3 
Click “Save” to complete config. 

3.6.4 Default 
Attention: 

Except network IP address and user management, other config of the device will be restored default, please operate carefully. 
You can restore the device to default by clicking “Default” in “Setup > System > Default”. Please see Figure 3-104 for more details. 
[image: image173.png][P F1z Bome [ e [ opwms [ s | aam ] ogon ]
Camera ®@

Network
PTZ Settings
Event
Storage

system
> General

> Account

5

> importExport
> Auto Maintain
> Upgrade
Information




Figure 3‑104
3.6.5 Import/Export 
It can realize quick config of several devices via configuring file import and export when the config method of several devices is the same. 

Step 1

Select “Setup > System > Import Export” on the WEB end of some device. 

The system will display the interface of “Import & Export”, which is shown in Figure 3-105.

[image: image174.png]i [ [ romsa | sor | om [ o]

Camera Import/Export ®@
Network

PTZ Settings
Event
Storage
system

> General
> Account

> Default

>

> Auto Maintain
> Upgrade
Information




Figure 3‑105
Step 2 

Click “Export” to export the config file (.backup file) to local. 
Step 3 

Click “Import” on the “Import & Export” interface of the WEB end of the device to be configured, and import the file into the system. So far the device config is completed. 

3.6.6 Auto Maintenance 
Users can set auto reboot system or auto delete file, it needs to set period and time for auto reboot system, it is 02:00 every Tuesday by default. It needs to set the period of the file if it needs to auto delete old files, and delete the file within the specific period. 

Step 1 

Select “Setup > System > Auto Maintenance”. 

The system will display the interface of “Auto Maintenance” in Figure 3-106. 
[image: image175.png]i [ [ romsa | sor | om [ o]

Camera Auto Maintain ®@
Network
PTZ Settings
Event
Storage
system

> General

> Account

> Default

> ImportExport
>

> Upgrade
Information




Figure 3‑106
Step 2 

Configure the info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Parameter
	Function

	Auto Reboot
	Check it and set auto reboot time.

	Auto Delete Old Files
	Check it and you can customize period, the range of period is from 1 to 31 days.


Step 3 

Click “Save” to make config valid. 

3.6.7 Upgrade
You can realize upgrade operation in “Setup > System > Upgrade”. See Figure 3-107 for more details. 

[image: image176.png]1P PTZ Dome [ [ romsa | sor | om [ o]

Camera Upgrade ®@
Network

PTZ Settings
Event
Storage
system

> General

> Account
> Default

> ImportExport
> Auto Maintain
>
Information




Figure 3-107

Click “Import” and select upgrade file, click “Upgrade” to realize firmware upgrade. The upgrade file is “*.bin” file. 

Note: 

It needs to reboot the device when upgrading wrong files; otherwise some module functions of the device will be disabled. 
3.7 Information 
The system supports checking system version, online users, log and etc.

3.7.1 Version 
Here you can view system hardware features, software version, release date and etc. Please note the following information is for reference only. 
Check the version info of the current WEB end in “Setup > System > Version”. See figure 3-108 for more details. 
[image: image177.png]7RIz Bome (ST SR TR ST TN

Camera Version ®@

Network
PTZ Settings Software Version  2.422.0000.0.R.D3.3019.9ANR, Build Date: 2016-04-25

Event WEB Version 321341350
Storage ONVIFVersion 242
System PTZVersion 20592 RHISFOC

Information CameraVersion  01.06.05

> Log
> Online User Copyright 2015,All Rights Reserved.





Figure 3‑108
3.7.2 Log 

In “Setup > System > Log’, you can check the device operation info implemented by users and some system info, see Figure 3-109 for more details. 

[image: image178.png]IP PTZ Dome

Camera Log
Network

PTZ Settings
Event

Storage
System
Information

> Version

> Online User

Detailed Information

Time:
User Name:
Type:
Content:

™

LR RV IR





Figure 3‑109
Please refer to the following sheet for log parameter information. 

	Parameter 
	Function 

	Start time
	Set the start time of the requested log. (The earliest time is 2000/1/1)


	End time
	Set the end time of the requested log. (The latest time is 2037/12/31)

	Type
	Log type consists of system operation, config operation, data management, alarm event, record operation, user management and log clear. 

	Search 


	First it needs to set start time and end time of the log to be searched, and select log type, click “Search”, it will display search bars dynamically; click “Stop” to pause log search, and it will display the searched bars and period area.  

	Log information 
	Click log record and it will display the detailed info of the log.

	Clear
	It is to clear all the log info on the device, but it fails to support classified clearance of log info.

	Backup 
	You can click this button to backup system log files which is searched to current PC.


Specific meaning included by different log types: 

· System operation: It includes application program enable, abnormal logout, logout, application program reboot, close/reboot device, system reboot, and system upgrade. 

· Config operation: It includes save config, delete config file. 

· Data operation; It includes setting hardware type, clearing data, hot swap, FTP status, record mode. 

· Event operation (it is to record the events such as video detection, IVS, alarm, abnormity and etc): It includes event start and event end.
· Record operation: It includes file access, file access error, and file inquiry. 

· User management (it is to record the user management modification and user login and logout): It includes login, logout, add user, delete user, modify user, add group, delete group, modify group.

· Clear log: It is to clear log. 

3.7.3 Online User
You can check the user info on the current WEB in “Setup > System > Online User”, see Figure 3-110 for more details.

[image: image179.png]T [ [ romsa | sor | om [ o]

Camera Online User ®@

e User Local Group User Login Time.
PTZ Settings 10.18.116.44 2016-05-06 14:34:42
Event

Storage

System

Information

> Version

> Log





Figure 3‑110
4 Alarm

The alarm module mainly provides users with alarm events to subscribe; it will record the alarm info in the right column when it triggers the alarm event which has been subscribed by users.
Step 1 
Click “Alarm” and the system will display the interface of “Alarm”, which is shown in Figure 4-1. 

[image: image180.png]IP PTZ Dome

Alarm Type Alarm Channel

Motion Detect Disk Full
Disk Error Video Tamper
External Alam Tlegal Access
Audio Detect Vs

Scene Changing
Operation

Prompt
Alarm Tone

Play Alarm Tone

Tone Path





Figure 4‑1
Step 2 

Configure info of each parameter according to the actual needs; please refer to the following sheet for more details. 
	Type 
	Parameter 
	Function 

	Alarm type 
	Motion detection 
	System records alarm when motion detection alarm occurs,

	
	Disk full
	System records alarm when disk is full.

	
	Disk error 
	System records an alarm when HDD malfunctions.

	
	Video tampering 
	System records alarm when camera is viciously masked. 

	
	External alarm 
	System records alarm when alarm inputs the device. 

	
	Illegal access
	System records alarm when there is unauthorized access. 

	
	Audio Detect 
	System records alarm when audio detection occurs. 

	
	IVS
	System records alarm when intelligent config occurs

	Operation 


	Prompt 
	When alarm is triggered, there will be an [image: image181.png]


 in main menu of alarm interface and system automatically records alarm info. The icon disappears when user click alarm menu bar. 
Note:
If alarm interface is displayed, when alarm is triggered, there will be no image prompt, but alarm record will be in list on the right. 

	Alarm Tone
	Play Alarm Tone
	When alarm occurs, system auto generates alarm audio. The audio supports customized setup. 

	
	Tone Path
	Here you can specify alarm sound file. 


5 Log out

Click log out button, system goes back to log in interface. See Figure 5-1.
[image: image182.png]“FEEbme |





Figure 5‑1
Note:
· This manual is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice.

· All trademarks and registered trademarks mentioned are the properties of their respective 
owners.

· If there is any uncertainty or controversy, please refer to the final explanation of us.

· Please visit our website for more information.
[image: image183.jpg]alhua




Dahua Technology Co., Ltd

Address：No.1199 Bin’an Road, Binjiang District, Hangzhou, PRC.

Postcode: 310053

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuatech.com

Website: www.dahuasecurity.com
PAGE  
95

_1380970326

_1380970338

_1380970349

_1380970309

